VAX/VMS KERMIT                                                         Page 100

COMMUNICATIONS  Parameters affecting the terminal  line  being  used  for  com-

munication.

DEBUGGING       Debugging mode in effect, if any.

DELAY           The  number  of  seconds Kermit-32 will delay before starting a

SEND or RECEIVE command when in remote mode.

ESCAPE          The current escape character for the CONNECT processing.

FILE_PARAMETERS File type, file naming, and incomplete file disposition.

INCOMPLETE_FILE_DISPOSITION

The action to take when a transfer is aborted.

LINE            Terminal line in use.

LOCAL_ECHO      Whether characters should be echoed locally when CONNECTed.

PACKET          For incoming and outbound packets.

PARITY          The parity type in use.

RECEIVE         For inbound packets.

RETRY           The number of retries to be done on bad packets.

SEND            For outbound packets.

7.4.6. Program Management Commands

THE HELP COMMAND

Syntax: HELP [topic {subtopic}]

Typing  HELP  alone  prints a brief summary of Kermit-20 and its commands.  You

can also type

HELP command

for any Kermit-20 command, e.g. "help send" or "help set parity"  to  get  more

detailed information about a specific command.

THE EXIT AND QUIT COMMANDS

Syntax: EXIT

Exit  from  Kermit-32.  You can also exit from the Kermit-32 when it is waiting

for a command by typing a control-Z.  When Kermit-32 is running  remotely,  two

control-Y's  will  abort the transfer, bringing Kermit-32 back to command mode.

The two control-Y's must be typed together, as if a timeout occurs between them

the  first is ignored.  When Kermit-32 is running locally, two control-Y's will

stop Kermit-32 and return you to DCL.  You will be able to CONTINUE if  you  do

VAX/VMS KERMIT                                                         Page 101

not  perform  any  command  which  runs  a program.  However, after continuing,

control-A, control-X and control-Z will no longer be accepted as commands.

QUIT is a synonym for EXIT.

THE LOG COMMAND

Syntax: LOG [option [filespec]]

Log the specified option to the specified file:

SESSION         During CONNECT log all characters that appear on the screen  to

the  specified  file.    During CONNECT, the session log can be

temporarily turned off during the remote session by typing  the

escape  character  followed by Q (for Quit logging), and turned

on again by typing the escape character followed by R (for  Re-

sume logging).

TRANSACTIONS    During  file  transfer,  log the progress of each file.  Trans-

action logging is  recommended  for  long  or  unattended  file

transfers, so that you don't have to watch the screen.  The log

may be inspected after the transfer is  complete  to  see  what

files were transferred and what errors may have occurred.

DEBUGGING       Log  debugging info to the specified file.  If no SET DEBUGGING

command was previously issued, the file will be opened  and  no

information written.  If DEBUGGING is turned on (either via the

SET DEBUGGING command or by  typed  control-D  during  a  local

transfer),  the packet debugging information will be written to

the file.  Packet format is described in  the  Kermit  Protocol

Manual.

Any log files are closed when you EXIT or QUIT from Kermit.  You may explicitly

close a log file and terminate logging by using the LOG command without a  file

specification.

7.5. Raw Download

"Raw  Download"  is  the term commonly used to describe the capture of a remote

file on the local system, without any kind of error  detection  or  correction.

This  allows  you  to obtain files from remote systems that do not have Kermit,

but with the risk of loss or corruption of data.

Kermit-32 provides raw downloading via the LOG SESSION command  during  CONNECT

to  a  remote system.  The session log is described above.  To use session log-

ging to capture a file:

1. Run Kermit on the VAX/VMS system.

2. SET LINE to the terminal line through which you will be connected to

the remote system.

3. Perform any required SET commands to condition Kermit for communica-

tion with the remote system.

VAX/VMS KERMIT                                                         Page 102

4. CONNECT to the remote system and log in.

5. Condition your job on the remote system not to pause at the end of a

screenful  of  text,  and give whatever commands may be necessary to

achieve a "clean" terminal listing -- for instance, disable messages

from the system or other users.

6. Type  the  appropriate command to have the desired file displayed at

the terminal, but do not type the terminating carriage return.    On

most systems, the command would be "type", on Unix it's "cat".

7. Escape  back  to Kermit-32 and give the LOG SESSION command with the

file specification where you wish to store the data.

8. CONNECT back to the remote system and type a carriage return.    The

file  will  be  displayed on your screen and recorded in the session

log file.

9. Escape back to Kermit-32 and give the LOG SESSION command without  a

file specification.

The  file  you specified will contain everything that was typed on your screen.

You will probably  find  that  some  editing  necessary  to  remove  extraneous

prompts, messages, padding characters, or terminal escape sequences, or to fill

in lost or garbled characters.

7.6. Installation of Kermit-32

Kermit-32 may be installed either by rebuilding the  entire  program  from  the

sources,  or  by making use of the distributed copy of the program.  When being

built from sources, it may be built using either a BLISS-32  compiler  or  just

the MACRO-32 assembler.

FILES

Kermit-32  is  built from a number of BLISS-32 sources and one MACRO-32 source.

In order to make it possible for sites  without  BLISS-32  to  build,  MACRO-32

sources  generated  by BLISS-32 are also included for all of the BLISS modules.

In the normal distribution of Kermit-32, all of the files start with the prefix

"VMS".    This  will  need to be changed to "KER" in order to build the program

properly.  The following files are distributed as part of Kermit-32:

VMSTT.BLI       Common BLISS source for the terminal text output support.

VMSGLB.BLI      Common BLISS source for the global storage for VMSMSG.BLI.

VMSMSG.BLI      Common BLISS source for the protocol handling module.

VMSCOM.REQ      Common  BLISS  require  file  which  defines   various   common

parameters.  This is required by VMSMSG.BLI.  This file must be

renamed to KERCOM.REQ.

VMSMIT.BLI      BLISS-32 source for the command parser, and some basic  support

routines.

VAX/VMS KERMIT                                                         Page 103

VMSFIL.BLI      BLISS-32 source for the file I/O.

VMSTRM.BLI      BLISS-32  source for the terminal processing.  This handles the

driving of the terminal line for the transfers and the  connect

command processing.

VMSSYS.BLI      System  interface  routines  for  the  Kermit  generic  command

processing.

VMSGEN.MAR      Macro-32 source file that contains the REMOTE command text that

is  given  to  VMS.  Sites desiring to change what DCL commands

are used to process the various  generic  server  commands  can

make those changes in this source.  This also contains the text

of the help message returned in response to the server  generic

help command.

VMSERR.MSG      MESSAGE source for error messages used by VAX/VMS Kermit.

VMSERR.REQ      BLISS-32  require  file  which defines the error codes. This is

REQUIREd by the BLISS-32 sources.

VMSMIT.RNH      RUNOFF source for the help files for VAX/VMS Kermit.  When this

is  run through RUNOFF with /VARIANT=SYSTEM, it produces a .HLP

(VMSSYS.HLP) file suitable for inserting into the  system  help

library  (SYS$HELP:HELPLIB.HLB)  to  provide a KERMIT topic for

the system HELP command.  When run through RUNOFF  without  the

/VARIANT=SYSTEM,  it  produces  a  .HLP file (VMSUSR.HLP) to be

stored on SYS$HELP: for use by the Kermit HELP command.

VMSSYS.HLP      RUNOFF output file for system wide Kermit HELP.

VMSUSR.HLP      RUNOFF output file for Kermit's HELP command.

VMSINS.COM      Command file to build and install VAX/VMS Kermit.

VMSMIT.EXE      The executable (binary) file of VAX/VMS Kermit.  Note that this

is  only  included  when  it is possible.  This may be either a

copy of the .EXE file which was transferred  to  a  TOPS-10  or

TOPS-20  system using Kermit with Kermit-10 (or -20) using FILE

BYTE-SIZE 8-BIT, or a direct copy of the .EXE file (if the file

is  either on a native VAX/VMS tape or is residing on a VAX/VMS

system).

VMSMIT.HEX      A hexifed version of .EXE file for VMS Kermit.  This  file  can

be  dehexified  using  the  supplied  program.  In the hexified

form, the file should be transferrable over  any  medium  which

handles normal text.  This is the most reliable copy of the ex-

ecutable version of VMS Kermit.

VMSHEX.MAR      Source for the hexification program.  This is the program which

was used to produce VMSMIT.HEX.  It can also be used to produce

hexified version of any (or at least almost any) Files-11 file.

The  dehexification  program should then be able to reproduce a

copy of the original file with the  file  parameters  correctly

set.  Note that the format used for the hexified files is basi-

cally Intel hex format.  There are some additional records used

VAX/VMS KERMIT                                                         Page 104

to  store the record format, etc.  Also, the file name as typed

to the prompt from VMSHEX is stored in the hexified version  of

the file for use by the dehexification program.  By doing this,

it is possible to store more than one binary file with a single

hexified file.

VMSDEH.MAR      Source for the dehexification program.

VMSV3.MEM       Documentation  on  the  changes between versions 2.0 and 3.0 of

Kermit-32, and additional installation information.

As distributed, Kermit-32 should work on any VAX/VMS system  (version  3.0  and

later).    Customization  is  possible  with  or  without  a BLISS-32 compiler.

Default parameter values may be changed by changing the appropriate LITERALs in

the  BLISS-32  source  for VMSMSG, or the actual values which are stored in the

routine MSG_INIT in the MACRO-32 source for VMSMSG.

Sites can also easily change the commands which are  used  for  processing  the

generic  server functions (REMOTE commands when running as a server).  The text

which makes up these commands is in the file VMSGEN.MAR, along with the text of

the  REMOTE HELP message.  This allows a site to make use of local programs for

performing some of the commands (perhaps using FINGER to perform the  WHO  com-

mand, etc.).

BUILDING KERMIT-32 FROM SOURCES

A  command file is included which will build Kermit-32 from either the BLISS-32

or MACRO-32 sources and optionally install Kermit-32 on the system.  This  file

(VMSINS.COM)  has  not  been extensively tested, however it should work on most

systems.  It is also a good example of what needs to be done  to  compile  each

file  and  load the entire program.  It also contains the commands necessary to

install a version of Kermit-32 on the system once the .EXE and .HLP  files  are

generated.

USING KERMIT-32 TO INSTALL A NEW VERSION OF ITSELF

If you already have a version of Kermit-32 running on a VAX/VMS system, you can

use it to transfer a new version of itself from another system.  If you have no

need to build Kermit-32 from sources (because you have no local modifications),

you can just transfer the new VMSMIT.EXE (or VMSMIT.HEX), and the new help file

VMSMIT.RNH.

If you have access to a system which has a copy of VMSMIT.EXE, you can transfer

it simply by setting the FILE TYPE to FIXED on the receiving Kermit.  Make sure

that  the  sending Kermit is willing to send the file as a binary file.  If the

sending Kermit is another copy of Kermit-32, you don't need to do anything.  If

the  sending  Kermit is Kermit-10 or Kermit-20, make sure the file will be sent

as an eight-bit binary file by using SET FILE BYTE-SIZE EIGHT.  If  some  other

Kermit  is sending the file, make sure to give it whatever command is necessary

to ensure that it sends eight-bit binary data from the  file.    Also,  if  the

originating  system is not a VAX/VMS, TOPS-10 or TOPS-20 system, make sure that

the file was stored on that system correctly to start out with.  Normally,  the

file  VMSMIT.EXE will only be available from a VAX/VMS, TOPS-10 or TOPS-20 sys-

tem.

VAX/VMS KERMIT                                                         Page 105

If you only have access to a copy of VMSMIT.HEX, you will need to transfer that

file  (as  a normal ASCII text file).  You will also need a copy of VMSDEH.MAR.

After you have obtained both files, you can produce a copy of the .EXE file  as

follows.  First compile VMSDEH by using the command MACRO VMSDEH.  Then load it

by LINK VMSDEH.  Now run VMSDEH  and  when  it  asks  for  a  file  name,  type

VMSMIT.HEX.  The program will run for a short time and produce the .EXE file.

The system wide and user help files are produced from VMSMIT.RNH by RUNOFF.  To

produce the user help file (the one used by Kermit-32's HELP command), type:

$ RUNOFF VMSMIT.RNH/OUTPUT=KERUSR.HLP

$ LIBARARY/CREATE/HELP SYS$HELP:KERMIT.HLB KERUSR.HLP

To produce the system wide help file and install it in the system help  library

type:

$ RUNOFF VMSMIT.RNH/OUTPUT=KERSYS.HLP/VARIANT=SYSTEM

$ LIBRARY/REPLACE/HELP SYS$HELP:HELPLIB.HLB KERSYS.HLP

This allows the DCL HELP command to provide information on Kermit-32.

IBM VM/CMS KERMIT                                                      Page 106

8. IBM VM/CMS KERMIT

Program:    Daphne  Tzoar,  Columbia University; contributions from Bob Shields

(U. of Maryland), Victor Lee (Queens U.), Gary Bjerke (U.  of Texas

at Austin), Greg Small (UC Berkeley)

Language:   CMS Assembler

Documentation:

Daphne Tzoar, Columbia University

Version:    2.01

Date:       May 1985

CMS Kermit Capabilities At A Glance:

Local operation:                   No

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes (fixed format)

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      No

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                No

Communication settings:            No

Transmit BREAK:                    No

Transaction logging:               Yes

Session logging:                   No

Raw transmit:                      No

Act as server:                     Yes

Talk to server:                    No

Advanced server functions:         No

Advanced commands for servers:     No

Local file management:             Yes

Handle Attribute Packets:          No

Command/init files:                Yes

Command macros:                    No

Kermit-CMS is a program that implements the KERMIT file transfer  protocol  for

IBM 370-series mainframes (System/370, 303x, 43xx, 308x, etc.) under the VM/CMS

operating system.  It is written in IBM 370 assembly language.    This  program

runs only over ASCII (asynchronous) lines attached to a 3705-style front end or

a Series/1 running the Yale ASCII Terminal Communication System.   The  program

should  also  work  on the IBM 7171 ASCII Device Control Unit and the 4994, al-

though this has not been verified as of this writing.    For  more  details  on

this, see the section SET SERIES1.

These  systems have several peculiarities that users should be aware of.  These

are half duplex systems; the communication line must "turn around"  before  any

data  can be sent to it.  The fact that a packet has been received from the IBM

system is no guarantee that it is ready for a reply.  Thus any  Kermit  talking

to  this system must wait for the line turnaround character (XON) before trans-

mitting the next character.  It is up to the user to tell the other Kermit that

it is talking to an IBM mainframe.

IBM VM/CMS KERMIT                                                      Page 107

Also,  a  program  running  under  VM/CMS  is unable to interrupt a read on its

"console".  This means that the CMS version of Kermit cannot timeout.  The only

way  to  "timeout" CMS Kermit is from the other side:  typing a carriage return

to the local Kermit causing it to retransmit its last packet, or  an  automatic

timeout as provided by most other Kermits.

8.1. The VM/CMS File System

The  features  of  the CMS file system of greatest interest to Kermit users are

the format of file specifications and the concept of records.

8.1.1. File Specifications

The VM/CMS file specification is in the form

FILENAME FILETYPE FILEMODE

(often abbreviated FN FT FM).  FILENAME and FILETYPE are at most  8  characters

in  length  each.   The name field is the primary identifier for the file.  The

type is an indicator which, by convention, tells what kind  of  file  we  have.

For  instance  TEST  FORTRAN  is  the  source  of a FORTRAN program named TEST.

MODULE is the normal suffix for executable programs.  Although  some  operating

systems  consider  the FILETYPE optional, VM/CMS requires a valid type.  There-

fore, Kermit-CMS supplies a default type of "X" if one is not provided  by  the

remote system.

The  FILEMODE,  which consists of a letter and a number, is similar to a device

specification on microcomputer systems: FN FT FM would translate to FM:FN.FT in

CP/M or MS-DOS.  If FILEMODE is omitted from a file specification when sending,

a FM of "*" is used.  In this case, the users disks are  scanned  according  to

the  search order and the first occurrence of the file is the one that is sent.

If FILEMODE is omitted from a file spec when receiving, a default  of  "A1"  is

used.

To  provide  compatibility  with most other operating systems, Kermit-CMS sends

only the FILENAME and FILETYPE.  It also converts the intervening  blank  to  a

period.

The  FN  and  FT  may contain, in any order, uppercase letters, digits, and the

special characters "$" (dollar sign), "#" (pound  sign),  "@"  (at  sign),  "+"

(plus), "-" (dash), ":" (colon), and "_" (underscore).  Other characters may be

not be included.  If an invalid character is found in the FN or FT field, it is

replaced by the letter "X".

VM/CMS  allows  a group of files to be specified in a single file specification

by including the special "wildcard" characters, "*" and "%".  A "*" matches any

string of characters from the current position to the end of the field, includ-

ing no characters at all; a "%" matches any single character.   Here  are  some

examples:

* COBOL A

All files of type COBOL (all COBOL source files) on the A disk.

F* *    All files whose names start with F.

IBM VM/CMS KERMIT                                                      Page 108

% * B   All files on the B disk whose FN is exactly one character long.

8.1.2. File Formats

Several differences exist between VM/CMS files and those of most other  operat-

ing  systems.    One  distinction is that CMS encodes its data using the EBCDIC

character set.  The operating system, VM, translates  all  incoming  characters

from ASCII to EBCDIC.  Kermit-CMS then translates the data it reads back to AS-

CII (characters not representable in ASCII are replaced by a null).    This  is

done in order to correctly calculate the checksum, the method used to guarantee

error-free transfer.  When Kermit-CMS sends packets, it converts all data  back

to  EBCDIC.    Note that the translate tables used by Kermit must correspond to

the ones used by the system (VM).  The ASCII  to  EBCDIC  translations  can  be

found in the Appendix.

Another  difference  is  that  VM/CMS  stores files as records rather than byte

streams.  VM/CMS Kermit has to worry about  assembling  incoming  data  packets

into records and appending carriage return-linefeed to outgoing records.

8.2. Program Operation

Kermit-CMS  can  be invoked at the command line or from an exec.  Commands con-

sist of one or more fields, separated by spaces.   Each  field  must  be  eight

characters or less.  Fields longer than the maximum length are truncated.

Upon  initial  startup, the program looks for two special initialization files,

SYSTEM KERMINI and (USERID) KERMINI (that is, the userid of the person  running

CMS-Kermit.)    These  files  can  be placed on any disk.  The purpose of these

files is to allow Kermit to be customized for a particular  system  and  for  a

user's  specific  settings  without  changing the source code.  The file SYSTEM

KERMINI should be placed on a publicly accessible disk by a systems programmer.

The file would contain commands that all users would need to issue in order for

Kermit to run on the system, such as commands to modify the ASCII-to-EBCDIC and

EBCDIC-to-ASCII  tables  used  by  Kermit-CMS.  The file (USERID) KERMINI would

contain commands that the user generally  issues  every  time  Kermit  is  run.

Kermit-CMS executes any commands found in these files as though they were typed

at the terminal.  Here is a sample init file:

* Asterisk in column one is a comment.

set debug on

set warning on

set block 3

Three CP SET parameters MSG, IMSG and WNG are always set OFF during the  actual

file transfer (and restored afterwards) to prevent CP from interrupting any I/O

in progress.

IBM VM/CMS KERMIT                                                      Page 109

Interactive Operation:

To run  Kermit-CMS  interactively,  invoke  the  program  from  CMS  by  typing

"KERMIT".  When you see the command's prompt,

KERMIT-CMS>.

you  may  type  Kermit  commands  repeatedly  until  you  are ready to exit the

program, for example:

.KERMIT

Kermit CMS Version 2.01

Enter ? for a list of valid commands

KERMIT-CMS>.send foo *

Files with fn FOO are sent

KERMIT-CMS>.receive test spss

File is received and called TEST SPSS A1

KERMIT-CMS>.exit

During interactive mode, you may  use  the  help  ("?")  feature  while  typing

Kermit-CMS  commands.   A question mark typed at almost any point in a command,

followed by a carriage return, produces a brief description of what is expected

or possible at that point.

Command Line Invocation:

Kermit-CMS  may  also be invoked with command line arguments from CMS.  For in-

stance:

.KERMIT send test fortran

or

.KERMIT set debug on # set file binary # server

Kermit will exit and return to CMS after completing the  specified  command  or

commands.   Note that several commands may be given on the command line as long

as they are separated by the linend character, which  is  pound  sign  in  this

case.  The command line may contain up to 130 characters.

Exec Operation:

Like  other  CMS  programs,  Kermit-CMS may be invoked from a CMS EXEC.  Kermit

will not run under CMSBATCH or disconnected since the user  does  not  actually

have  a  console.   Commands can be passed using TAKE files and/or command line

arguments.  For example, to start up Kermit-CMS and have it act  as  a  server,

include the line:

IBM VM/CMS KERMIT                                                      Page 110

KERMIT server

To  pass more than one command, they must be stacked in the order in which they

are to be executed.  To start up a Kermit-CMS server  with  a  three  character

CRC, include:

&STACK set block 3

&STACK server

KERMIT

Prompts and messages will still be displayed on the screen.

8.3. Kermit-CMS Commands

Here's a brief summary of CMS Kermit commands:

CMS  executes a CMS command.

CP  executes a CP command.

EXIT  from Kermit-CMS.

HELP  about Kermit-CMS.

QUIT  from Kermit-CMS

RECEIVE  files from other Kermit.

SEND  files to other Kermit.

SERVER  mode of remote operation.

SET  various parameters.

SHOW  various parameters.

STATUS  inquiry.

TAKE  commands from file.

TDUMP  dumps the contents of a particular table.

The  remainder  of  this section concentrates on the commands that have special

form or meaning for CMS Kermit.

THE SEND COMMAND

Syntax:  SEND filespec

The SEND command causes a file or file group to be sent from the CMS system  to

the Kermit on the remote system.  filespec takes the form:

filename filetype [filemode]

filespec  may contain the wildcard characters "*" or "%".  If filespec contains

wildcard characters then all matching files will be sent.  If, however, a  file

exists  by  the  same name on more than one disk, only the first one CMS Kermit

encounters, according to the disk search order, is sent.

Although the file transfer cannot be cancelled from the CMS side, Kermit-CMS is

capable of responding to "cancel file" or "cancel batch" signals from the local

Kermit; these are typically entered by typing Control-X and  Control-Z  respec-

tively.

IBM VM/CMS KERMIT                                                      Page 111

THE RECEIVE COMMAND

Syntax: RECEIVE [filespec]

The  RECEIVE  command tells Kermit-CMS to receive a file or file group from the

other system.  You should then issue a SEND command to the remote Kermit.

The format of the filespec is:

filename filetype [filemode]

If the optional filespec is not  included,  Kermit-CMS  will  use  the  name(s)

provided  by  the  other  Kermit.    If that name is not a legal CMS file name,

Kermit-CMS will delete excessive characters from it, and  will  change  illegal

characters to the letter X.

Use  the file specification to indicate that the incoming file should be stored

under a different name.  The filespec may include a filemode to  designate  the

destination  disk.  If none is provided, the file will be saved with fm A1.  If

you want to use the same name  but  a  different  filemode,  specify  =  =  FM.

Wildcards may not be used in any field.

If the optional filespec is provided, but more than one file arrives, the first

file will be stored under the given filespec, and the remainder will be  stored

under  their  own  names on the A disk.  If, however, = = FM is used, all files

will be placed onto the specified disk.

When receiving files, if the record format is fixed, any record longer than the

logical record length will be split up to as many records as necessary.  If the

record format is variable, the record length can be as high as 64K.  For  send-

ing  files,  the maximum record length is 64K.  See the SET LRECL and SET RECFM

commands.

If an error occurs during the file  transfer,  as  much  of  the  file  as  was

received  is  saved on disk.  If the sending of a file is cancelled by the user

of the remote system, Kermit-CMS will discard whatever had arrived.

If the incoming file has the same name as a file that already exists, and WARN-

ING  is  OFF,  the  original  file  will be overwritten.  If WARNING is set ON,

Kermit-CMS will change the incoming name so as not to obliterate the pre-exist-

ing file.  It attempts to rename the file by replacing the first character with

a dollar sign ($).  If a file by that name exists, it also replaces the  second

character  with  a dollar sign.  It continues in this manner for all characters

of the filename and filetype.  If the file  cannot  be  renamed,  the  transfer

fails.

THE TAKE COMMAND

Syntax: TAKE filespec

Execute  Kermit commands from the specified file, where filespec has the format

fn ft [fm].  The command file may include TAKE commands.

IBM VM/CMS KERMIT                                                      Page 112

THE SERVER COMMAND

Kermit-CMS is capable of acting as a server.  The user connects to the CMS sys-

tem once to set various options and to start the server.  From then on, he need

not connect to the CMS system again.  The current  version  of  Kermit-CMS  can

send  files  (the  user  on the other end types the GET command, using either a

space or a period as the delimiter between filename, filetype,  and  filemode),

receive  files  (the user types SEND), and terminate by either returning to CMS

(user types FINISH) or logging the user out (user types BYE).

To put Kermit-CMS into server mode, first issue any  desired  SET  commands  to

select various options and then type the SERVER command.  Kermit-CMS will await

all further instructions from the user Kermit on the other end of  the  connec-

tion.  For example:

KERMIT-CMS>.set warning on

KERMIT-CMS>.set debug on

KERMIT-CMS>.set block 3

KERMIT-CMS>.server

THE SET COMMAND

Syntax: SET parameter [value]

Establish  or  modify  various  parameters  for file transfer.  You can examine

their values with the SHOW command.  The following SET commands  are  available

in Kermit-CMS:

ATOE  Modify ASCII-to-EBCDIC table used by Kermit-CMS

BLOCK  Level of error checking for file transfer

DEBUG  Log packets sent and received during file transfer

END-OF-LINE  Packet terminator

ETOA  Modify EBCDIC-to-ASCII table used by Kermit-CMS

FILE  Type of incoming or outgoing file

LRECL  Logical Record length for incoming file

PACKET-SIZE  Maximum receive packet size

QUOTE  Use to quote outgoing control characters

RECFM  Record format for incoming files

SERIES1  Indicate if coming in via a Series/1

WARNING  Specify how to handle filename collisions

SET ATOE

Syntax: SET ATOE num1 num2

This  command  allows you to modify the ASCII-to-EBCDIC translate table used by

Kermit-CMS to conform to your system.  Specify the offset of  the  ASCII  value

within  the  table  and  the  new  value for that location.  Both num1 and num2

should be between 0 and 255 (decimal).  Note that the table is twice as long as

necessary  because  the translate instruction expects a table that contains 256

characters.

IBM VM/CMS KERMIT                                                      Page 113

SET BLOCK

SYNTAX: SET BLOCK number

Determine the type of block check used during file transfer.  Valid options for

number  are: 1 (for a one character checksum), 2 (for a two character checksum)

and 3 (for a three character CRC).

SET DEBUG

Syntax: SET DEBUG ON or OFF

ON      Keep a journal of all packets sent and received in the file KER LOG A1.

If the file already exists, it is overwritten.

OFF     Stop logging the packets.

SET END-OF-LINE

Syntax: SET END-OF-LINE number

If the remote system needs packets to be terminated by anything other than car-

riage return, specify the decimal value of the desired ASCII character.  number

must be between 0 and 31 (decimal).

SET ETOA

Syntax: SET ETOA num1 num2

This  command  allows you to modify the EBCDIC-to-ASCII translate table used by

Kermit-CMS to conform to your system.  Specify the offset of the  EBCDIC  value

within the table and the new ASCII value for that location.  Both num1 and num2

should be between 0 and 255 (decimal).

SET FILE

Syntax: SET FILE BINARY or TEXT

BINARY  Tells CMS Kermit to treat each character as a string of bits and not to

perform  translation on the data.  Also, no carriage-return is added to

the end of outgoing records.  Incoming bytes are added to  the  end  of

the  current  record  which  is written out when the specified LRECL is

reached.

TEXT    Tells CMS Kermit that the file is  plain  text.    ASCII-to-EBCDIC  and

EBCDIC-to-ASCII  translation  is  performed  on  the  data.    Carriage

return-linefeed are appended to outgoing records and are used to deter-

mine the end of incoming records.

IBM VM/CMS KERMIT                                                      Page 114

SET LRECL

Syntax: SET LRECL number

Set  the  logical  record length for incoming files to a number from 1 to 65536

(64K).  This variable is used only for fixed format files.  The default is 80.

SET PACKET-SIZE

Syntax: SET PACKET-SIZE number

Use the specified number as the maximum length for incoming packets.  The valid

range is 26-94, where 94 is the default.

SET QUOTE

Syntax: SET QUOTE char

Use  the  indicated printable character for prefixing (quoting) control charac-

ters and other prefix characters.  The only reason to change this would be  for

sending  a  very  long  file that contains very many "#" characters (the normal

control prefix) as data.  It must be a single character in  the  range:  33-62,

96, or 123-126 (decimal).

SET RECFM

Syntax: SET RECFM option

Set  the  record  format  to use for incoming files.  Valid options are "F" for

fixed format or "V" for variable format.  The default is variable.

SET SERIES1

Syntax: SET SERIES1 ON or OFF

Kermit-CMS automatically determines whether you are connected  via  a  Series/1

emulation controller or a TTY line.  This command is provided though so you can

override Kermit-CMS.  When  you  SET  SERIES1  ON,  Kermit  disables  the  3270

protocol  conversion  function by putting the Series/1 into "transparent mode";

this allows Kermit packets to pass through the Series/1 intact.

SET WARNING

Syntax: SET WARNING ON or OFF

ON      If an incoming file has the same  name  as  an  existing  file  on  the

specified  disk,  Kermit will attempt to rename the incoming file so as

not to destroy (overwrite) the pre-existing one.

OFF     Upon a filename collision, the existing file will be  replaced  by  the

incoming file.

IBM VM/CMS KERMIT                                                      Page 115

THE SHOW COMMAND

Syntax: SHOW option

Use  to  display  the values of all parameters that can be changed with the SET

command, except for ATOE and ETOA (see the TDUMP command).   option  can  be  a

particular parameter or the keyword "ALL".

THE STATUS COMMAND

Syntax: STATUS

Returns  the  status of the previous command.  The response will either display

the message "Kermit completed successfully", or the last error encountered.

THE TDUMP COMMAND

Syntax: TDUMP table-name

Display the contents of table-name since it can be modified using the SET  com-

mand.  The ATOE and ETOA tables can presently be 'dumped'.

CP AND CMS COMMANDS

Syntax: CP text of command

CMS text of command

Although Kermit-CMS does not provide explicit commands for managing local files

or interacting with the operating system, it is possible to do so.  You can is-

sue any CP or CMS command, though each word will be truncated to eight  charac-

ters.  You can list, type, rename or delete files, send messages and so on.  At

this time, though, you cannot run another program from Kermit-CMS.

8.4. Before Connecting to the Mainframe

When connecting to the CMS system as a TTY  device  ("line  at  a  time"  mode)

several flags must first be set on the micro version of Kermit.  You should set

the LOCAL-ECHO flag to ON (this is used to indicate half-duplex).  This is  the

norm  but  not  true  in  every  case; if each character appears twice, set the

LOCAL-ECHO flag OFF.  HANDSHAKE should be set to XON and FLOW-CONTROL should be

set  to  NONE.    The parity should be set according to the system's specifica-

tions.  On some micro versions of Kermit, all of the above is done in one  step

using the DO IBM macro (or SET IBM ON).  Set the baud rate to correspond to the

line speed.

When connecting to the CMS system through the Series/1 terminal emulation  con-

troller ("full-screen" mode), certain flags must be set on the micro version of

Kermit.  You should set the LOCAL-ECHO flag to OFF (this is  used  to  indicate

full-duplex).  HANDSHAKE should be set to OFF and FLOW-CONTROL should be set to

XON/XOFF.  For most systems, the PARITY should be set to EVEN.   Set  the  baud

rate  to  correspond  to  the  line  speed.   Immediately after issuing a SEND,

RECEIVE or SERVER command to Kermit-CMS, the screen will be cleared.   This  is

IBM VM/CMS KERMIT                                                      Page 116

to  make sure the terminal is not stuck in a MORE or HOLDING state before pack-

ets are sent via the full-screen I/O operation.  After  the  file  transfer  is

complete,  and  you  re-connect  to  Kermit-CMS,  you should enter the Series/1

"Master Reset" sequence (probably "CTRL-G") so that the screen is refreshed.

8.5. How to build an executable version of Kermit-CMS

Kermit-CMS is composed of three assembly language source  files  that  are  as-

sembled  separately  and then linked together.  They are:  KERMIT, NEXTFST, and

WILD.  To create a runnable version:

1. GLOBAL the necessary MACLIBs.  For SP, the MACLIBs used  are  DMSSP,

CMSLIB and TSOMAC.

2. Make  sure  the source files are named as listed above and are saved

on disk in fixed format with a logical record length of 80.

3. Assemble the three source files.

4. Load the files into memory via: LOAD KERMIT

5. Create a runnable version called KERMIT MODULE via: GENMOD KERMIT

6. If your site's ASCII/EBCDIC translation table does  not  conform  to

the  one  listed  in the appendix (which in turn conforms to the one

given in the IBM System/370 Reference Summary), then enter  the  ap-

propriate  SET  ATOE/ETOA commands in the SYSTEM KERMINI file.  Note

that if the ASCII/EBCDIC translation is not invertible, Kermit  will

not and cannot work.

To run CMS Kermit, simply type "KERMIT" to the CMS system prompt.

8.6. What's New

Below is a list of the more important additions in Version 2.01 of CMS Kermit:

1. Add  prefixing  of the "8th bit".  This allows CMS Kermit to send or

receive fixed format  binary  data,  such  as  microcomputer  binary

files.

2. The maximum record length allowed has been increased to 64K for both

incoming and outgoing files.

3. Repeat count prefixing has been added to speed up transmission.   In

addition, Kermit-CMS now packs as much data as possible into an out-

going packet rather than one packet  per  record  (it  makes  a  big

difference).

4. Support for two character checksum and three character CRC.

5. If  filetype  is not supplied by the remote Kermit, use a default of

"X" rather than failing.  Also allow the option to rename an  incom-

ing file upon filename collision.

IBM VM/CMS KERMIT                                                      Page 117

6. Add  debugging  mode to log packets and to acknowledge the attention

if the user types a BREAK.  Add a SHOW ALL command.

7. Allow input to command parser from command line.  Multiple  commands

should  be  separated  by  the linend character.  After execution of

these commands, Kermit returns control to CMS.

8. Add support for Series/1 front end.

9. Add server support.

10. Upon startup, read commands from two init files: SYSTEM KERMINI  and

(USERID)  KERMINI (that is the userid of the person running Kermit).

Lines with an asterisk as the first character are comments.  Add the

TAKE command.  The LRECL for these files must be 130 or less.

11. Implement skip file or file group when sending or receiving (in this

case, discard incoming file).

12. Allow system manager to change ASCII/EBCDIC  translation  tables  to

conform  to  the various specifications of different systems.  Also,

bypass user translate tables when sending and receiving data.

8.7. What's Missing

Work on Kermit-CMS will continue.  Features that need to be improved  or  added

include:

- Allow  timeouts  so CMS Kermit does not wait forever if a packet does

not arrive in a timely fashion.

- Allow any binary file to be sent  and  received  properly,  not  only

fixed format binary files.

- Better keyword parsing, which is rather crude at this time.

- Support of the more advanced server functions.

- Addition of a RUN command.

- Ability  to  SET LINE, so that Kermit-CMS can be used as a local Ker-

mit, connecting to a remote host over another communication port.

UNIX KERMIT                                                            Page 118

9. UNIX KERMIT

Program:    Frank da Cruz, Bill Catchings, Jeff  Damens,  Columbia  University;

Herm Fischer, Encino CA; contributions by many others.

Language:   C

Documentation:

Frank da Cruz, Herm Fischer

Version:    4C(057)

Date:       July 26, 1985

C-Kermit is a completely new implementation of Kermit,  written  modularly  and

transportably  in  C. The protocol state transition table is written in wart, a

(non-proprietary) lex-like preprocessor for C. System-dependent primitive func-

tions  are isolated into separately compiled modules so that the program should

be easily portable among Unix systems and also to non-Unix systems that have  C

compilers.    This document applies to Unix implementations of C-Kermit, and in

most ways also to the VMS implementation.

Unix Kermit Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Login scripts:                     Yes

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     Yes

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes

Support for dialout modems:        Yes

IBM mainframe communication:       Yes

Transaction logging:               Yes

Session logging:                   Yes

Debug logging:                     Yes

Packet logging:                    Yes

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Local file management:             Yes

Command/Init files:                Yes

UUCP and multiuser line locking:   Yes

File attributes packets:           No

Command macros:                    No

Raw file transmit:                 No

C-Kermit provides traditional Unix command line operation  as  well  as  inter-

active  command  prompting and execution.  The command line options provide ac-

cess to a minimal subset of C-Kermit's capabilities;  the  interactive  command

set is far richer.

UNIX KERMIT                                                            Page 119

On  systems  with  dialout  modems, C-Kermit can use its command file and login

script facilities to replicate the file transfer functionality  of  UUCP  among

heterogeneous  operating  systems,  including  the  use of scheduled (e.g. late

night) unattended operation.

9.1. The Unix File System

Consult your Unix manual for details about the file system under  your  version

of  Unix.  For the purposes of Kermit, several things are worth briefly noting.

Unix files generally have lowercase names, possibly containing one or more dots

or  other special characters.  Unix directories are tree-structured.  Directory

levels are separated by slash ("/") characters.  For example,

/usr/foo/bar

denotes the file bar in the directory /usr/foo.  Wildcard or "meta"  characters

allow groups of files to be specified.  "*" matches any string; "?" matches any

single character.

When C-Kermit is invoked with file arguments  specified  on  the  Unix  command

line,  the  Unix shell (Bourne Shell, C-Shell, etc) expands the meta characters

itself, and in this case a wider variety is available.  For example,

kermit -s ~/ck[uvm]*.{upd,bwr}]

is expanded by the Berkeley C-Shell into a list of all the files in the  user's

home  directory  (~/) that start with the characters "ck", followed by a single

character "u", "v", or "m", followed by zero or more characters, followed by  a

dot,  followed  by one of the strings "upd" or "bwr".  Internally, the C-Kermit

program itself expands only the "*" and "?" meta characters.

Unix files are linear streams of 8-bit bytes.  Text files consist of 7-bit  AS-

CII  characters,  with  the  high-order bit off (0), and lines separated by the

Unix newline character, which is linefeed (LF, ASCII 10).   This  distinguishes

Unix  text  files  from  those  on most other ASCII systems, in which lines are

separated by a carriage-return linefeed sequence (CRLF, ASCII  13  followed  by

ASCII  10).    Binary  files are likely to contain data in the high bits of the

file bytes, and have no particular line or record structure.

When transferring files, C-Kermit will convert between  upper  and  lower  case

filenames  and  between LF and CRLF line terminators automatically, unless told

to do otherwise.  When binary files must be transferred, the  program  must  be

instructed  not  to  perform LF/CRLF conversion (-i on the command line or "set

file type binary" interactively; see below).

9.2. File Transfer

If C-Kermit is in local mode, the screen (stdout)  is  continously  updated  to

show  the  progress  of the file transer.  A dot is printed for every four data

packets, other packets are shown by type:

I Exchange Parameter Information

R Receive Initiate

S Send Initiate

UNIX KERMIT                                                            Page 120

F File Header

G Generic Server Command

C Remote Host Command

N Negative Acknowledgement (NAK)

E Fatal Error

T Indicates a timeout occurred

Q Indicates a damaged, undesired, or illegal packet was received

% Indicates a packet was retransmitted

You may type certain "interrupt" commands during file transfer:

Control-F:  Interrupt the current File, and go on to the next (if any).

Control-B:  Interrupt the entire Batch of files, terminate the transaction.

Control-R:  Resend the current packet

Control-A:  Display a status report for the current transaction.

These interrupt characters differ from the ones used in other Kermit  implemen-

tations  to  avoid conflict with commonly used Unix shell interrupt characters.

With Version 7, System III, and System V  implementations  of  Unix,  interrupt

commands must be preceeded by the 'connect' escape character (e.g. normally-\).

CAUTION:  If Control-F or Control-B is used to cancel an incoming file,

and a file of the same name previously existed, and the "file  warning"

feature  is  not  enabled, then the previous copy of the file will dis-

appear.

EMERGENCY EXIT:  When running Unix Kermit in remote mode, if you have started a

protocol operation (sending or receiving a file, server command wait, etc), you

will not be able to regain control of the terminal until the protocol operation

has  run  its  course (completed or timed out).  In particular, you cannot stop

the protocol by typing the normal Unix interrupt characters, since the terminal

has  been  put in "raw mode".  If you need to regain control quickly -- for in-

stance, because the protocol is stuck -- you can type the following sequence of

four  characters  directly  to  the  Unix  Kermit  program  ("connect" first if

necessary):

Control-A Control-C Control-C Carriage-Return

This will cause the program to exit and restore the terminal to normal.

9.3. Command Line Operation

The C-Kermit command line syntax has been changed from that of earlier releases

of Unix Kermit to conform to the Proposed Syntax Standards for Unix System Com-

mands put forth by Kathy Hemenway and Helene Armitage of AT&T Bell Laboratories

in Unix/World, Vol.1, No.3, 1984.  The rules that apply are:

- Command names must be between 2 and 9 characters ("kermit" is 6).

- Command names must include lower case letters and digits only.

- An option name is a single character.

- Options are delimited by '-'.

- Options  with  no  arguments  may  be  grouped  (bundled)  behind one

delimiter.

- Option-arguments cannot be optional.

- Arguments immediately follow options, separated by whitespace.

UNIX KERMIT                                                            Page 121

- The order of options does not matter.

- '-' preceded and followed by whitespace means standard input.

A group of bundled options may end with an option that has an argument.

The following notation is used in command descriptions:

fn      A Unix file specification, possibly containing the  "wildcard"  charac-

ters  `*'  or  `?' (`*' matches all character strings, `?'  matches any

single character).

fn1     A Unix file specification which may not contain `*' or `?'.

rfn     A remote file specification in the remote system's  own  syntax,  which

may denote a single file or a group of files.

rfn1    A remote file specification which should denote only a single file.

n       A decimal number between 0 and 94.

c       A  decimal  number between 0 and 127 representing the value of an ASCII

character.

cc      A decimal number between 0 and 31, or else  exactly  127,  representing

the value of an ASCII control character.

[ ]     Any field in square braces is optional.

{x,y,z} Alternatives are listed in curly braces.

C-Kermit  command  line  options  may  specify  either actions or settings.  If

C-Kermit is invoked with a command line that specifies no actions, then it will

issue  a  prompt  and  begin interactive dialog.  Action options specify either

protocol transactions or terminal connection.

-s fn   Send the specified file or files.    If  fn  contains  wildcard  (meta)

characters,  the  Unix shell expands it into a list.  If fn is '-' then

kermit sends from standard input, which must come from a file:

kermit -s - < foo.bar

or a parallel process:

ls -l | kermit -s -

You cannot use this mechanism to send terminal typein.  If you want  to

send  a  file whose name is "-" you can precede it with a path name, as

in

kermit -s ./-

-r      Receive a file or files.  Wait passively for files to arrive.

-k      Receive (passively) a file or files, sending them to  standard  output.

This option can be used in several ways:

UNIX KERMIT                                                            Page 122

kermit -k

Displays the incoming files on your screen;  to  be  used  only  in

"local mode" (see below).

kermit -k > fn1

Sends the incoming file or files to the named file, fn1.   If  more

than  one  file  arrives,  all  are  concatenated together into the

single file fn1.

kermit -k | command

Pipes the incoming data (single or multiple files) to the indicated

command, as in

kermit -k | sort > sorted.stuff

-a fn1  If you have specified a file transfer option, you may specify an alter-

nate name for a single file with the -a option.  For example,

kermit -s foo -a bar

sends  the file foo telling the receiver that its name is bar.  If more

than one file arrives or is sent, only the first file  is  affected  by

the -a option:

kermit -ra baz

stores the first incoming file under the name baz.

-x      Begin server operation.  May be used in either local or remote mode.

Before  proceeding, a few words about remote and local operation are necessary.

C-Kermit is "local" if it is running on PC or workstation that  you  are  using

directly, or if it is running on a multiuser system and transferring files over

an external communication line -- not your job's controlling terminal  or  con-

sole.   C-Kermit is remote if it is running on a multiuser system and transfer-

ring files over its own controlling terminal's communication line, connected to

your PC or workstation.

If  you  are running C-Kermit on a PC, it is in local mode by default, with the

"back port" designated for file transfer and terminal connection.  If  you  are

running  C-Kermit on a multiuser (timesharing) system, it is in remote mode un-

less you explicitly point it at an external line for file transfer or  terminal

connection.  The following command sets C-Kermit's "mode":

-l dev  Line  --  Specify a terminal line to use for file transfer and terminal

connection, as in

kermit -l /dev/ttyi5

When an external line is being used, you might also need  some  additional  op-

tions for successful communication with the remote system:

-b n    Baud  --  Specify the baud rate for the line given in the -l option, as

in

kermit -l /dev/ttyi5 -b 9600

UNIX KERMIT                                                            Page 123

This option should always be included with the  -l  option,  since  the

speed of an external line is not necessarily what you expect.

-p x    Parity  --  e,o,m,s,n  (even, odd, mark, space, or none).  If parity is

other than none, then the 8th-bit prefixing mechanism will be used  for

transferring  8-bit  binary  data, provided the opposite Kermit agrees.

The default parity is none.

-t      Specifies half duplex,  line  turnaround  with  XON  as  the  handshake

character.

The  following  commands may be used only with a C-Kermit which is local -- ei-

ther by default or else because the -l option has been specified.

-g rfn  Actively request a remote server to send the named file or  files;  rfn

is a file specification in the remote host's own syntax.  If fn happens

to contain any special  shell  characters,  like  '*',  these  must  be

quoted, as in

kermit -g x\*.\?

-f      Send a 'finish' command to a remote server.

-c      Establish  a  terminal  connection  over  the specified or default com-

munication line, before any protocol transaction takes place.  Get back

to   the   local  system  by  typing  the  escape  character  (normally

Control-Backslash) followed by the letter 'c'.

-n      Like -c, but after a protocol transaction takes place; -c  and  -n  may

both  be used in the same command.  The use of -n and -c is illustrated

below.

On a timesharing system, the -l and -b options will also have  to  be  included

with the -r, -k, or -s options if the other Kermit is on a remote system.

Several other command-line options are provided:

-i      Specifies that files should be sent or received exactly "as is" with no

conversions.  This option is necessary for transmitting  binary  files.

It may also be used to slightly boost efficiency in Unix-to-Unix trans-

fers of text files by eliminating CRLF/newline conversion.

-w      Write-Protect -- Avoid filename collisions for incoming files.

-q      Quiet -- Suppress screen update during file transfer, for  instance  to

allow a file transfer to proceed in the background.

-d      Debug -- Record debugging information in the file debug.log in the cur-

rent directory.  Use this option if you believe  the  program  is  mis-

behaving, and show the resulting log to your local kermit maintainer.

-h      Help -- Display a brief synopsis of the command line options.

The command line may contain no more than one protocol action option.

Files  are  sent with their own names, except that lowercase letters are raised

UNIX KERMIT                                                            Page 124

to upper, pathnames are stripped off, certain special characters like (`~') and

(`#')  are changed to `X', and if the file name begins with a period, an `X' is

inserted before it.  Incoming files are stored under  their  own  names  except

that  uppercase  letters  are  lowered, and, if -w was specified, a "generation

number" is appended to the name if it has the same name  as  an  existing  file

which  would  otherwise be overwritten.  If the -a option is included, then the

same rules apply to its argument.  The file transfer display shows  any  trans-

formations performed upon filenames.

During transmission, files are encoded as follows:

- Control characters are converted to prefixed printables.

- Sequences  of repeated characters are collapsed via repeat counts, if

the other Kermit is also capable of repeated-character compression.

- If parity is being used on the communication  line,  data  characters

with  the  8th  (parity)  bit on are specially prefixed, provided the

other Kermit is capable of 8th-bit prefixing; if  not,  8-bit  binary

files cannot be successfully transferred.

- Conversion is done between Unix newlines and carriage-return-linefeed

sequences unless the -i option was specified.

Command Line Examples:

kermit -l /dev/ttyi5 -b 1200 -cn -r

This command connects you to the system on the other end of ttyi5 at 1200 baud,

where  you  presumably  log in and run Kermit with a 'send' command.  After you

escape back, C-Kermit waits for a file (or files) to arrive.    When  the  file

transfer is completed, you are again connected to the remote system so that you

can logout.

kermit -l /dev/ttyi4 -b 1800 -cntp m -r -a foo

This command is like the preceding one, except the remote system in  this  case

uses  half  duplex communication with mark parity.  The first file that arrives

is stored under the name foo.

kermit -l /dev/ttyi6 -b 9600 -c | tek

This example uses Kermit to connect your terminal to the system  at  the  other

end of ttyi6.  The C-Kermit terminal connection does not provide any particular

terminal  emulation,  so  C-Kermit's  standard   i/o   is   piped   through   a

(hypothetical) program called tek, which performs (say) Tektronix emulation.

kermit -l /dev/ttyi6 -b 9600 -nf

This command would be used to shut down a remote server and then connect to the

remote system, in order to log out or to make further use of it.  The -n option

UNIX KERMIT                                                            Page 125

is invoked after -f (-c would have been invoked before).

kermit -l /dev/ttyi6 -b 9600 -qg foo.\* &

This  command  causes C-Kermit to be invoked in the background, getting a group

of files from a remote server (note the quoting of the `*' character).  No dis-

play  occurs  on  the  screen, and the keyboard is not sampled for interruption

commands.  This allows other work to be done while file  transfers  proceed  in

the background.

kermit -l /dev/ttyi6 -b 9600 -g foo.\* > foo.log < /dev/null &

This  command  is  like  the previous one, except the file transfer display has

been redirected to the file foo.log.  Standard input  is  also  redirected,  to

prevent C-Kermit from sampling it for interruption commands.

kermit -iwx

This  command  starts  up  C-Kermit as a server.  Files are transmitted with no

newline/carriage-return-linefeed conversion; the -i option is necessary for bi-

nary  file transfer and useful for Unix-to-Unix transfers.  Incoming files that

have the same names as existing files are given new, unique names.

kermit -l /dev/ttyi6 -b 9600

This command sets the communication  line  and  speed.    Since  no  action  is

specified,  C-Kermit issues a prompt and enters an interactive dialog with you.

Any settings given on the command line remain in force during the  dialog,  un-

less explicitly changed.

kermit

This command starts up Kermit interactively with all default settings.

The  next  example shows how Unix Kermit might be used to send an entire direc-

tory tree from one Unix system to another, using the tar  program  as  Kermit's

standard  input and output.  On the orginating system, in this case the remote,

type (for instance):

tar cf - /usr/fdc | kermit -is -

This causes tar to send the directory /usr/fdc (and all its files and  all  its

subdirectories and all their files...) to standard output instead of to a tape;

kermit receives this as standard input and sends it as a binary file.   On  the

receiving system, in this case the local one, type (for instance):

kermit -il /dev/ttyi5 -b 9600 -k | tar xf -

Kermit  receives  the  tar archive, and sends it via standard output to its own

UNIX KERMIT                                                            Page 126

copy of tar, which extracts from it a replica of the original directory tree.

A final example shows how a Unix compression utility might be used to speed  up

Kermit file transfers:

compress file | kermit -is -     (sender)

kermit -ik | uncompress          (receiver)

Exit Status Codes:

Unix  Kermit  returns  an exit status of zero, except when a fatal error is en-

countered, where the exit status is set to  one.    With  background  operation

(e.g., `&' at end of invoking command line) driven by scripted interactive com-

mands (redirected standard input and/or take  files),  any  failed  interactive

command (such as failed dial or script attempt) causes the fatal error exit.

9.4. Interactive Operation

C-Kermit's  interactive  command  prompt  is  "C-Kermit>".  In response to this

prompt, you may type any valid command.  C-Kermit executes the command and then

prompts  you for another command.  The process continues until you instruct the

program to terminate.

Commands begin with a keyword, normally an English verb, such as "send".    You

may  omit  trailing  characters  from  any keyword, so long as you specify suf-

ficient characters to distinguish it from  any  other  keyword  valid  in  that

field.    Certain commonly-used keywords (such as "send", "receive", "connect")

have special non-unique abbreviations ("s" for "send", "r" for  "receive",  "c"

for "connect").

Certain  characters  have  special  functions during typein of interactive com-

mands:

?   Question mark, typed at any point in a command, will produce a  message

explaining  what  is  possible or expected at that point.  Depending on

the context, the message may be a brief phrase, a menu of keywords,  or

a list of files.

ESC (The  Escape  or  Altmode  key)  --  Request  completion of the current

keyword or filename, or insertion of a default value.  The result  will

be a beep if the requested operation fails.

DEL (The  Delete  or  Rubout key) -- Delete the previous character from the

command.  You may also use BS (Backspace, Control-H) for this function.

^W  (Control-W) -- Erase the rightmost word from the command line.

^U  (Control-U) -- Erase the entire command.

^R  (Control-R) -- Redisplay the current command.

SP  (Space) -- Delimits fields (keywords, filenames, numbers) within a com-

mand.  HT (Horizontal Tab) may also be used for this purpose.

UNIX KERMIT                                                            Page 127

CR  (Carriage  Return)  -- Enters the command for execution.  LF (Linefeed)

or FF (formfeed) may also be used for this purpose.

\   (Backslash) -- Enter any of the  above  characters  into  the  command,

literally.    To enter a backslash, type two backslashes in a row (\\).

A backslash at the end of a command line causes the  next  line  to  be

treated  as a continuation line; this is useful for readability in com-

mand files, especially in the 'script' command.

You may type the editing characters (DEL, ^W, etc) repeatedly,  to  delete  all

the  way  back to the prompt.  No action will be performed until the command is

entered by typing carriage return, linefeed, or formfeed.  If you make any mis-

takes,  you  will receive an informative error message and a new prompt -- make

liberal use of `?' and ESC to feel your way through the commands.   One  impor-

tant command is "help" -- you should use it the first time you run C-Kermit.

A command line beginning with a percent sign "%" is ignored.  Such lines may be

used to include illustrative commentary in Kermit command dialogs.

Interactive C-Kermit accepts commands from files as well as from the  keyboard.

When  you enter interactive dialog, C-Kermit looks for the file .kermrc in your

home or current directory (first it looks in the home directory,  then  in  the

current  one) and executes any commands it finds there.  These commands must be

in interactive format, not Unix command-line format.  A "take" command is  also

provided  for use at any time during an interactive session.  Command files may

be nested to any reasonable depth.

Here is a brief list of C-Kermit interactive commands:

!  Execute a Unix shell command, or start a shell.

bye  Terminate and log out a remote Kermit server.

close  Close a log file.

connect  Establish a terminal connection to a remote system.

cwd  Change Working Directory.

dial  Dial a telephone number.

directory  Display a directory listing.

echo  Display arguments literally.

exit  Exit from the program, closing any open files.

finish  Instruct a remote Kermit server to exit, but not log out.

get  Get files from a remote Kermit server.

help  Display a help message for a given command.

log  Open a log file -- debugging, packet, session, transaction.

quit  Same as 'exit'.

receive  Passively wait for files to arrive.

remote  Issue file management commands to a remote Kermit server.

script  Execute a login script with a remote system.

send  Send files.

server  Begin server operation.

set  Set various parameters.

show  Display values of 'set' parameters.

space  Display current disk space usage.

statistics  Display statistics about most recent transaction.

take  Execute commands from a file.

The 'set' parameters are:

UNIX KERMIT                                                            Page 128

block-check  Level of packet error detection.

delay  How long to wait before sending first packet.

duplex  Specify which side echoes during 'connect'.

escape-character  Prefix for "escape commands" during 'connect'.

file  Set various file parameters.

flow-control  Communication line full-duplex flow control.

handshake  Communication line half-duplex turnaround character.

incomplete  Disposition for incompletely received files.

line  Communication line device name.

modem-dialer  Type of modem-dialer on communication line.

parity  Communication line character parity.

prompt  The C-Kermit program's interactive command prompt.

receive  Parameters for inbound packets.

send  Parameters for outbound packets.

speed  Communication line speed.

The 'remote' commands are:

cwd  Change remote working directory.

delete  Delete remote files.

directory  Display a listing of remote file names.

help  Request help from a remote server.

host  Issue a command to the remote host in its own command language

space  Display current disk space usage on remote system.

type  Display a remote file on your screen.

who  Display who's logged in, or get information about a user.

Most of these commands are described adequately in the Kermit User Guide.  Spe-

cial aspects of certain Unix Kermit commands are described below.

THE 'SEND' COMMAND

Syntax:  send fn  - or -  send fn1 rfn1

Send  the file or files denoted by fn to the other Kermit, which should be run-

ning as a server, or which should be given the 'receive' command.  Each file is

sent  under  its own name (as described above, or as specified by the 'set file

names' command).  If the second form of the 'send' command is used, i.e.   with

fn1 denoting a single Unix file, rfn1 may be specified as a name to send it un-

der.  The 'send' command may be abbreviated to 's', even though 's'  is  not  a

unique abbreviation for a top-level C-Kermit command.

The wildcard (meta) characters `*' and `?' are accepted in fn.  If `?' is to be

included, it must be prefixed by `\' to override its normal function of provid-

ing  help.    `*'  matches any string, `?' matches any single character.  Other

notations for file groups, like `[a-z]og', are  not  available  in  interactive

commands  (though  of  course they are available on the command line).  When fn

contains `*' or `?' characters, there is a limit to the number  of  files  that

can  be  matched,  which  varies from system to system.  If you get the message

"Too many files match" then you'll have to make a more judicious selection.  If

fn was of the form

usr/longname/anotherlongname/*

then  C-Kermit's  string  space  will  fill  up rapidly -- try doing a cwd (see

UNIX KERMIT                                                            Page 129

below) to the path in question and reissuing the command.

Note -- C-Kermit sends only from the current or specified directory.   It  does

not traverse directory trees.  If the source directory contains subdirectories,

they will be skipped.  Conversely, C-Kermit does not  create  directories  when

receiving  files.    If  you  have  a need to do this, you can pipe tar through

C-Kermit, as shown in the example on page 125, or under System III/V  Unix  you

can use cpio.

Another  Note  --  C-Kermit does not skip over "invisible" files that match the

file specification; Unix systems usually treat files whose names start  with  a

dot (like .login, .cshrc, and .kermrc) as invisible.  Similarly for "temporary"

files whose names start with "#".

THE 'RECEIVE' COMMAND

Syntax:  receive  - or -  receive fn1

Passively wait for files to arrive from the other Kermit, which must  be  given

the 'send' command -- the 'receive' command does not work in conjunction with a

server (use 'get' for that).  If fn1 is specified,  store  the  first  incoming

file under that name.  The 'receive' command may be abbreviated to 'r'.

THE 'GET' COMMAND:

Syntax:  get rfn

or: get

rfn

fn1

Request a remote Kermit server to send the named file or files.  Since a remote

file specification (or list)  might  contain  spaces,  which  normally  delimit

fields  of  a C-Kermit command, an alternate form of the command is provided to

allow the inbound file to be given a new name: type 'get' alone on a line,  and

you  will  be prompted separately for the remote and local file specifications,

for example

C-Kermit>get

Remote file specification: profile exec

Local name to store it under: profile.exec

As with 'receive', if more than one file arrives as a result of the 'get'  com-

mand,  only the first will be stored under the alternate name given by fn1; the

remaining files will be stored under their own names if possible.  If a `?'  is

to be included in the remote file specification, you must prefix it with `\' to

suppress its normal function of providing help.

If you have started a multiline 'get' command, you may escape from  its  lower-

level prompts by typing a carriage return in response to the prompt, e.g.

C-Kermit>get

Remote file specification: foo

Local name to store it under: (Type a carriage return here)

UNIX KERMIT                                                            Page 130

(cancelled)

C-Kermit>

THE 'SERVER' COMMAND:

The 'server' command places C-Kermit in "server mode" on the currently selected

communication line.  All further commands must arrive as valid  Kermit  packets

from  the  Kermit  on  the  other  end of the line.  The Unix Kermit server can

respond to the following commands:

Command                Server Response

get                    Sends files

send                   Receives files

bye                    Attempts to log itself out

finish                 Exits to level from which it was invoked

remote directory       Sends directory lising

remote delete          Removes files

remote cwd             Changes working directory

remote type            Sends files to your screen

remote space           Reports about its disk usage

remote who             Shows who's logged in

remote host            Executes a Unix shell command

remote help            Lists these capabilities

Note that the Unix Kermit server cannot always respond to a BYE  command.    It

will  attempt to do so using "kill()", but this will not work on all systems or

under all conditions.

If the Kermit server is directed at an external line  (i.e.  it  is  in  "local

mode")  then  the console may be used for other work if you have 'set file dis-

play off'; normally the program expects the console to be used to observe  file

transfers  and  enter  status queries or interruption commands.  The way to get

C-Kermit into background operation from interactive command level  varies  from

system  to system (e.g. on Berkeley Unix you would halt the program with ^Z and

then use the C-Shell 'bg' command to continue it in the background).  The  more

common method is to invoke the program with the desired command line arguments,

including "-q", and with a terminating "&".

When the Unix Kermit server is given a 'remote host' command,  it  executes  it

using  the  shell  invoked  upon  login,  e.g. the Bourne shell or the Berkeley

C-Shell.

THE 'REMOTE', 'BYE', AND 'FINISH' COMMANDS:

C-Kermit may itself request services from a remote Kermit server.  In  addition

to 'send' and 'get', the following commands may also be sent from C-Kermit to a

Kermit server:

remote cwd [directory]

If the optional remote directory specification is included, you will be

prompted on a separate line for a password, which will not echo as  you

type it.

remote delete rfn       delete remote file or files.

UNIX KERMIT                                                            Page 131

remote directory [rfn]  directory listing of remote files.

remote host command     command in remote host's own command language.

remote space            disk usage report from remote host.

remote type [rfn]       display remote file or files on the screen.

remote who [user]       display information about who's logged in.

remote help             display remote server's capabilities.

bye and finish:

When connected to a remote Kermit  server,  these  commands  cause  the

remote  server  to  terminate;  'finish' returns it to Kermit or system

command level (depending on the implementation or how the  program  was

invoked); 'bye' also requests it to log itself out.

THE 'LOG' AND 'CLOSE' COMMANDS:

Syntax: log {debugging, packets, session, transactions} [ fn1 ]

C-Kermit's  progress  may be logged in various ways.  The 'log' command opens a

log, the 'close' command closes it.  In addition, all open logs are  closed  by

the 'exit' and 'quit' commands.  A name may be specified for a log file; if the

name is omitted, the file is created with a default name as shown below.

log debugging

This produces a voluminous log of the internal workings of C-Kermit, of use

to Kermit developers or maintainers in tracking down suspected bugs in  the

C-Kermit  program.   Use of this feature dramatically slows down the Kermit

protocol.  Default name: debug.log.

log packets

This  produces  a  record of all the packets that go in and out of the com-

munication port.  This log is of use to Kermit maintainers who are tracking

down  protocol  problems  in either C-Kermit or any Kermit that C-Kermit is

connected to.  Default name:  packet.log.

log session

This  log  will  contain a copy of everything you see on your screen during

the 'connect' command, except for local messages or interaction with  local

escape commands.  Default name:  session.log.

log transactions

The transaction log is a record of all the files that were sent or received

while  transaction  logging  was  in  effect.   It includes time stamps and

statistics, filename transformations, and records of any  errors  that  may

have occurred.  The transaction log allows you to have long unattended file

transfer sessions without  fear  of  missing  some  vital  screen  message.

Default name:  transact.log.

The 'close' command explicitly closes a log, e.g. 'close debug'.

Note:    Debug  and Transaction logs are a compile-time option; C-Kermit may be

compiled without these logs, in which case it will run faster, it will take  up

less space on the disk, and the commands relating to them will not be present.

UNIX KERMIT                                                            Page 132

LOCAL FILE MANAGEMENT COMMANDS:

Unix  Kermit  allows some degree of local file management from interactive com-

mand level:

directory [fn]

Displays  a listing of the names, modes, sizes, and dates of files matching

fn (which defaults to `*').  Equivalent to `ls -l'.

cwd [directory-name]

Changes  Kermit's  working  directory  to  the one given, or to the default

directory if the directory name is omitted.  This command affects only  the

Kermit process and any processes it may subsequently create.

space

Display information about disk space and/or quota in the current  directory

and device.

! [command]

The command is executed by the Unix shell.  If  no  command  is  specified,

then  an  interactive  shell  is  started;  exiting from the shell, e.g. by

typing Control-D, will return you to C-Kermit command level.  C-Kermit  at-

tempts  to  use  your  preferred,  customary shell.  Use the `!' command to

provide file management or  other  functions  not  explicitly  provided  by

C-Kermit commands.  The `!'  command has certain peculiarities:

- At least one space must separate the '!' from the shell command.

- A 'cd' command executed in this manner will have no effect -- use

the C-Kermit 'cwd' command instead.

THE 'SET' AND 'SHOW' COMMANDS:

Since Kermit is designed to allow diverse systems to communicate, it  is  often

necessary  to  issue  special  instructions  to  allow  the program to adapt to

peculiarities of the another system or the communication path.  These  instruc-

tions are accomplished by the 'set' command.  The 'show' command may be used to

display current settings.  Here is a brief synopsis of  settings  available  in

the current release of C-Kermit:

block-check {1, 2, 3}

Determines the level of per-packet error  detection.    "1"  is  a  single-

character  6-bit  checksum,  folded  to include the values of all bits from

each character.   "2"  is  a  2-character,  12-bit  checksum.    "3"  is  a

3-character,  16-bit  cyclic  redundancy check (CRC).  The higher the block

check, the better the error detection and correction  and  the  higher  the

resulting  overhead.   Type 1 is most commonly used; it is supported by all

Kermit implementations, and it has proven adequate in  most  circumstances.

Types  2  or  3  would  be used to advantage when transferring 8-bit binary

files over noisy lines.

delay n

How  many  seconds  to  wait before sending the first packet after a 'send'

command.  Used in remote mode to give you time to escape back to your local

Kermit and issue a 'receive' command.  Normally 5 seconds.

UNIX KERMIT                                                            Page 133

duplex {full, half}

For use during 'connect'.  Specifies  which  side  is  doing  the  echoing;

'full' means the other side, 'half' means C-Kermit must echo typein itself.

escape-character cc

For use during 'connect' to get C-Kermit's attention.  The escape character

acts  as a prefix to an 'escape command', for instance to close the connec-

tion and return to C-Kermit or Unix  command  level.    The  normal  escape

character  is Control-Backslash (28).  The escape character is also used in

System III/V implementations  to  prefix  interrupt  commands  during  file

transfers.

file {display, names, type, warning}

Establish various file-related parameters:

display {on, off}

Normally  'on';  when in local mode, display progress of file transfers

on the screen (stdout), and listen to the keyboard (stdin)  for  inter-

ruptions.    If  off (-q on command line) none of this is done, and the

file transfer may proceed in the background oblivious to any other work

concurrently done at the console terminal.

names {converted, literal}

Normally converted,  which  mean  that  outbound  filenames  have  path

specifications  stripped, lowercase letters raised to upper, tildes and

extra periods changed to X's, and an X inserted in front  of  any  name

that  starts  with  period.    Incoming  files  have  uppercase letters

lowered.  Literal means that none of these conversions are done; there-

fore,  any  directory  path  appearing in a received file specification

must exist and be write-accessible.  When literal naming is being used,

the  sender  should not use path names in the file specification unless

the same path exists on the target system and is writable.

type {binary, text}

Normally text, which means that conversion is done between Unix newline

characters and the carriage-return/linefeed sequences required  by  the

canonical  Kermit  file  transmission format, and in common use on non-

Unix systems.  Binary means to transmit file contents  without  conver-

sion.    Binary (`-i' in command line notation) is necessary for binary

files, and desirable in all Unix-to-Unix transactions to  cut  down  on

overhead.

warning {on, off}

Normally off, which means that incoming files will  silently  overwrite

existing files of the same name.  When on (`-w' on command line) Kermit

will check if an arriving file would overwrite an existing file; if so,

it  will construct a new name for the arriving file, of the form foo~n,

where foo is the name they share and n is a "generation number"; if foo

exists,  then  the new file will be called foo~1.  If foo and foo~1 ex-

ist, the new file will be foo~2, and so on.  If the new name  would  be

longer than the maximum length for a filename, then characters would be

deleted from the end first, for instance, thelongestname  on  a  system

with a limit of 14 characters would become thelongestn~1.

CAUTION:  If Control-F or Control-B is used to cancel an incom-

ing file, and a file of the same name previously  existed,  and

UNIX KERMIT                                                            Page 134

the  "file  warning"  feature is not enabled, then the previous

copy of the file will disappear.

flow-control {none, xon/xoff}

Normally xon/xoff for full duplex flow control.  Should be set to 'none' if

the other system cannot do xon/xoff flow control, or if you have  issued  a

'set  handshake' command.  If set to xon/xoff, then handshake should be set

to none.  This setting applies during both  terminal  connection  and  file

transfer.

incomplete {discard, keep}

Disposition for incompletely received files.  If an incoming file is inter-

rupted  or  an  error occurs during transfer, the part that was received so

far is normally discarded.  If you "set incomplete  keep"  then  such  file

fragments will be kept.

handshake {xon, xoff, cr, lf, bell, esc, none}

Normally none.  Otherwise, half-duplex communication line turnaround  hand-

shaking  is  done, which means Unix Kermit will not reply to a packet until

it has received the indicated handshake character or has timed out  waiting

for  it;  the  handshake setting applies only during file transfer.  If you

set handshake to other than none, then flow should be set to none.

line [device-name]

The device name for the communication line to be used for file transfer and

terminal connection, e.g. /dev/ttyi3.  If you specify a device name, Kermit

will be in local mode, and you should remember to issue any other necessary

'set' commands, such as 'set speed'.  If you omit the device  name,  Kermit

will  revert  to  its  default mode of operation.  If you specify /dev/tty,

Kermit will enter remote mode (useful when  logged  in  through  the  "back

port"  of  a  system normally used as a local-mode workstation).  When Unix

Kermit enters local mode, it attempts to synchronize  with  other  programs

(like  uucp)  that  use  external  communication lines so as to prevent two

programs using the same  line  at  once;  before  attempting  to  lock  the

specified  line,  it  will  close  and  unlock  any  external line that was

previously in use.  The method used for locking is the  "uucp  lock  file",

explained in more detail later.

modem-dialer {direct, hayes, racalvadic, ventel, ...}

The type of modem dialer on the communication line.  "Direct" indicates ei-

ther there is no dialout modem, or that if the line requires carrier detec-

tion to open, then 'set line' will  hang  waiting  for  an  incoming  call.

"Hayes",  "Ventel",  and  the  others  indicate  that 'set line' (or the -l

argument) will prepare for  a  subsequent  'dial'  command  for  the  given

dialer.    Support for new dialers is added from time to time, so type 'set

modem ?' for a list of those supported in your copy of  Kermit.    See  the

description of the 'dial' command

parity {even, odd, mark, space, none}

Specify character parity for use in packets and terminal  connection,  nor-

mally  none.    If  other  than none, C-Kermit will seek to use the 8th-bit

prefixing mechanism for transferring 8-bit binary data, which can  be  used

successfully  only  if  the  other Kermit agrees; if not, 8-bit binary data

cannot be successfully transferred.

prompt [string]

UNIX KERMIT                                                            Page 135

The  given  string  will  be  substituted for "C-Kermit>" as this program's

prompt.  If the string is omitted, the prompt will revert  to  "C-Kermit>".

If  the string is enclosed in doublequotes, the quotes will be stripped and

any leading and trailing blanks will be retained.

send parameter

Establish  parameters to use when sending packets.  These will be in effect

only for the initial packet sent, since the other Kermit may override these

parameters during the protocol parameter exchange (unless noted below).

end-of-packet cc

Specifies the control character needed by the other Kermit to recognize

the  end of a packet.  C-Kermit sends this character at the end of each

packet.  Normally 13 (carriage return),  which  most  Kermit  implemen-

tations  require.    Other  Kermits require no terminator at all, still

others may require a different terminator, like linefeed (10).

packet-length n

Specify  the  maximum  packet  length  to  send.  Normally 90.  Shorter

packet lengths can be useful on noisy lines, or with systems  or  front

ends  or networks that have small buffers.  The shorter the packet, the

higher the overhead, but the lower the chance of a  packet  being  cor-

rupted  by  noise,  and  the less time to retransmit corrupted packets.

This command overrides the value requested by the other  Kermit  during

protocol initiation.

pad-character cc

Designate a character to send before each packet.   Normally,  none  is

sent.    Outbound padding is sometimes necessary for communicating with

slow half duplex systems that provide no other means of line turnaround

control.   It can also be used to send special characters to communica-

tions equipment that needs to be put  in  "transparent"  or  "no  echo"

mode,  when this can be accomplished in by feeding it a certain control

character.

padding n

How many pad characters to send, normally 0.

start-of-packet cc

The normal Kermit packet prefix is Control-A (1); this command  changes

the  prefix  C-Kermit  puts on outbound packets.  The only reasons this

should ever be changed would be: Some piece of equipment somewhere  be-

tween  the  two  Kermit programs will not pass through a Control-A; or,

some piece of of equipment similarly placed is echoing its input.    In

the  latter  case,  the recipient of such an echo can change the packet

prefix for outbound packets to be different from that of arriving pack-

ets,  so  that the echoed packets will be ignored.  The opposite Kermit

must also be told to change the prefix for its inbound packets.

timeout n

Specifies the number of seconds you want the other Kermit to wait for a

packet before timing it out and requesting retransmission.

receive parameter

Establish  parameters to request the other Kermit to use when sending pack-

ets.

UNIX KERMIT                                                            Page 136

end-of-packet cc

Requests  the  other Kermit to terminate its packets with the specified

character.

packet-length n

Specify  the maximum packet length to that you want the other Kermit to

send.  Normally 90.

pad-character cc

C-Kermit  normally does not need to have incoming packets preceded with

pad characters.  This command allows C-Kermit to request the other Ker-

mit to use cc as a pad character.  Default cc is NUL, ASCII 0.

padding n

How many pad characters to ask for, normally 0.

start-of-packet cc

Change  the  prefix C-Kermit looks for on inbound packets to correspond

with what the other Kermit is sending.

timeout n

Normally, each Kermit partner sets its packet timeout interval based on

what the opposite Kermit requests.  This command allows you to override

the  normal procedure and specify a timeout interval for Unix Kermit to

use when waiting for packets from the other Kermit.  If you specify  0,

then  no timeouts will occur, and Unix Kermit will wait forever for ex-

pected packets to arrive.

speed {0, 110, 150, 300, 600, 1200, 1800, 2400, 4800, 9600}

The  baud rate for the external communication line.  This command cannot be

used to change the speed of your own console terminal.  Many  Unix  systems

are set up in such a way that you must give this command after a 'set line'

command before you can use the line.  'set baud'  is  a  synomym  for  'set

speed'.

THE 'SHOW' COMMAND:

Syntax: show {parameters, versions}

The  "show"  command  with  the  default  argument of "parameters" displays the

values of all the  'set'  parameters  described  above.    If  you  type  "show

versions",  then C-Kermit will display the version numbers and dates of all its

internal modules.  You should use the "show versions" command to ascertain  the

vintage of your Kermit program before reporting problems to Kermit maintainers.

THE 'STATISTICS' COMMAND:

The  statistics  command  displays  information  about  the  most recent Kermit

protocol transaction, including file and communication line i/o, timing and ef-

ficiency,  as  well  as  what  encoding options were in effect (such as 8th-bit

prefixing, repeat-count compression).

UNIX KERMIT                                                            Page 137

THE 'TAKE' AND 'ECHO' COMMANDS:

Syntax: take fn1

echo [text to be echoed]

The 'take' command instructs C-Kermit to execute commands from the named  file.

The  file may contain any interactive C-Kermit commands, including 'take'; com-

mand files may be nested to any reasonable depth.  The 'echo'  command  may  be

used  within command files to issue greetings, announce progress, ring the ter-

minal bell, etc.

The 'echo' command should not be confused with the Unix 'echo'  command,  which

can  be  used  to  show how meta characters would be expanded.  The Kermit echo

command simply displays its text argument (almost) literally at  the  terminal;

the  argument may contain octal escapes of the form "\ooo", where o is an octal

digit (0-7), and there may be 1, 2, or 3 such digits, whose  value  specify  an

ASCII  character,  such  as "\007" (or "\07" or just "\7") for beep, "\012" for

newline, etc.  Of course, each backslash must be must be entered twice in order

for it to be passed along to the echo command by the Kermit command parser.

Take-command  files  are  in  exactly  the same syntax as interactive commands.

Note that this implies that if you want  to  include  special  characters  like

question  mark  or  backslash  that you would have to quote with backslash when

typing interactive commands, you must quote these characters the  same  way  in

command  files.  Long lines may be continued by ending them with a single back-

slash.

Command files may be used in lieu of command macros, which have not been imple-

mented in this version of C-Kermit.  For instance, if you commonly connect to a

system called 'B' that is connected to ttyh7 at 4800 baud, you could  create  a

file called b containing the commands

% C-Kermit command file to connect to System B thru /dev/ttyh7

set line /dev/ttyh7

set speed 4800

% Beep and give message

echo \\007Connecting to System B...

connect

and  then simply type 'take b' (or 't b' since no other commands begin with the

letter 't') whenever you wish to connect to system B. Note  the  comment  lines

and the beep inserted into the 'echo' command.

For  connecting  to  IBM  mainframes,  a number of 'set' commands are required;

these, too, are conveniently collected into a 'take' file like this one:

% Sample C-Kermit command file to set up current line

% for IBM mainframe communication

%

set parity mark

set handshake xon

set flow-control none

set duplex half

Note that no single command is available to wipe out all of these settings  and

return  C-Kermit  to its default startup state; to do that, you can either res-

UNIX KERMIT                                                            Page 138

tart the program, or else make a command file that executes the necessary 'set'

commands:

% Sample C-Kermit command file to restore normal settings

%

set parity none

set handshake none

set flow-control xon/xoff

set duplex full

An  implicit  'take' command is executed upon your .kermrc file upon C-Kermit's

initial entry into interactive dialog.  The .kermrc file should  contain  'set'

or  other  commands  you  want to be in effect at all times.  For instance, you

might want override the default action when incoming files have the same  names

as existing files -- in that case, put the command

set file warning on

in  your  .kermrc  file.  On some non-Unix systems that run C-Kermit, this file

might have a different name, such as kermit.ini.

NOTE: The initialization file is currently not processed if  Kermit  is

invoked  with an action command from the command line.  The same effect

can be achieved, however, by defining an alias or shell procedure  that

starts up Kermit with the desired command line options.

Commands  executed from take files are not echoed at the terminal.  If you want

to see the commands as well as their output, you could feed the command file to

C-Kermit via redirected stdin, as in

'kermit < cmdfile'

Errors  encountered during execution of take files (such as failure to complete

dial or script operations) cause termination of the current take file,  popping

to  the  level  that  invoked  it (take file, interactive level, or the shell).

When kermit is executed in the background, errors during execution  of  a  take

file are fatal.

THE 'CONNECT' COMMAND:

The connect command links your terminal to another computer as if it were a lo-

cal terminal to that computer, through the device specified in the most  recent

'set  line'  command,  or  through the default device if your system is a PC or

workstation.  All characters you type at your keyboard are sent  out  the  com-

munication  line,  all  characters  arriving at the communication port are dis-

played on your screen.  Current settings of speed, parity,  duplex,  and  flow-

control  are  honored.   If you have issued a 'log session' command, everything

you see on your screen will also  be  recorded  to  your  session  log.    This

provides  a way to "capture" files from systems that don't have Kermit programs

available.

To get back to your own system, you must type the escape  character,  which  is

Control-Backslash  (^\)  unless  you have changed it with the 'set escape' com-

mand,  followed  by  a  single-character  command,  such  as  'c'  for   "close

connection".  Single-character commands include:

UNIX KERMIT                                                            Page 139

c     Close the connection

b     Send a BREAK signal

0     (zero) send a null

s     Give a status report about the connection

h     Hangup the phone

^\    Send  Control-Backslash  itself  (whatever  you have defined the escape

character to be, typed twice in a row sends one copy of it).

Uppercase and control equivalents for these letters are also accepted.  A space

typed  after the escape character is ignored.  Any other character will produce

a beep.

The connect command simply displays incoming characters on the screen.   It  is

assumed  any  screen  control sequences sent by the host will be handled by the

firmware in your terminal or PC.  If terminal emulation is  desired,  then  the

connect  command  can  invoked  from  the  Unix  command line (-c or -n), piped

through a terminal emulation filter, e.g.

kermit -l /dev/acu -b 1200 -c | tek

'c' is an acceptable non-unique abbreviation for 'connect'.

THE 'DIAL' COMMAND:

Syntax: dial telephone-number-string

This command controls dialout modems; you should have  already  issued  a  "set

line"  and  "set  speed"  command  to  identify the terminal device, and a "set

modem" command to identify the type of modem to be used for dialing.    In  the

"dial"  command, you supply the phone number and the Kermit program feeds it to

the modem in the appropriate format and then interprets dialer return codes and

modem  signals  to  inform  you whether the call was completed.  The telephone-

number-string may contain imbedded modem-dialer commands,  such  as  comma  for

Hayes  pause, or `&' for Ventel dialtone wait and `%' for Ventel pause (consult

your modem manual for details).

At the time of this writing, support is included for the following modems:

- Cermetek Info-Mate 212A

- DEC DF03-AC

- DEC DF100 Series

- DEC DF200 Series

- General DataComm 212A/ED

- Hayes Smartmodem 1200 and compatibles

- Penril

- Racal Vadic

- US Robotics 212A

- Ventel

Support for new modems is added to the program from time to time; you can check

the current list by typing "set modem ?".

The  device  used  for  dialing out is the one selected in the most recent "set

line" command (or on a workstation, the default line if no "set  line"  command

was  given).   The "dial" command calls locks the path (see the section on line

UNIX KERMIT                                                            Page 140

locking below) and establishes a call on an exclusive basis.  If it is  desired

to  dial  a  call and then return to the shell (such as to do kermit activities

depending on standard in/out redirection), it is necessary to place the  dialed

call  under  one  device  name  (say, "/dev/cua0") and then escape to the shell

within Kermit on a linked device which is separate from the dialed  line  (say,

"/dev/cul0").    This  is the same technique used by uucp (to allow locks to be

placed separately for dialing and conversing).

Because modem dialers have strict requirements to override  the  carrier-detect

signal most Unix implementations expect, the sequence for dialing is more rigid

than most other C-Kermit procedures.

Example one:

kermit -l /dev/cul0 -b 1200

C-Kermit>set modem-dialer hayes    hint: abbreviate set m h

C-Kermit>dial 9,5551212

Connected!

C-Kermit>connect                   hint: abbreviate c

logon, request remote server, etc.

C-Kermit> ...

C-Kermit>quit                      hint: abbreviate q

this disconnects modem, and unlocks line.

Example two:

kermit

C-Kermit>set modem-dialer ventel

C-Kermit>set line /dev/cul0

C-Kermit>dial 9&5551212%

Connected!

C-Kermit> ...

Example three:

kermit

C-Kermit>take my-dial-procedure

Connected!

file my-dial-procedure:

set modem hayes

set line /dev/tty99

dial 5551212

connect

For Hayes  dialers, two important switch settings are #1 and #6.  #1 should  be

up  so  that the DTR is only asserted when the line is 'open'.  #6 should be up

so carrier-detect functions properly.  Switches #2 (English versus digit result

codes) and #4 (Hayes echoes modem commands) may be in either position.

For any dialers in general, this Kermit program requires that the modem provide

the "carrier detect" signal when a call is in progress, and remove that  signal

when the call completes or the line drops.  If a switch setting is available to

force carrier detect, it should not be in that setting.  Secondly, this  Kermit

program requires that the modem track the computer's "data terminal ready" sig-

UNIX KERMIT                                                            Page 141

nal (DTR).  If a switch setting is available to simulate  DTR  asserted  within

the  modem, then it should not be in that setting.  Otherwise the modem will be

unable to hang up at the end of a call or when interrupts are received by  Ker-

mit.

If  you want to interrupt a dial command in progress (for instance, because you

just realize that you gave it the wrong number), type a Control-C to  get  back

to command level.

THE 'SCRIPT' COMMAND:

Syntax: script expect send [expect send] . . .

"expect" has the syntax: expect[-send-expect[-send-expect[...]]]

This  command facilitates logging into a remote system and/or invoking programs

or other facilities after login on a remote system.

This login script facility operates in a manner similar to that  commonly  used

by  the  Unix uucp System's "L.sys" file entries.  A login script is a sequence

of the form:

expect send [expect send] . . .

where expect is a prompt or message to be issued by the remote site,  and  send

is  the  string  (names,  numbers,  etc)  to  return.  The send may also be the

keyword EOT, to send Control-D, or BREAK, to send a break signal.   Letters  in

send may be prefixed by `~' to send special characters.  These are:

~b  backspace

~s  space

~q  `?'(trapped by Kermit's command interpreter)

~n  linefeed

~r  carriage return

~t  tab

~'  single quote

~~  tilde

~"  double quote

~x  XON (Control-Q)

~c  don't append a carriage return

~o[o[o]]  an octal character

~d  delay approx 1/3 second during send

~w[d[d]]  wait specified interval during expect, then time out

As  with  some uucp systems, sent strings are followed by ~r unless they have a

~c.

Only the last 7 characters in each expect are matched.  A null expect, e.g.  ~0

or two adjacent dashes, causes a short delay before proceeding to the next send

sequence.  A null expect always succeeds.

As with uucp, if the expect string does not arrive, the script  attempt  fails.

If  you  expect that a sequence might not arrive, as with uucp, conditional se-

quences may be expressed in the form:

UNIX KERMIT                                                            Page 142

-send-expect[-send-expect[...]]

where dashed sequences are followed as long as previous expects fail.  Timeouts

for expects can be specified using ~w; ~w with no arguments waits 15 seconds.

Expect/send  transactions  can  be  easily be debugged by logging transactions.

This records all exchanges, both expected and actual.

Note that `\' characters in login scripts, as in any other C-Kermit interactive

commands,  must  be doubled up.  A line may be ended with a single `\' for con-

tinuation.

Example one:

Using a modem, dial a UNIX host site.   Expect  "login"  (...gin),  and  if  it

doesn't come, simply send a null string with a ~r.  (Some Unixes require either

an EOT or a BREAK instead of the null sequence,  depending  on  the  particular

site's "logger" program.)  After providing user id and password, respond "x" to

a question-mark prompt, expect the Bourne shell "$" prompt (and send return  if

it  doesn't  arrive).   Then cd to directory kermit, and run the program called

"wermit", entering the interactive connect state after wermit is loaded.

set modem-dialer ventel

set line /dev/tty77

set baud 1200

dial 9&5551212

script gin:--gin:--gin: smith ssword: mysecret ~q x $--$ \

cd~skermit $ wermit

connect

Example two:

Using a modem, dial the Telenet network.  This network  expects  three  returns

with  slight  delays between them.  These are sent following null expects.  The

single return is here sent as a null string, with a return appended by default.

Four  returns  are  sent  to  be  safe before looking for the prompt.  Then the

Telenet id and password are entered.  Then telenet is instructed to connect  to

a  host  site  (c 12345).  The host has a data switch, and to "which system" it

responds "myhost".  This is followed by a TOPS-20 logon, and a request to  load

Kermit,  set even parity, and enter the server mode.  Files are then exchanged.

The commands are in a take file; note the continuation of the 'script'  command

onto several lines using the `\' terminator.

set modem-dialer hayes

set line /dev/cul0

set baud 1200

dial 9,5551212

set parity even

script ~0 ~0 ~0 ~0 ~0 ~0 ~0 ~0 @--@--@ id~saa001122 = 002211 @ \

c~s12345 ystem-c~s12345-ystem myhost @ joe~ssecret @ kermit \

> set~sparity~seven > server

send some.stuff

get some.otherstuff

bye

quit

UNIX KERMIT                                                            Page 143

Since  these  commands may be executed totally in the background, they can also

be scheduled.  A typical shell script, which might be scheduled by cron,  would

be as follows (csh used for this example):

#

#keep trying to dial and log onto remote host and exchange files

#wait 10 minutes before retrying if dial or script fail.

#

cd someplace

while ( 1 )

kermit < /tonight.cmd >> nightly.log &

if ( ! $status ) break

sleep 600

end

File  tonight.cmd  might  have two takes in it, for example, one to take a file

with the set modem, set line, set baud, dial, and script, and a second take  of

a  file  with  send/get  commands  for  the  remote  server.  The last lines of

tonight.cmd should be a bye and a quit.

THE 'HELP' COMMAND:

Syntax: help

or: help keyword

or: help {set, remote} keyword

Brief help messages or menus are always available at interactive command  level

by  typing  a question mark at any point.  A slightly more verbose form of help

is available through the 'help' command.  The 'help' command with no  arguments

prints  a  brief  summary of how to enter commands and how to get further help.

'help' may be followed by one of the top-level C-Kermit command keywords,  such

as  'send', to request information about a command.  Commands such as 'set' and

'remote' have a further level of help.  Thus you may type 'help',  'help  set',

or  'help  set parity'; each will provide a successively more detailed level of

help.

THE 'EXIT' AND 'QUIT' COMMANDS:

These two commands are identical.  Both of them do the following:

- Attempt to insure that the terminal is returned to normal.

- Relinquish access to any communication line assigned via 'set line'.

- Relinquish any uucp and multiuser locks on the communications line.

- Hang up the modem, if the communications line supports data  terminal

ready.

- Close any open log files.

After  exit  from C-Kermit, your default directory will be the same as when you

started the program.  The 'exit' command is issued implicitly whenever C-Kermit

halts normally, e.g. after a command line invocation, or after certain kinds of

interruptions.

UNIX KERMIT                                                            Page 144

9.5. UUCP Lock Files

Unix has no standard way of obtaining exclusive  access  to  an  external  com-

munication  line.    When you issue the 'set line' command to Unix Kermit, Unix

would normally grant you access to the line  even  if  some  other  process  is

making  use  of  it.    The  method adopted by most Unix systems to handle this

situation is the "UUCP lock  file".    UUCP,  the  Unix-to-Unix  Copy  program,

creates  a  file  in  its  directory  (usually /usr/spool/uucp, on some systems

/etc/locks) with a name like LCK..name, where name is the device name, for  in-

stance tty07.

Unix Kermit uses UUCP lock files in order to avoid conflicts with UUCP, tip, or

other programs that follow this convention.  Whenever you attempt to access  an

external  line using the 'set line' command or `-l' on the command line, Kermit

looks in the UUCP directory for a lock file corresponding to that device.   For

instance, if you 'set line /dev/ttyi6' then Kermit looks for the file

/usr/spool/uucp/LCK..ttyi6

If it finds this file, it gives you an error message and a directory listing of

the file so that you can see who is using it, e.g.

-r--r--r--  1 fdc        4 May  7 13:02 /usr/spool/uucp/LCK..ttyi6

In this case, you would look up user fdc to find out how soon the line will be-

come free.

This  convention  requires  that  the  uucp  directory be publicly readable and

writable.  If it is not, the program will issue an appropriate warning message,

but will allow you to proceed at your own risk (and the risk of anyone else who

might also be using the same line).

If no lock file is found, Unix Kermit will attempt create one, thus  preventing

anyone  who subsequently tries to run Kermit, UUCP, tip, or similar programs on

the same line from gaining access until you release the line.  If Kermit  could

not  create  the  lock  file (for instance because the uucp directory is write-

protected), then you will receive a warning message  but  will  be  allowed  to

proceed  at  your  -- and everyone else's -- risk.  When Kermit terminates nor-

mally, your lock file is removed.

Even when the lock directory is writable and readable,  the  locking  mechanism

depends  upon  all  users using the same name for the same device.  If a device

has more than one path associated with it, then a lock can be  circumvented  by

using an alias.

When a lock-creating program abruptly terminates, e.g. because it crashes or is

killed via shell  command,  the  lock  file  remains  in  the  uucp  directory,

spuriously  indicating  that  the line is in use.  If the lock file is owned by

yourself, you may remove it.  Otherwise, you'll have to get the  owner  or  the

system manager to remove it, or else wait for a system task to do so; uucp sup-

ports a function (uuclean) which removes these files after a predetermined  age

-- uucp sites tend to run this function periodically via crontab.

Locking  is  not needed, or used, if communications occur over the user's login

terminal line (normally /dev/tty).

UNIX KERMIT                                                            Page 145

It may be seen that line locking is fraught with peril.  It is included in Unix

Kermit  only because other Unix communication programs rely on it.  While it is

naturally desirable to assure exclusive access  to  a  line,  it  is  also  un-

desirable  to  refuse  access  to a vacant line only because of a spurious lock

file, or because the uucp directory is not appropriately protected.

9.6. C-Kermit under Berkeley or System III/V Unix:

C-Kermit may be interrupted at command level or during file transfer by  typing

Control-C.    The  program will perform its normal exit function, restoring the

terminal and releasing any lock.  If a protocol transaction was in progress, an

error  packet  will  be  sent  to  the opposite Kermit so that it can terminate

cleanly.

C-Kermit may be invoked in the background ("&" on shell commmand line).   If  a

background  process is "killed", the user will have to manually remove any lock

file and may need to restore the modem.    This  is  because  the  kill  signal

(kill(x,9)) cannot be trapped by Kermit.

During execution of a system command ('directory', 'cwd', or `!'), C-Kermit can

often be returned to command level by typing a single Control-C.  (With  System

III/V,  the  usual  interrupt  function  (often  the  DEL  key)  is replaced by

Control-C.)

Under Berkeley Unix only: C-Kermit may also be interrupted by  ^Z  to  put  the

process  in  the  background.   In this case the terminal is not restored.  You

will have to type Control-J followed by "reset" followed by  another  Control-J

to get your terminal back to normal.

Control-C, Control-Z, and Control-\ lose their normal functions during terminal

connection and also during file transfer when the controlling tty line is being

used for packet i/o.

If  you are running C-Kermit in "quiet mode" in the foreground, then interrupt-

ing the program with a console interrupt like Control-C will  not  restore  the

terminal  to  normal conversational operation.  This is because the system call

to enable console interrupt traps will cause the program to block if it's  run-

ning  in  the background, and the primary reason for quiet mode is to allow the

program to run in the background without blocking, so that  you  can  do  other

work in the foreground.

If C-Kermit is run in the background ("&" on shell commmand line), then the in-

terrupt signal (Control-C) (and System III/V quit signal) are  ignored.    This

prevents  an interrupt signal intended for a foreground job (say a compilation)

from being trapped by a background Kermit session.

9.7. C-Kermit on the DEC Pro-3xx with Pro/Venix Version 1

The DEC Professional 300 series are PDP-11/23 based personal computers.   Venix

Version  1  is a Unix v7 derivative.  It should not be confused with Venix Ver-

sion 2, which is based on ATT System V; these comments apply to Venix Version 1

only.    C-Kermit  runs in local mode on the Pro-3xx when invoked from the con-

sole; the default device is /dev/com1.dout.  When connected to a remote  system

(using  C-Kermit's  'connect'  command), Pro/Venix itself (not Kermit) provides

UNIX KERMIT                                                            Page 146

VT52 terminal emulation.  Terminal operation at high speeds  (like  9600  baud)

requires  xon/xoff  flow  control, which unfortunately interferes with applica-

tions such as the EMACS that use Control-Q and Control-S as commands.

When logging in to a Pro-3xx (or any workstation) through the "back  port",  it

may  be  necessary  to  give  the  command  "set line /dev/tty" in order to get

C-Kermit to function correctly in remote mode (on a system in which it normally

expects to be operating in local mode).

9.8. C-Kermit under VAX/VMS

Version  4C  of C-Kermit can be built using VAX-11 C to run under VMS.  Most of

the descriptions in this manual hold true, but it should be noted  that  as  of

this  writing the VMS support is not thoroughly tested, and no explicit support

exists for the various types of VMS files and their attributes.

The C-Kermit init file for VMS is called KERMIT.INI.

9.9. C-Kermit on the Macintosh

The "protocol kernel" of C-Kermit is also used by Columbia's Macintosh  Kermit.

The  user  and  system  interface  is  entirely  different, and is covered in a

separate document.

9.10. C-Kermit Restrictions and Known Bugs

1. Editing characters:  The program's  interactive  command  interrupt,

delete,  and  kill  characters are Control-C, Delete (or Backspace),

and Control-U, respectively.  There is currently no  way  to  change

them  to  suit your taste or match those used by your shell, in case

those are different.

2. High baud rates:  There's no way to specify baud rates  higher  than

9600  baud.  Most Unix systems don't supply symbols for them (unless

you use EXTA, EXTB), and even when they do, the program has  no  way

of  knowing whether a specific port's serial i/o controller supports

those rates.

3. Modem controls:  If a connection is made over a  communication  line

(rather  than  on  the controlling terminal line), and that line has

modem controls, (e.g. data  terminal  ready  and  carrier  detection

implementation),  returning  to  the shell level will disconnect the

conversation.  In that case, one should use  interactive  mode  com-

mands,  and  avoid use of piped shell-level operation (also see 'set

modem-dialer' and 'dial' commands.)

4. Login Scripts:  The present login scripts implementation follows the

Unix conventions of uucp's "L.sys" file, rather than the normal Ker-

mit "INPUT/OUTPUT" style, so there's no way  to  arbitrarily  mingle

script  output  with Kermit commands (e.g. changing parity or duplex

in the middle of a script).

5. Dial-out vs dial-in  communications  lines:    C-Kermit  requires  a

UNIX KERMIT                                                            Page 147

dial-out or dedicated line for the "set line" or "-l" options.  Most

systems have some lines dedicated  to  dial-in,  which  they  enable

"loggers"  on,  and some lines available for dial-out.  Where a line

must be shared between dial-in and  dial-out,  several  options  are

available (though they are, strictly speaking, outside the pervue of

C-Kermit).

A simple shell program can be used to change directionality  of  the

line  if  your  Unix  has the enable(8) and disable(8) commands.  In

that case, the shell program could "grep" a "who" to see if  anybody

is  logged  onto  the  desired  line; if not, it could "disable" the

line.  The shell program will need to be set-uID'ed to  root.    The

shell  program  can  be  called from kermit prior to a dial command,

e.g., "! mydisable.shellprog".   Prior  to  the  final  "quit"  from

C-Kermit,  another  shell  program could be executed to "enable" the

line again.  This program also needs to be set-uID'ed to root.

If your Unix lacks the enable(8) and  disable(8)  commands,  another

common  technique  works if your system supports the /etc/ttys file.

A shell program could call up an awk program to find the line in the

file  and  set  the enable byte to 0 (to directly disable the line).

Likewise, it can be reenabled by a counterpart at the end.   It  may

be  necessary  to pause for 60 seconds after modifying that file be-

fore the logger sees it and actually disables the line.

6. Using C-Kermit on Local Area Networks:   C-Kermit  can  successfully

operate  at  speeds  up to 9600 baud over LANs, provided the network

buffers are big enough to accommodate Kermit packets (which are  al-

most always less than 100 characters long).

When  computers are connected to LAN's through asynchronous terminal

interfaces, then the connection should be configured to do  XON/XOFF

flow  control between the network interface and the computer, rather

than passing these signals through transparently.    This  can  help

prevent  Kermit from overrunning the LAN's buffers if they are small

(or if the LAN is congested), and will can also prevent the LAN from

overrunning a slow Kermit's buffers.

If  the network hardware cannot accept 100 characters at a time, and

flow control cannot be done between the network  and  the  computer,

then  Kermit's  "set send/receive packet-length" command can be used

to shorten the packets.

7. Resetting terminal after abnormal termination or kill: When C-Kermit

terminates abnormally (say, for example, by a kill command issued by

the operator) the user may need to reset the  terminal  state.    If

commands  do  not  seem  to  be  accepted  at  the shell prompt, try

Control-J "stty sane" Control-J  (use  "reset"  on  Berkeley  Unix).

That  should  take  the  terminal  out of "raw mode" if it was stuck

there.

8. Remote host commands  may  time-out  on  lengthy  activity:    Using

"remote  host"  to instruct the C-Kermit server to invoke Unix func-

tions (like "make") that might take a long time  to  produce  output

can cause timeout conditions.

UNIX KERMIT                                                            Page 148

9. XOFF  deadlocks:    When  connecting back to C-Kermit after a trans-

action, or after finishing the server, it may be necessary to type a

Control-Q  to  clear  up  an  XOFF  deadlock.   There's not much the

program can do about this...

10. PC/IX Login Scripts -- Unfound Bug:  Though login scripts appear  to

work  properly  on  most  processors,  in the case of the PC/XT with

PC/IX, it appears that longer scripts need  to  be  broken  up  into

shorter  scripts (invoked sequentially from the take file).  This is

because the portion of the script handler which checks if an  opera-

tion timed out seems to leave the processor in a strange state (i.e.

hung).

9.11. How to Build C-Kermit for a Unix System

The C-Kermit files, as distributed from Columbia, all  begin  with  the  prefix

"ck".    You  should  make  a  directory  for these files and then cd to it.  A

makefile is provided to build C-Kermit for  various  Unix  systems  (there  are

separate  makefiles  for  VMS and the Macintosh).  As distributed, the makefile

has the name "ckuker.mak".  You should rename it to "makefile"  and  then  type

"make xxx", where xxx is the symbol for your system, for instance "make bsd" to

make C-Kermit for 4.x BSD Unix.  The result will be a program called  "wermit".

You  should test this to make sure it works; if it does, then you can rename it

to "kermit" and install it for general use.  See the makefile for a list of the

systems supported and the corresponding "make" arguments.

9.12. Adapting C-Kermit to Other Systems

C-Kermit is designed for portability.  The level of portability is indicated in

parentheses after the module name: "C" means any system that has a  C  compiler

that conforms to the description in "The C Programming Language" by Kernighan &

Ritchie (Prentice-Hall, 1978).  "Cf" is like "C", but also requires  "standard"

features like printf and fprintf, argument passing via argv/argc, and so on, as

described in Kernighan & Ritchie.  "Unix" means the module should be useful un-

der  any  Unix  implementation;  it requires features such as fork() and pipes.

Anything else means that the module is  particular  to  the  indicated  system.

C-Kermit file names are of the form:

ck<system><what>.<type>

where the part before the dot is no more than 6 characters long, the part after

the dot no more than 3 characters long, and:

<type> is the file type:

c:    C language source

h:    Header file for C language source

w:    Wart preprocessor source, converted by Wart (or Lex) to a C program

nr:   Nroff/Troff text formatter source

mss:  Scribe text formatter source

doc:  Documentation

hlp:  Help text

bld:  Instructions for building the program

bwr:  A "beware" file - list of known bugs

UNIX KERMIT                                                            Page 149

upd:  Program update log

mak:  Makefile

<system> is a single character to tell what system the file applies to:

a:    Descriptive material, documentation

c:    All systems with C compilers

d:    MS-DOS

m:    Macintosh

u:    Unix

v:    VAX/VMS

w:    Wart

<what> is mnemonic (up to 3 characters) for what's in the file:

aaa:  A "read-me" file, like this one

cmd:  Command parsing

con:  Connect command

deb:  Debug/Transaction Log formats, Typedefs

dia:  Modem/Dialer control

fio:  System-depdendent File I/O

fns:  Protocol support functions

fn2:  More protocol support functions

ker:  General C-Kermit definitions, information, documentation

mai:  Main program

pro:  Protocol

scr:  Script command

tio:  System-dependent terminal i/o & control and interrupt handing

usr:  User interface

us2:  More user interface

us3:  Still more user interface

Examples:

ckufio.c        File i/o for Unix

ckmtio.c        Terminal i/o for Macintosh

ckuker.mss      Scribe source for for Kermit User Guide chapter

ckuker.nr       Nroff source file for Unix C-Kermit man page

The following material discusses each of the C-Kermit modules briefly.

ckcmai.c, ckcker.h, ckcdeb.h (Cf):

This  is  the  main program.  It contains declarations for global variables

and a small amount of code to initialize some variables and invoke the com-

mand  parser.    In  its distributed form, it assumes that command line ar-

guments are passed to it via argc and argv.  Since this portion of code  is

only several lines long, it should be easy to replace for systems that have

different styles of user interaction.  The header files define symbols  and

macros  used  by  the  various  modules  of C-Kermit.  ckcdeb.h is the only

header file that is included by all the C-Kermit modules,  so  it  contains

not  only  the  debug  format  definitions, but also any compiler-dependent

typedefs.

ckwart.c (Cf), ckcpro.w (C):

The  ckcpro module embodies the Kermit protocol state table and the code to

accomplish state switching.  It is written in "wart", a language which  may

UNIX KERMIT                                                            Page 150

be regarded as a subset of the Unix "lex" lexical analyzer generator.  Wart

implements enough of lex to allow the ckprot module to function.   Lex  it-

self  was not used because it is proprietary.  The protocol module ckcpro.w

is read by wart, and a system-independent C program is produced.  The  syn-

tax  of  a  Wart  program  is  illustrated by ckcpro.w, and is described in

ckwart.doc.

ckcfns.c (C):

The  module  contains  all  the Kermit protocol support functions -- packet

formation, encoding, decoding, block check calculation, filename  and  data

conversion, protocol parameter negotiation, and high-level interaction with

the communication line and file system.  To accommodate small systems, this

module has been split into two -- ckcfns.c and ckcfn2.c.

ckutio.c:

This module contains the system-dependent primitives for communication line

i/o,  timers, and interrupts for the various versions of Unix.  Certain im-

portant variables are defined  in  this  module,  which  determine  whether

C-Kermit  is  by  default  remote  or local, what the default communication

device is, and so forth.  The tio module maintains its own private database

of  file descriptors and modes for the console terminal and the file trans-

fer communication line so that other modules (like ckcfns or  the  terminal

connect module) need not be concerned with them.  The variations among Unix

implementations with respect to terminal  control  and  timers  are  accom-

modated via conditional compilation.

ckufio.c:

This module contains system-dependent primitives  for  file  i/o,  wildcard

(meta  character) expansion, file existence and access checking, and system

command execution for the various versions of Unix.  It maintains an inter-

nal  database  of i/o "channels" (file pointers in this case) for the files

C-Kermit cares about -- the input file (the file which is being sent),  the

output file (the file being received), the various logs, the screen, and so

forth.  This module varies little among Unix implementations except for the

wildcard  expansion  code;  the  directory structure of 4.2bsd Unix is dif-

ferent from that of other Unix systems.  Again, variation among  Unix  sys-

tems is selected using conditional compilation.

ckuusr.h, ckuusr.c, ckuus2.c, ckuus3.c (Unix):

This is the "user interface" for C-Kermit.  It includes the command parser,

the screen output functions, and console input functions.  The command par-

ser comes in two pieces -- the traditional Unix command line decoder (which

is  quite  small and compact), and the interactive keyword parser (which is

rather large).  This module is fully replacable; its interface to the other

modules  is  very  simple,  and is explained at the beginning of the source

file.  The ckuusr  module  also  includes  code  to  execute  any  commands

directly  which don't require the Kermit protocol -- local file management,

etc.  The module is rated "Unix" because it makes  occasional  use  of  the

system() function.

Note  that  while ckuusr is logically one module, it has been split up into

three C source files, plus a header file for the symbols they share in com-

mon.   This is to accommodate small systems that cannot handle big modules.

ckuusr.c has the command line and  top-level  interactive  command  parser;

ckuus2.c  has  the  help command and strings; ckuus3 has the set and remote

commands along with the logging, screen, and "interrupt" functions.

UNIX KERMIT                                                            Page 151

ckucmd.c, ckucmd.h (Cf):

This  is an interactive command parsing package developed for C-Kermit.  It

is written portably enough to be usable on any system that has a C compiler

that  supports  functions  like  printf.    The file name parsing functions

depend upon primitives defined in the fio module; if these primitives  can-

not  be  supplied for a certain system, then the filename parsing functions

can be deleted, and the package will still be useful for parsing  keywords,

numbers, arbitrary text strings, and so forth.  The style of interaction is

the same as that found on the DECSYSTEM-20.

ckucon.c (Unix):

This is the connect module.  As supplied, it should operate in any Unix en-

vironment, or any C-based environment that provides  the  fork()  function.

The  module  requires  access  to global variables that specify line speed,

parity, duplex, flow control, etc,  and  invokes  functions  from  the  tio

module  to  accomplish the desired settings and input/output, and functions

from the fio module to perform session logging.  No terminal  emulation  is

performed,  but  since  standard  i/o  is used for the console, this may be

piped through a terminal emulation filter.  The ckucon function may be  en-

tirely replaced, so long as the global settings are honored by its replace-

ment.  PC implementations of C-Kermit may require the ck?con module  to  do

screen  control,  escape sequence interpretation, etc, and may also wish to

write special code to get the best possible performance.

ckudia.c (Unix):

This  is the dialer module.  As supplied, it handles Hayes, Ventel, Penril,

Racal-Vadic, and several other modems.

ckuscr.c (Unix):

This  is  the  login  script  module.    As supplied, it handles uucp-style

scripts.

Moving C-Kermit to a new system entails:

1. Creating a new ck?tio module in C, assembler, or  whatever  language

is  most  appropriate  for system programming on the new system.  If

the system is Unix-like,  then  support  may  be  added  within  the

ckutio.c module itself using conditional compilation.

2. Creating a new ck?fio module, as above.

3. If  the system is not Unix-like, then a new ckuusr module may be re-

quired, as well as a different invocation of it from ckcmai.

4. If the distributed connect module doesn't work or  performs  poorly,

then  it may be replaced.  For instance, interrupt-driven i/o may be

required, especially if the system doesn't have forks.

Those who favor  a  different  style  of  user/program  interaction  from  that

provided  in ckuusr.c may replace the entire module, for instance with one that

provides a mouse/window/icon environment, a menu/function-key environment, etc.

A few guidelines should be followed to maintain portability:

- Keep variable and function names to 6 characters or less.  Don't  use

identifiers  that  are  distinguished  from  one  another only by al-

UNIX KERMIT                                                            Page 152

phabetic case.

- Keep modules small.  For instance, on a PDP-11  it  is  necessary  to

keep  the  code segment of each module below 8K in order to allow the

segment mapping to occur which is necessary to  run  programs  larger

than 64K on a non-I-and-D-space machine.

- Keep  strings short; many compilers have restrictive maximum lengths;

128 is the smallest maximum string constant length we've  encountered

so far.

- Keep  (f,s)printf  formats  short.  If these exceed some compiler de-

pendent maximum (say, 128) memory will be overwritten and the program

will probably core dump.

- Do not introduce system dependencies into ckcpro.w or ckcfn*.c.

- If  a  variable  is a character, declare as CHAR, not int, to prevent

the various sign extension and byte swapping foulups that occur  when

characters are placed in integer variables.

- Remember  that  different  systems may use different length words for

different things.  Don't assume an integer can be used as a  pointer,

etc.

- Don't  declare  static  functions; these can wreak havoc with systems

that do segment mapping.

- In conditional compilations expressions, use #ifdef and  #ifndef  and

not  #if,  which is not supported by some compilers.  Also, don't use

any operators in these expressions; many compilers will fail  to  un-

derstand expressions like #ifdef FOO | BAR.

- Don't define multiline macros.

In  general,  remember that this program will have to be compilable by old com-

pilers and runnable on small systems.

MACINTOSH KERMIT                                                       Page 153

10. MACINTOSH KERMIT

Program:        Bill  Catchings,  Bill  Schilit,  Frank   da   Cruz   (Columbia

University),

Davide Cervone, University of Rochester

Language:       C (SUMACC)

Documentation:  Frank da Cruz, Bill Schilit

Version:        0.8(34)

Date:           March, 1986

Macintosh  Kermit, or "MacKermit", is an implemtation of the Kermit file trans-

fer protocol for the Apple Macintosh (and Macintosh-XL) computer, developed  at

Columbia  University,  based  on C-Kermit (which also forms the nucleus of Unix

Kermit).

MacKermit Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes (server mode only)

Login scripts:                     No

Transfer text files:               Yes

Transfer binary files:             Yes

Wildcard send:                     No

File transfer interruption:        Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes (VT100,VT102)

Communication settings:            Yes

Transmit BREAK:                    Yes

Support for dialout modems:        No

IBM mainframe communication:       Yes

Transaction logging:               No

Session logging:                   No

Debug logging:                     No

Packet logging:                    No

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         No

Local file management:             Yes

Command/Init files:                Yes

File attributes packets:           No

Command macros:                    No

Raw file transmit:                 No

The main differences between MacKermit and other Kermit programs are:

- In MacKermit  you  are  always  connected  via  a  terminal  emulator

(VT102).

- MacKermit  commands are issued by means of pull-down menus that over-

lay your terminal session.

MACINTOSH KERMIT                                                       Page 154

The major menus are File, Settings, Remote, and Transfer.  The  File  menu  in-

vokes  Kermit's  file  transfer functions, allows settings to be saved and res-

tored, and like most Macintosh applications, includes a  "quit"  selection  for

leaving the program.

The  Settings menu provides dialog boxes for file, communications, and protocol

settings.  The Remote menu has the commands that can be sent to Kermit servers,

as  well  as  an  option  to  turn  Macintosh Kermit itself into a server.  The

Transfer menu gives you a standard Macintosh file box, allowing you to transfer

directly to the selected application.

10.1. The Macintosh File System

The  Macintosh  file system consists of one or more disks, each disk containing

files.  All files on a disk must have a unique name.  Files  may  be  collected

together  into "folders", but folders are not analogous to directories on other

file systems, and no two folders on the same disk may contain files of the same

name.    Macintosh file names may contain practically any printable characters,

including space and punctuation -- but colon (":")  should be  avoided  because

it is used in device names.

10.2. File Transfer

Glossary:

- Mode - Text or Binary.  Binary means the data is sent or stored with-

out modification. Text means that  every  carriage  return  character

(CR)  in a Macintosh file is translated to a carriage-return-linefeed

(CRLF) sequence when sending, and every CRLF in an incoming  file  is

turned  into  a  CR  when  stored  on  the  Mac disk.  A text file is

produced when you save a file from MacWrite using the "text only" op-

tion; text files are not associated with any particular Macintosh ap-

plication and can be sent in a useful fashion to other kinds of  com-

puters.

- Fork  - Data or Resource.  Macintosh files may have two "forks".  The

data fork contains data for an application; the  resource  fork  con-

tains  icons,  strings,  dialog boxes, and so forth.  For instance, a

MacWrite document contains text and  formatting  information  in  the

data fork, and fonts in the resource fork.  For applications, the ex-

ecutable code is stored in the resource fork.

Macintosh Kermit supports the standard Kermit commands for  transferring  files

--  Send,  Receive,  and  Get.   Invocation of any of these commands produces a

MacKermit file dialog box in which you specify the file name, the mode, and the

fork.  Defaults are determined from the selected file or taken from the current

file settings, described below.

When you select the Send command, you get a MacKermit file open box, which  in-

cludes  the standard Macintosh dialog items -- a file list, Disk and Eject but-

tons, etc.  You can only send one file at a time, by clicking on  its  name  in

the  file  list.  Clicking the Disk button will switch the file list to another

physical disk.  If desired, you can type an alternate name to send the file un-

der.  When you select a file, MacKermit examines its type; if the type is APPL,

MACINTOSH KERMIT                                                       Page 155

then MacKermit expects to send the resource fork in binary mode, otherwise  the

data  fork  in  text  mode.  The Mode and Fork radio buttons will display these

choices; you may change them before clicking the Send button.

You can receive or get multiple files, providing the opposite Kermit is capable

of sending multiple files in a single transaction (most are).  As files arrive,

they will be decoded according to the current mode (text or binary), and stored

in  the  default fork (data or resource) under either the name they arrive with

(overwriting existing files of the same names) or under new unique names  (when

name  conflicts  occur),  according to the current default for name collisions.

You may also elect to perform an "attended" receive, in which you have  an  op-

portunity  to  override all file defaults on a per-file basis.  But this option

must be used with caution -- if you take too long (more than about a minute) to

execute  an  incoming file's dialog box, the opposite Kermit could time out and

terminate the transaction.

The folder for new files is the same as the location of the settings  file,  or

if  no settings file was used then the new files appear on the desktop.  If you

are transferring a lot of files and  want  to  keep  them  together,  create  a

folder,  drag the settings file into it, and double click on the settings file;

all created files will appear in that folder.

File transfers can be cancelled by clicking on the Cancel File or Cancel  Group

buttons.   These will always work when sending.  When receiving, they will work

if the opposite Kermit honors this (optional) feature of the protocol.

In any case, an "emergency exit" from any protocol operation can  be  taken  at

any  time  by  typing  "Command-."  --  that  is,  hold  down the Command (Fan,

Cloverleaf) key and type period.

10.3. Remote Commands

The Remote menu allows you to send commands to a Kermit server.   The  response

from  these  commands  (if  any)  is  displayed  in  a  special  pop-up window.

Responses to multiple Remote commands are separated by  a  dashed  line.    The

response  window  can  be scrolled, sized, and positioned, and can be hidden by

clicking the menu item "Hide Response" or the window's go-away  box;  all  text

remains  intact  and will be appended to the next time you do a Remote command;

it can also be brought to the foreground by clicking  the  Show  Response  menu

item.   Note that typein to the terminal emulator will not take effect when the

response window -- or any other window -- is up front.

If the response window gets too full (i.e. fills up the free  memory  available

to the MacKermit application), the program will probably bomb.

If the remote Kermit server is in binary mode, its responses to Remote commands

may look strange.  For instance, a Unix Kermit server in binary mode will  send

lines of text separated by only linefeeds, rather than CRLFs.

A Remote command can be cancelled by taking the Emergency Exit (Command-.).

MACINTOSH KERMIT                                                       Page 156

10.4. Settings

You  can  change  File, Communications, and Protocol settings by using the Set-

tings pull-down menu.  You can save and restore these settings by invoking  the

appropriate  selection  in the File menu.  If the bundle bit has been correctly

set on your version of MacKermit you can double-click on the resulting document

to start MacKermit with those settings.

The File settings establish the defaults for file transfer:

- Mode:  text  or binary.  Used for received files only.  When sending,

MacKermit tries to figure out an appropriate mode for the file  being

sent (but then lets you override it the Send File dialog).

- Fork:  which  fork -- data or resource -- to send, or to store an in-

coming file into.

- Naming: Whether incoming files should supersede existing files of the

same  name,  or a new unique name should be assigned to them.  If the

latter, the new name is formed by adding a dot and a  number  to  the

end.  For instance, if a file called FOO exists and a file called FOO

arrives, MacKermit will store the arriving file as  FOO.1;  if  FOO.1

exists, then FOO.2, etc.

- Attended versus Unattended operation for incoming files.

The  Communications  settings  allow you to set the baud rate (anywhere between

300 baud and 57.6K baud), parity (odd, even, mark, space, or none), and  duplex

(full - remote echo, half - local echo).

The  Protocol settings allow you to set packet parameters for both incoming and

outbound packets.  These include the block check type (1 or 2 character  check-

sum,  3-character  16-bit  CRC-CCITT), line turnaround handshake character (for

file transfer with half duplex systems), packet start and end characters,  pad-

ding,  packet  length,  timeout  interval,  and  so forth (Refer to Kermit User

Guide).  Characters are specified by entering their  ASCII  value  in  decimal,

e.g. 1 for Control-A, 13 for Control-M (Carriage Return), etc.

10.5. Terminal Emulation

MacKermit  provides  a  subset  of  the features of the DEC VT102 terminal; the

VT102 is a VT100 with line and character insert/delete functions  added.    The

functions  provided  are sufficient to allow MacKermit to act as a terminal for

EMACS as it exists on the DEC-20, VAX (CCA EMACS on VMS or UNIX), and for  most

host-resident  display-oriented applications that expect to do cursor position-

ing and editing on the VT100 screen.  MacKermit does not currently support  the

following VT100/102 functions:

- double height or double width lines

- smooth scrolling

- 132 columns

- Interpretation of multiple parameters in a single escape sequence

- etc

(this is not an exhaustive list)

MACINTOSH KERMIT                                                       Page 157

The keyboard is set up by default as follows: The COMMAND (Fan, Cloverleaf) key

is used as the Control key.  The CAPS LOCK key forces all alphabetic characters

to  upper  case, and causes keys on the numeric keypad to send VT100 keypad es-

cape sequences.  The OPTION key is "Control-Meta" (explained below).  The  ter-

minal  emulator  sends ESC (escape) when the "`" key is pressed unshifted.  The

character "`" can be sent by typing Control (Command) and the same  key.    The

Backspace  key sends a Delete (Rubout) and Control-Backspace sends a Backspace.

The main keypad Enter key sends a "short" (250ms) BREAK signal.  The Mac+  does

not  have  a main keypad Enter key, so the BREAK function must be reassigned to

another key.  Use CKMKEY (see below) to do this.    The  short  break  is  F126

(function number 126) and long break is F127.

MacKermit  (V0.8  and  later)  comes with a separate key configuration program,

CKMKEY, which lets you change the behavior of the keys, define  function  keys,

and so forth.  CKMKEY is described in detail below.

MacKermit  (V0.8(43A)  and later) includes a mouse-controlled cursor postioning

feature for use during terminal emulation.  When the mouse  button  is  pressed

while  the  Option  and  Command keys are held down, the program acts as if you

typed the keypad arrow keys to move the terminal cursor to where the mouse cur-

sor  is.    You must have already defined the keypad arrow keys to send the ap-

propriate character sequences for your host application.  The Catch-22 here  is

that  if  you  don't have a keypad, there's no way for you to define the keypad

keys using MacKermit's keyboard configurator.  In that case, you  can  use  the

VT100  startup file provided with MacKermit, which assigns the normal VT100 ar-

row key sequences  to  the  keypad  arrow  keys,  and  therefore  also  to  the

mouse-cursor feature.

MacKermit honors your parity communications setting by using built-in functions

of the Mac's serial i/o chip.  Unfortunately, the chip has an unpleasant  quirk

--  arriving  characters  that  do  not have the specified parity are discarded

rather than passed to the requesting application.  Thus, if you  are  connected

as  a  terminal  using  MacKermit to a device that requires, say, odd parity on

characters sent to it, but does not put odd parity on characters  it  sends  to

you, then many incoming characters will not appear on your screen.

To allow useful coexistence of desk accessories and Kermit, the terminal emula-

tion may be dragged using the drag bar.  A desk  accessory  that  overlays  the

Kermit window can be clicked upon to move it behind the Kermit window, and then

the Kermit window can be dragged to reveal the hidden desk accessory so that it

can  be  restored  to the foreground.  The same thing can be done with Kermit's

own remote response window.  Note that Kermit's terminal emulation window  does

not accept input when any other window is in the foreground.

The  following  features  are missing from the MacKermit terminal emulator, and

may be added in subsequent releases:

- capturing text from the screen (e.g. cutting to clipboard, saving off

top)

- screen rollback, sizing

- modem or dialer control

- login scripts

- transmission of raw text to host (e.g. pasting to screen)

- printer support

MacKermit  does not use XON/XOFF flow control during terminal emulation or file

MACINTOSH KERMIT                                                       Page 158

transfer.  The terminal emulator can normally keep up at 9600 baud,  but  after

several  continuous  scrolling  screens  at  this speed, some characters may be

lost.  In the present version, when running at high baud rates keep  your  ter-

minal  in  page  mode,  or use "more", or view text with a non-scrolling screen

editor.  Also, don't drag the terminal emulation window  while  characters  are

arriving;  if  you  do, the characters will be lost and the display will become

confused.

10.6. Installation

MacKermit is distributed in source form for building on  Unix  (or  VMS/Eunice)

systems  that  have  the  Stanford SUMACC Macintosh cross-development tools, in

.HQX "binhex" form, and sometimes also as a binary resource file.    Those  who

want  to  work from the source are referred to the file CKMKER.BLD for instruc-

tions.

If you have the binary resource file available (its name will  be  CKMKER.RSRC,

ckmker.rsrc,  CKMKER.RSR,  ckmker.rsr, or some variation on these, depending on

what system it's stored on and how it got there), AND if you have  "MacPut"  on

your  system  and  MacTerminal  on  your Mac, AND if you have an 8-bit-wide (no

parity) data path between your Mac and your system, use MacPut to download  the

binary  resource  file to MacTerminal's XMODEM option on your Mac.  After doing

this you must use SetFile on the Mac to set the author to  KERM,  the  type  to

APPL, and turn on the bundle bit.  For CKMKEY, the author should be KERK.

If  you  have  an  earlier release of Columbia MacKermit, you may use Kermit in

place of MacTerminal and MacPut.

If you don't have the binary resource file  available,  you  can  download  the

CKMKER.HQX file in the same manner, then run "binhex" (version 4) on it.

10.7. CKMKEY - Macintosh Kermit's Keyboard Configurator

This describes CKMKEY V0.8(0), May 1985.

The  version  number of CKMKEY indicates compatability with the like version of

CKMKER -- Macintosh Kermit, referred to simply as "Kermit" from now on.    Edit

numbers  (within  parentheses)  may  differ.  If Kermit is used with a settings

file containing a key configuration produced  by  an  incompatible  version  of

CKMKEY, then that configuration will be ignored.

10.7.1. What is CKMKEY?

CKMKEY  is  a  keyboard  configurator  program  for  use  with Macintosh Kermit

(versions 0.8 and greater). CKMKEY allows:

- Redefinitions of keys

- Definitions of multicharacter function keys

- Selection of long and short BREAK keys

CKMKEY is a separate program from Kermit.  It may be thought of  as  an  editor

MACINTOSH KERMIT                                                       Page 159

for Kermit's terminal emulator key definition resource, which is kept in a Ker-

mit settings file.  Before you can use CKMKEY, you must already have used  Ker-

mit to create a settings file to operate on.

The  reason  CKMKEY is separate from Kermit is that there is not enough room in

the memory of a 128K Macintosh to hold a program that can do both.  CKMKEY dis-

plays and changes key settings, Kermit uses them.  Once you have started Kermit

with a given set of key definitions, there is no way to examine or change them.

Some familiarity with the ASCII alphabet is assumed in  the  following  discus-

sion.

10.7.2. Modifier vs Normal Keys

The  Macintosh keyboard is composed of normal keys and modifier keys.  Modifier

keys are SHIFT, CAPS LOCK, OPTION, and COMMAND (also known as APPLE, CLOVER, or

FAN).   Only one normal key can be selected at a time, but one or more modifier

keys can be depressed along with it.

10.7.3. Key Maps

When a key on the keyboard or numeric keypad is depressed the result is a  scan

code -- a number between 0 and 127 (see Inside Mac Event Manager for details if

you're interested).  A table indexed by scan code resulting in the character to

be displayed or transmitted will be referred to as a "keymap" or "key mapping."

On  the  standard  Mac  many keymaps exist -- the modifier keys (such as SHIFT)

specify which keymap is selected.   For  example,  when  no  modifer  keys  are

depressed the keymap contains the lowercase alphabet, numbers and some punctua-

tion.  The keymap in use when the SHIFT modifer is depressed contains the capi-

tal letters and special characters.

All  in  all  it  is possible to select 16 different keymaps by depressing from

zero to four modifier keys.  Normally however, 6 or so  distinct  keymaps  will

suffice.

CKMKEY  allows you to redefine 6 keymaps: shifted and unshifted combinations of

keymaps  named  "normal",  "capslock",  and  "control".    These  keymaps   are

predefined  with  the expected values -- the control map is preloaded with con-

trol codes, the capslock preloaded with the unmodifed keymap but with all  let-

ters uppercase.

In this document modifier keys are written in capital letters and key map names

are written in lowercase.  SHIFT, CAPS LOCK, COMMAND, and OPTION  are  modifier

keys, "normal" "capslock" and "control" are key maps internal to CKMKER.  Since

one of the major functions of CKMKEY is to  change  maps  invoked  by  modifier

keys, it is important to keep this distinction in mind.

MACINTOSH KERMIT                                                       Page 160

10.7.4. What's in CKMKEY's Keymaps

A keymap is a list of 128 numbers.  Which keymap is selected depends upon which

modifier keys are depressed, and the entry within the key map is determined  by

the  scan code.  A keymap entry is an 8-bit quantity:  if the high order bit is

0, then the entry is the 7-bit ASCII character to be  transmitted  through  the

serial  port; if the high bit is 1, then the remaining 7 bits are an index into

the function-key table.

Notice that only single 7-bit values can be  directly  translated  through  the

CKMKEY  keymap.  If you want a single key to transmit multiple characters, then

you can designate that key to be a "function key", and the key map will contain

an indirect reference to the function-key table.  If you want a key to transmit

an 8-bit value, assign the "meta" operation to one of the modifier keys and use

the meta key together with the desired key (see below).

Functions  are  numbered  0-127 with the highest few being reserved for special

use.  Currently functions 126 and 127 send a short 250 millisecond BREAK signal

and a long 3.5 second BREAK respectively.  In the future more special functions

may be allocated so (since it is arbitrary  anyway)  please  use  low  numbered

functions when defining your own.

10.7.5. Menus

CKMKEY has two menus, File and Set.  First you must use the File menu to select

and open a Macintosh Kermit settings file, which in turn has been created using

the  Kermit Save Settings option from its own File menu.  Then use the Set menu

to establish or alter key definitions, then use the File menu again to save the

settings file back for Kermit.  A variety of Kermit settings files can be kept,

each with its own collection of settings and key  definitions;  Kermit  can  be

started  with  the desired settings by double clicking on one of these settings

files from the Macintosh desktop.

Menus consist of options.  If an option  is  followed  by  an  ellipsis  (three

dots...)  then  clicking  it will produce a dialog box of some kind; otherwise,

clicking it causes the indicated action to be performed immediately.  If an op-

tion  is  dimmed  then it is not available for some reason -- for instance, you

can't set any keys until you open a settings file.

10.7.6. MENU: Set

The Set menu includes dialogs for setting keys, defining functions,  and  reas-

signing modifier keys.

10.7.6.1. DIALOG: Set Modifer Keys

Background:

Skip  ahead  to  the  next section if you already know about things like SHIFT,

CAPS LOCK, CONTROL, and META.

On a typewriter the only modifier key is SHIFT.  Typing  a  character  with  no

modifier  key  depressed selects a lowercase letter or the character printed on

MACINTOSH KERMIT                                                       Page 161

the lower face of the keytop (say, the digit "4").   Typing  a  character  with

SHIFT depressed selects an uppercase letter or the character printed on the up-

per face of the keytop (say, a dollar sign).  Some keyboards also have a  SHIFT

LOCK key, which stays down once pressed and pops up the next time it's pressed;

its operation is equivalent to holding down SHIFT.  And some keyboards  have  a

CAPS lock key which operates like SHIFT LOCK, but only upon letters.

Computer  terminals  also  have  a  modifier key called CONTROL (or CTRL).  Its

function is a little less obvious: it is intended to  produce  one  of  the  33

characters  in  the  "control range" of the ASCII alphabet.  Control characters

are not graphic -- they are intended for use as format effectors (like carriage

return, formfeed, tab, backspace), for transmission control, or for device con-

trol.  The remaining 95 characters -- letters, digits, and punctuation  --  are

the  graphic  characters.   When a character is typed with the CONTROL modifier

pressed, its "control equivalent" is transmitted.  By convention,  the  control

equivalent  of  A  is  Control-A, B is Control-B, etc, and there are also seven

special control characters generally associated with punctuation characters  or

special  keys.    For  the  "alphabetic"  control  characters Control-A through

Control-Z, SHIFT or CAPS LOCK modifiers are ignored; for the others,  operation

varies from terminal to terminal.

The  SHIFT and CONTROL modifiers allow all 128 ASCII characters to be sent from

a normal typewriter-like keyboard that has about 50  keys.    However,  certain

host-resident  computer  applications  --  notably  the full screen text editor

EMACS and its descendents -- can be  used  to  greater  advantage  with  a  256

character  alphabet  (EMACS responds to single-character commands, and the more

characters a terminal can send, the more commands are directly available).  For

this purpose, some terminals also provide a META modifier key.  This key simply

causes the high-order ("8th") bit of the selected ASCII value to be  set  to  1

upon transmission.  META characters can only be transmitted when the communica-

tion path allows all 8 bits to pass transparently; when this is  not  possible,

software  like  EMACS  allows  a  sequence  of  two  7-bit  ASCII characters to

represent a single meta character.    The  advantage  of  having  a  real  META

modifier  key  is  that  it  can  be  held  down while the actual key is struck

repeatedly or even autorepeats, whereas a use of a "meta prefix" requires  much

more  typing.    To illustrate, suppose META-F is the command to go forward one

word.  If you want to execute this operation repeatedly, just  hold  down  META

and  F  and  let it autorepeat.  If you don't have a META key, then you have to

use a "meta prefix" character (usually escape), and to enter META-F  repeatedly

in this case, you'd have to type <escape>F<escape>F<escape>F...etc.

Macintosh Kermit Modifier Keys:

You can define the modifier key to keymap correspondence in CKMKEY by selecting

the "Modifer Keys..." menu item under SET, or by double clicking on a  modifier

key while in the SET KEYS dialog.

The SET MODIFIERS dialog lets you define what map OPTION, CAPS LOCK and COMMAND

refer to.  Notice that SHIFT is missing -- SHIFT always references the  shifted

equivalents to the normal, control and caps lock maps.

The dialog is layed out in columns with the three modifier keys as column head-

ings and the three map names below.  Also under each column is a  "pseudo"  key

map for "meta."

Meta  is  not  a map, but an operation: it augments the value being transmitted

MACINTOSH KERMIT                                                       Page 162

after it has been read from its map.  Meta can either be set to send  a  prefix

string  before  the  character  or  to  turn the high order (8th) bit on in the

transmitted character.  The default prefix for meta is set to be 033  (escape).

If a meta modifier key is depressed and the key results in a function reference

then no modification occurs; functions are not "metized".   However,  functions

can be defined to include 8-bit values.

Notice  that  meta  can be set in conjunction with a key map.  Since meta is an

operation as described above there is no ambiguity.  Consider for example  set-

ting  OPTION  to  reference  the  "control"  map  and selecting "meta" for this

modifier key as well.  The result is a control-meta key.

CAUTION: If you have used  Kermit's  communications  settings  menu  to

select  any parity other than "none", then any high order bits that may

be set by CKMKER's key mapping will be superseded when  Kermit  applies

the selected parity to outbound characters.

The  SET  MODIFIER  KEYS  dialog  also  lets  you select your meta preferences:

whether you want to use the 8th bit toggled on, or a prefix string.  The prefix

string  is  entered in the same manner as a function definition (backslash fol-

lowed by 3 octal digits for non-printable characters, see below).

Note that it is possible to cause ambiguities when selecting and using modifier

keys.   For example say you set OPTION to refer to the control map, and you set

CAPS LOCK to refer to the caps map; at this point if you hold both  OPTION  and

CAPS  LOCK  down  it is unclear which map you want your character to come from.

To try to prevent this type of ambiguity the SET KEY dialog will beep when  you

are holding down or mousing an ambigous set of modifier keys.

The  Kermit  code  itself references maps in this precendence: if a control map

modifier is depressed then use control map, else  if  a  capslock  modifier  is

depressed use capslock, otherwise use the normal map.

A sample modifier key configuration is shown in Figure 10-1.

-------------------------------------------------------------------------

OPTION          COMMAND         CAPS LOCK

o  Normal       *  Normal       o  Normal

*  Control      o  Control      *  Control

o  Caps         o  Caps         o  Caps

[X] Meta        [X] Meta        [ ] Meta

Figure 10-1:   Macintosh Kermit Modifier Key Dialog

-------------------------------------------------------------------------

Here  the  CAPS  LOCK key is used to reference "control", the COMMAND key to do

the "meta" operation, and OPTION is "control-meta".  Holding down  COMMAND  and

CAPS LOCK together will also result in control-meta.

MACINTOSH KERMIT                                                       Page 163

10.7.6.2. DIALOG: Set Function Definitions

Background:

Skip to next section if you know what function keys are.

Many  popular  terminals  have  "function  keys"  in addition to the alpabetic,

numeric, punctuation, and modifier keys described above.    Function  keys  are

usually  labeled F0, F1, F2, ..., or PF1, PF2, ...  On some terminals, like the

DEC VT100, the function keys send predefined sequences of characters -- for in-

stance  PF1  sends  three  characters: ESCAPE (ASCII 033), followed by "O", and

"P".  On others, the function keys may have arbitrary strings assigned to them.

For  instance,  if  you find yourself typing "Aaaarrrgggghhh!!! Sigh..." a lot,

you can assign this string to function key F1, and then  pressing  the  F1  key

will cause this entire character string to be transmitted.

Macintosh Kermit Function Keys:

The  Macintosh  has no physical function keys -- no keys are marked F0, F1, F2,

etc.  However, any key (modified by any combination of modifier  keys)  may  be

designated as a "soft" function key.

Selecting  "Function  Definitions..."  from  the SET menu brings you to the SET

FUNCTIONS dialog (it would be nice if you could double click on a function  key

in the SET KEYS dialog but that is not yet available).

Use  SET FUNCTIONS to declare a function definition string.  Scroll through the

function definition list and select a function to define,  preferably  starting

with  F0, though this is not required (high numbered functions are reserved for

special uses).  Type in the function definition; non printable characters  must

be entered with a backslash ("\") followed by exactly (yes exactly) three octal

characters representing the ASCII  value,  for  instance  "\015"  for  carriage

return.   A backslash itself is entered as "\134".  The function definition has

to fit in the box.

Having defined a function, you must use SET KEYS to actually associate it  with

a  key.    Note  that it is possible to associate a function with more than one

key.

10.7.6.3. DIALOG: Set Keys

Selecting the "Keys..." menu item under SET initiates the SET KEYS  dialog  for

redefining individual keys.

SET  KEYS  displays  a  picture  of the keyboard.  You can either hold down the

modifier and key you wish to define or click on the displayed picture with  the

mouse  (double  clicking on one of the modifier keys brings up the SET MODIFIER

KEYS dialog).  Once a key is selected, it and any  modifiers  are  highlighted,

the  name  of  the  key and its value are displayed in the lower portion of the

dialog.  You may enter the new value in the little box  by  selecting  the  box

with  the  mouse  and then typing a DECIMAL (yes decimal) number from 0 to 127.

Then you should click on either SET KEY or else SET FUNCTION KEY.  Clicking  on

SET KEY means that the key should transmit the ASCII character corresponding to

the given value (subject to modification by the meta key); clicking  SET  FUNC-

TION  KEY means the number you entered in the box is a function number and that

MACINTOSH KERMIT                                                       Page 164

the key should transmit the character string associated with that function.

SET KEYS does not display a picture of the numeric keypad, but may be used with

the keypad anyway -- just select the desired key by pressing it and then define

it as above.

10.7.7. MENU: File

The File menu must be used to Open a Kermit settings file  before  CKMKEY  will

allow  you  do  perform  any  other  operations.  You may also Quit from CKMKEY

through the File menu, and you can save your work.  The Save option allows  you

to save the settings file back under its own name, replacing the previous copy.

If you need to make copies of settings files, you can use Kermit itself to save

them under different names, or else you can use the Finder.

There is also a Decompile option, that is of use only to programmers working on

Macintosh Kermit -- it decompiles the key definition resource into a form  that

can be included in a C program.

10.7.8. CKMKEY Known Limitations, Restrictions, Bugs

- There is no picture of the numeric keypad in Set Keys.

- In Set Keys, when you strike a key on the numeric keypad, its name is

not displayed.  You can still make assignments to the key.

- There is no way to define a key from the numeric  keypad  unless  you

actually have a numeric keypad.

- You can't save from CKMKEY under a different name.  Use the Finder or

Kermit to do that.

- You must use decimal numbers in the SET  KEY  dialog,  and  backslash

followed by 3 octal digits in function definitions, which can be con-

fusing.

- You may have problems on a 128K mac if you  define  many  long  func-

tions.

- CKMKEY doesn't deal with write protected diskettes very well.

10.7.9. Unlocking CAPS LOCK

(Adapted    from    directions   posted   by   David   Chase   <rbbb@rice>   on

INFO-MAC@SUMEX-AIM, Friday 14 December 1984.  Follow these instructions at your

own  risk.  Not the authors, nor David Chase, nor Columbia University, nor Rice

University provide any warranty,  nor  acknowledge  any  liability  or  respon-

sibility  for  damage, injury, inconvenience, or loss of Apple or other service

warranty sufferred as a result of the publication of these directions.)

A major impediment to using the Macintosh as a terminal is that the  CAPS  LOCK

key  is  where  you  would  normally  expect  to  find  the CONTROL key.  A key

redefinition package, such as CKMKEY, can assign the CONTROL  function  to  the

MACINTOSH KERMIT                                                       Page 165

COMMAND  or OPTION keys but these keys are not easy to reach.  CONTROL can also

be assigned to the CAPS LOCK key using software, but the CAPS LOCK key includes

a  mechanical  locking device.  The following directions tell how to remove the

locking device so that the CAPS LOCK key will go up and  down  like  the  other

keys.  PROCEED AT YOUR OWN RISK.

Tools you'll need:

- Phillips screwdriver for screws on bottom of the keyboard.

- Solder sucker/wick.

- Soldering iron.

- Small prying tools (jewelers screwdrivers, small knife blade, etc).

- Tweezers/small needlenose pliers.

- Some paper clips or straight pins.

Now follow these steps:

1. Remove the five screws.  The keyboard should fall into three pieces.

2. GENTLY pry off the Caps Lock keycap.  This takes a little patience.

3. Remove the restoring spring so it doesn't get in the way.

4. Locate  the  two connections to the Caps Lock key on the back of the

PC board, and remove all solder from them using wick or sucker.   Be

careful  not  to overheat the solder pads, since they can be damaged

(come loose from the PC board).

5. Pry back the plastic locking clips holding the key  in,  and  remove

it.    (All  the  keys are clipped into a metal frame.  Removing the

metal frame is not possible, since all the keys are soldered to  the

PC  board,  and clipped to the frame.  The clips are located "north"

and "south" of the key, where the number  row  is  "north"  and  the

space  bar "south".)  There are four clips holding the bottom of the

key on; pry these back, and, WHILE HOLDING THE KEY BOTTOM UP, remove

the  bottom  of  the key.  You may have to use some makeshift tools,

like a couple of unbent paper clips, to hold the four clips open.

6. Two  pieces  should  be  ready  to  fall  out;  a  small  piece   of

PC-board-like material (about 7/16 by 3/32 inch, with two notches on

one edge and a tiny hole in the center), and a tiny piece of wire (a

small,  beefy  staple  with short legs).  Let them fall out. (It may

help to toggle the key).  These two pieces are the  locking  device,

they should be removed and left out of the reassembly.

7. Replace the restoring spring, snap the key back into place, resolder

the two leads, screw the keyboard back together, and replace the key

cap.  You may wish to experiment with the spring to reduce the key's

springiness (this can be done with the  keyboard  assembled,  though

removing the cap is more difficult).

For  those  who  want to map the CAPS LOCK key to CTRL, but don't want to alter

the keyboard as described in the manual, but still want to inhibit the  locking

function, the following suggestion is offered:

Pry  off  the  key using a small screwdriver.  There is a spring whose end goes

MACINTOSH KERMIT                                                       Page 166

through the plastic support.  Stick a very small wad of paper or soft putty be-

tween  the  tip and the bottom of the keyboard.  This will prevent the key from

depressing all the way and locking, but still allow contact of the key.   Even-

tually,  the  paper will work loose and you will need to find it and repeat the

procedure.

MS-DOS KERMIT                                                          Page 167

11. MS-DOS KERMIT

Program:        Daphne Tzoar and Jeff Damens (Columbia University),

Joe R. Doupnik (Utah State University),

James Harvey (Indiana/Purdue University),

contributions by many others.

Language:       Microsoft Macro Assembler (MASM)

Documentation:  Frank da Cruz and Christine Gianone (Columbia University)

Version:        2.29

Date:           May 26, 1986

Kermit-MS Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes

IBM mainframe communication:       Yes

Transaction logging:               No

Session logging:                   Yes

Raw transmit:                      No

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         Yes

Advanced commands for servers:     Yes

Local file management:             Yes

Command/init files:                Yes

Command macros:                    Yes

Attribute packets:                 No

Extended-length packets:           No

Sliding windows:                   No

Kermit-MS is a program that implements the Kermit file  transfer  protocol  for

the  IBM PC family, IBM compatibles, and several other machines using the Intel

8086 processor series (8088, 80286, etc) and operating system family (PC-DOS or

MS-DOS, henceforth referred to collectively as MS-DOS or simply DOS).

Kermit-MS  version  2.29  runs  in  as  little  as  60K  of  memory  (about 55K

contiguous), but will occupy up to 100K, if it can be found, for  extra  screen

rollback memory.  It will also try to leave 24 Kbytes free for a second copy of

COMMAND.COM which is needed for the SPACE command and  others.    Versions  not

using screen rollback memory will not require the additional space.

On  the  IBM  PC,  Kermit-MS 2.29 performs almost complete emulation of the DEC

VT-102 terminal at speeds up to 19,200 baud (lacking only smooth scrolling, 132

MS-DOS KERMIT                                                          Page 168

column  mode, and ANSI printer control).  Much of the speed is accomplished via

direct writes to screen memory, but this is done in a "TopView" aware manner to

allow  successful operation in windowing environments like TopView, MS-Windows,

and DesqView.  Speed is also due to direct  access  of  the  serial  port  UART

(8250)  chip, with buffered, interrupt-driven receipt of characters and select-

able XON/XOFF flow control.  Full-speed 9600  baud  operation  is  possible  on

4.77Mhz  systems  without  flow  control, but flow control is required on these

systems for 19,200 baud or higher rates.

As of version 2.28, MS-DOS Kermit requires version 2.0 or later of DOS.   Older

versions  of  IBM  PC Kermit are no longer distributed, but if you have version

2.27 or earlier, it can run under version 1.0 or 1.1 of PC-DOS.

Kermit-MS 2.29 runs on a wide variety of MS-DOS systems, including  the  entire

IBM  PC  family  (the  PC,  XT, AT, PCjr, Portable PC, PC Convertible) and com-

patibles (Compaq, Z150, etc), the DEC Rainbow, NEC APC, Sanyo MBC, Victor 9000,

HP-110, HP-150, HP Portable Plus, and many others.

This  document  will  describe the things you should know about the MS-DOS file

system in order to make effective use of Kermit, and then it will describe  the

Kermit-MS  program  itself.   In system-specific areas like terminal emulation,

most discussion focuses on the IBM PC version.  It is assumed you  are  already

familiar  with the general ideas of Kermit file transfer.  If you are not, con-

sult the Kermit User Guide, or Kermit,  A  File  Transfer  Protocol,  by  Frank

da Cruz, Digital Press (1986).

11.1. The MS-DOS File System

The features of the MS-DOS file system of greatest interest to Kermit users are

the form of the file specifications, and the formats of the files themselves.

11.1.1. File Specifications

MS-DOS file specifications (in version 2.0 or later of DOS) are of the form

DEVICE:\PATHNAME\NAME.TYPE

where the DEVICE is a single character identifier  (for  instance,  A  for  the

first  floppy  disk,  C  for  the first fixed disk, D for a RAM disk emulator),

PATHNAME is up to 63 characters of identifier(s) (up to 8 characters each) sur-

rounded  by  reverse  slashes, NAME is an identifier of up to 8 characters, and

TYPE is an identifier of up to 3 characters in length.  Device and pathname may

be  omitted.    The  first  backslash  in  the  pathname  may be omitted if the

specified path is relative to the current directory.  In the  path  field,  "."

means  the  current  directory,  ".." means the parent directory.  Some DOS im-

plementations (like Wang) may use slash "/" rather than backslash as  a  direc-

tory separator.

Pathname  is  normally  omitted, but can be specified in all Kermit-MS commands

(as of version 2.29).  Device and directory pathnames, when omitted, default to

either  the  user's  current  disk  and  directory, or to the current directory

search path as specified in the DOS PATH environment variable, depending on the

context in which the file name appears.

MS-DOS KERMIT                                                          Page 169

When  this  document  says  that a file is searched for "in the current

path," it means that the PATH is searched first, and if the file is not

found,  then Kermit-MS looks on the current disk and directory.  If the

PATH environment variable is empty, Kermit looks only  at  the  current

disk and directory.

NAME.TYPE  is  normally sufficient to specify a file, and only this information

is sent along by Kermit-MS with an outgoing file.

The device, path, name, and type fields may contain uppercase letters,  digits,

and the special characters "-" (dash), "_" (underscore), and "$" (dollar sign).

When you type lowercase letters in filenames, they are converted  automatically

to  uppercase.  There are no imbedded or trailing spaces.  Other characters may

not be included; there is no mechanism for "quoting" otherwise illegal  charac-

ters  in  filenames.  The fields of the file specification are set off from one

another by the punctuation indicated above.

The name field is the primary identifier for the file.  The type,  also  called

the  extension or suffix, is an indicator which, by convention, tells what kind

of file we have.  For instance FOO.BAS is the source of a BASIC  program  named

FOO;  FOO.OBJ  might  be  the  relocatable  object module produced by compiling

FOO.BAS; FOO.EXE could be an executable program produced  by  loading  FOO.OBJ,

and so forth.  .EXE and .COM are the normal suffixes for executable programs.

The  MS-DOS allows a group of files to be specified in a single file specifica-

tion by including the special "wildcard"  characters,  "*"  and  "?".    A  "*"

matches  any  string  of characters from the current position to the end of the

field, including no characters at all; a  "?"  matches  any  single  character.

Here are some examples:

*.BAS   All  files of type BAS (all BASIC source files) in the current direc-

tory.

FOO.*   Files of all types with name FOO.

F*.*    All files whose names start with F.

F?X*.*  All files whose names start with F and contain X in the  third  posi-

tion, followed by zero or more characters.

*.?     All files whose types are exactly one character long.

Wildcard  notation  is used on many computer systems in similar ways, and it is

the mechanism most commonly used to instruct Kermit to send a group of files.

Users of Kermit-MS should bear in mind that other (non-MS-DOS) systems may  use

different wildcard characters.  For instance the DEC-20 uses "%" instead of "?"

as the single character wildcard; when using Kermit-MS to  request  a  wildcard

file  group from a Kermit-20 server, the DOS "?" must be replaced by the DEC-20

"%".

MS-DOS KERMIT                                                          Page 170

11.1.2. File Formats

MS-DOS systems store files as bulk collections of 8-bit  bytes,  with  no  par-

ticular  differences  between text, program code, and binary files.  ASCII text

files consist of lines separated by carriage-return-linefeed sequences (CRLFs),

which conforms exactly to the way Kermit represents text files during transmis-

sion.  Since a non-MS-DOS receiving system might need to make  distinctions  as

to file type, you may need to use various SET functions on the remote system to

inform it that the incoming file is of some particular (non-default) type, such

as  binary.    In  transmitting files between Kermit-MS programs, regardless of

file contents, the receiving MS-DOS system is  equally  capable  of  processing

text, code, and data, and in fact requires no knowledge of how the bytes in the

file are to be used.  Unlike most other Kermit programs, MS-DOS Kermit does not

require  a command like SET FILE TYPE BINARY to switch from text to binary file

transfer.

MS-DOS (unlike CP/M) is capable of pinpointing the end of file  with  precision

by  keeping  a  byte  count in the directory, so one would expect no particular

confusion in this regard.  However, certain MS-DOS programs continue to use the

CP/M  convention  of  terminating  a  text file with a Control-Z character, and

won't operate correctly unless this terminating byte is  present.    Therefore,

you  should be aware of a special SET EOF option for both incoming and outbound

files, described later.

Non-MS-DOS systems may  well  be  confused  by  nonstandard  ASCII  files  from

Kermit-MS:

- Files  containing  any  of the 8-bit "extended ASCII" characters will

probably need conversion (or translation) to 7-bit ASCII.

- Files produced by word processing programs like  Easywriter  or  Word

Star  may may contain special binary formatting codes, and could need

conversion to conventional 7-bit ASCII format prior to  transmission,

using commonly available "exporter" programs.

- Files  created  by  word processors that store formatting data at the

end of the file, after the Control-Z and before physical end, may re-

quire  special  processing  via SET EOF to strip the formatting data,

lest they confuse non-MS-DOS recipients.

- Spreadsheet or database files usually need special formatting  to  be

meaningful  to  non-MS-DOS recipients (though they can be transmitted

between MS-DOS systems with Kermit-MS).

11.2. Program Invocation

Kermit-MS can be run interactively, from a batch file, or as an "external"  DOS

command.   Commands consist of one or more fields, separated by "whitespace" --

one or more spaces or tabs.

Upon initial startup, the program executes  any  commands  found  in  the  file

MSKERMIT.INI in the current path.  This initialization file may contain command

macro definitions, communications settings for one or more ports, or any  other

Kermit-MS commands.  Here is a sample MSKERMIT.INI file:

MS-DOS KERMIT                                                          Page 171

comment -- MSKERMIT.INI, MS-DOS Kermit initialization file

comment -- Don't overwrite my files!

set warning on

comment -- Define macros for the systems I use...

define unix set local-echo off, set flow xon, set timer off

def ibm set par odd, set loc on, set hands xon, set flo off, set tim on

def modem set port 2, set baud 1200

comment -- Define a macro for quickly adapting to noisy connections...

def noisy set block-check 3, set send packet-length 40, set retry 20

comment -- I always start out by connecting to my UNIX system...

set port 1

set baud 4800

do unix

connect

Interactive Operation:

To  run  Kermit-MS  interactively, invoke the program from DOS command level by

typing its name.  When you see the command's prompt,

Kermit-MS>

you may type Kermit commands  repeatedly  until  you  are  ready  to  exit  the

program, for example:

A>

A>kermit

IBM PC Kermit-MS V2.29

Type ? for help

Kermit-MS>send foo.*

The files are sent.

Kermit-MS>get bar.*

The requested files are received.

Kermit-MS>exit

A>

During  interactive operation, you may edit the command you're currently typing

using BACKSPACE to erase the character most recently typed,  Ctrl-W  to  delete

the  most  recent  field,  or Ctrl-U to delete the entire command.  The editing

characters may be used in any combination until the command is finally  entered

by typing RETURN (Carriage Return, Enter) or Ctrl-L.

While  typing  commands,  you  may the help ("?") and keyword completion  (ESC)

features freely while typing Kermit-MS commands.  A question mark typed at  al-

most any point in a command produces a brief description of what is expected or

MS-DOS KERMIT                                                          Page 172

possible at that point (for this reason, Kermit-MS uses  "#"  for  the  single-

character  match  wildcard in the first position of the local filenames and "?"

thereafter, and ?-help is not available from within a filename).  ESC typed  at

any  point,  except  in  a  local  filename, will cause the current field to be

filled out if what you have typed so far is sufficient to identify it, and will

leave you in position to type the next field (or to type a "?" to find out what

the next field is); otherwise, the program will beep at you and wait for you to

type further characters.

Some Kermit-MS commands, like GET, SHOW KEY, SET KEY, may prompt for additional

information on subsequent lines.  If you have reached one of these prompts  and

then wish to cancel the command, you may type Control-C.

Summary of Kermit-MS Command Editing Characters:

SPACE   Separates fields within the command.

TAB     Same  as  Space,  and echoes as Space.  You may also use Ctrl-I for

Tab.

BACKSPACE

Deletes the character most recently typed.  May be typed repeatedly

to delete all the way back to the prompt.  You may also use DELETE,

RUBOUT, Ctrl-H, or equivalent keys.

Ctrl-W  Deletes the most recent "word", or field, on the command line.  May

be typed repeatedly.

Ctrl-U  Deletes the entire command line, back to the prompt.

Ctrl-C  Cancels the current command and returns to the "Kermit-MS>" prompt.

ESC     If enough characters have been supplied in the current  keyword  to

identify  it uniquely, supply the remainder of the field, and posi-

tion to the next field of the command.  Otherwise,  sound  a  beep.

ESC does not provide filename completion in version 2.29.

?       Displays  a  brief message describing what may be typed in the cur-

rent command field.  Also,  wildcard  character  for  matching  any

single character in all but the first position of a filename.

#       Wildcard  character  for  matching  single characters in filenames.

Equivalent to MS-DOS "?", but used  in  the  first  position  of  a

filename only, so that "?" may be used to get help at the beginning

of a filename field.

RETURN  Enters the command.  On most keyboards, you may also use  ENTER  or

Ctrl-M.

Ctrl-L  Clears the screen and enters the command.

Liberal  use  of "?" allows you to feel your way through the commands and their

fields.  This feature is sometimes called "menu on demand"  --  unlike  systems

that  force  you to negotiate menus at every turn, menu-on-demand provides help

only when it is needed.

MS-DOS KERMIT                                                          Page 173

Command Line Invocation:

Kermit-MS may also be invoked with command  line  arguments  from  DOS  command

level, for instance:

A>kermit send foo.bar

or

A>kermit set port 1, set baud 9600, connect

In  this case, help and completion  are not available (because the program that

provides them won't start running until  after  you  type  the  entire  command

line),  and Kermit-MS will exit back to DOS after completing the specified com-

mand or commands.    Therefore,  when  invoked  with  command  line  arguments,

Kermit-MS  will  behave as if it were an external DOS command, like MODE.  Note

that several commands may be given on the command line, separated by commas.

Batch Operation:

Like other MS-DOS programs, Kermit-MS may be operated under batch with  command

line  arguments.   If you invoke it without command line arguments, it will run

interactively, reading commands from the keyboard and not the batch file.  When

it exits, batch processing will continue to the end of the batch file.

Remote Operation:

The MS-DOS CTTY command allows an MS-DOS system to be used from a terminal con-

nected to its communication port.  Such sessions must be conducted  with  great

care, since many programs assume that they are running on the real console, and

explicitly reference screen memory or keyboard scan codes.  Kermit can be  used

in this manner too, but before you give it any file transfer commands, you must

inform it that it is running in "remote mode" rather  than  its  normal  "local

mode."    Use  the  SET REMOTE ON command for this purpose, to prevent the file

transfer display from being sent out the port.

MS-DOS KERMIT                                                          Page 174

11.3. Kermit-MS Commands

MS-DOS Kermit implements a large subset of  the  commands  of  "ideal"  Kermit.

Here's a brief summary:

BYE  to remote server.

CLEAR  key redefinitions.

CLOSE  log file and stop logging remote session.

COMMENT  a command file.

CONNECT  as terminal to remote system.

CWD  change local working directory.

DEFINE  macros of Kermit-MS commands.

DELETE  local files.

DIRECTORY  listing of local files.

DO  a macro expansion.

EXIT  from Kermit-MS.

FINISH  Shut down remote server.

GET  remote files from server.

HANGUP  the phone.

HELP  about Kermit-MS.

LOCAL  prefix for local file management commands.

LOG  remote terminal session.

LOGOUT  remote server.

PUSH  to MS-DOS command level.

QUIT  from Kermit-MS

RECEIVE  files from remote Kermit.

REMOTE  prefix for remote file management commands.

RUN  an MS-DOS program.

SEND  files to remote Kermit.

SERVER  mode of remote operation.

SET  various parameters.

SHOW  various parameters.

SPACE  inquiry (about disk space).

STATUS  inquiry (about settings).

TAKE  commands from file.

TYPE  display a local file.

VERSION  display Kermit-MS program version number.

The  remainder  of  this section concentrates on the commands that have special

form or meaning for MS-DOS Kermit.  Not  all  of  the  following  commands  are

necessarily  available on all MS-DOS systems, and some of the commands may work

somewhat differently between DOS versions.

The notation used is as follows: Optional  fields  are  in  [square  brackets],

lists  of  alternatives  are in {curly braces}.  Parameters, such as numbers or

filenames, are shown in italics (providing the printer is capable  of  printing

italics),  and  in  dialog examples user typein is underlined (on printers that

can show it) to distinguish it from computer typeout.

MS-DOS KERMIT                                                          Page 175

11.3.1. Commands for Terminal Connection

The CONNECT command connects your PC as a terminal to  the  remote  system,  so

that you can start up Kermit there.

The CONNECT Command

The  CONNECT command establishes an interactive terminal connection to the sys-

tem connected to the currently selected communications port (e.g. COM1 or COM2)

using  full duplex (remote) echoing and no parity unless otherwise specified in

previous SET commands.  You can type the escape character followed by the  let-

ter C to get back to Kermit-MS.  On most MS-DOS systems the escape character is

Ctrl-] by default.

You can use the SET ESCAPE command to define a different escape character,  and

on  most  systems  you can SET BAUD (or SPEED) to change the baud rate, and SET

PORT to switch between ports.  Read about the SET COMMAND in section 11.3.6 for

more information on these and other settings.

Terminal emulation is described in greater detail in section 11.4 below.

The HANGUP Command

HANGUP command attempts to lower the modem signals DTR and RTS.  It may be used

to hang up the phone when dialed up through a modem, or to get the attention of

port contention units or terminal concentrators that operate in this manner.

11.3.2. Commands for File Transfer

The file transfer commands are SEND, GET, and RECEIVE.

The SEND Command

Syntax:  SEND filespec1 [filespec2]

The  SEND  command causes a file or file group to be sent from the local MS-DOS

system to the Kermit on the remote system.  The remote Kermit may be running in

either  server or interactive mode; in the latter case, you should already have

given it a RECEIVE command and escaped back to your PC.

filespec1 may contain the wildcard characters "*" to match zero or more charac-

ters  within  a  field, and/or "#" (first position) or "?" (elsewhere) to match

any single character.  If  filespec1  contains  wildcard  characters  then  all

matching files will be sent, in the same order that MS-DOS would show them in a

directory listing.  If filespec1  specifies  a  single  file,  you  may  direct

Kermit-MS to send that file with a different name, given in filespec2.  For in-

stance, in the command

Kermit-MS>send foo.bar framus.widget

filespec2 begins with the first nonblank character  after  filespec1  and  ends

with  the  carriage return; thus it may contain blanks or other unusual charac-

MS-DOS KERMIT                                                          Page 176

ters that may be appropriate on the target machine.   The  alphabetic  case  of

text in filespec2 is preserved in transmission.

If  the  SEND command is specified by itself on the command line, then you will

be prompted separately for the name of the file to send, and the name  to  send

it under:

Kermit-MS>send

Local Source File: c:\chris\xcom1.txt

Remote Destination File: com1.txt

If  a file can't be opened for read access, standard MS-DOS recovery procedures

will take place.  For example:

Not ready error reading drive A

Abort, Retry, Ignore?

If you select "Abort," you will be returned to DOS.

Files will be sent with  their  MS-DOS  filename  and  filetype  (for  instance

FOO.TXT,  no  device or pathname).  If there is no filetype, then only the name

will be sent, without the terminating dot.  Each file is sent as  is,  with  no

conversions  done  on  the  data, except for possibly adding or deleting a ter-

minating Control-Z character (see the SET EOF command).

Once you give Kermit-MS the SEND command, the name of each file  will  be  dis-

played  on your screen as the transfer begins.  Packet, retry, and other counts

will be displayed along with informational messages during the  transfer.    If

the  file is successfully transferred, you will see "Complete", otherwise there

will be an error message.  When the specified operation is  done,  the  program

will sound a beep.

Several  single-character  commands  may  be  given while a file transfer is in

progress:

^X  (Control-X) Stop sending the current file and go on to the next  one,  if

any.

^Z  Stop sending this file, and don't send any further files.

^C  Return to Kermit-MS command level immediately without sending any kind of

notification to the remote system.

^E  Like ^C, but send an Error packet to the remote Kermit in an  attempt  to

bring it back to server or interactive command level.

CR  Simulate a timeout: resend the current packet, or NAK the expected one.

Control-X,  Control-Z,  and  Control-E send the proper protocol messages to the

remote Kermit to bring it gracefully to the desired state.    Control-C  leaves

the  remote  Kermit in whatever state it happens to be in, possibly retransmit-

ting its last packet over and over, up to its retry limit.    You  should  only

have  to  use  Control-C  in  dire emergencies (the remote Kermit is stuck, the

remote system crashed, etc), or at those times when you realize that  you  have

given a file transfer command to Kermit-MS without first having told the remote

Kermit about it.

MS-DOS KERMIT                                                          Page 177

The RECEIVE Command

Syntax: RECEIVE [filespec]

The RECEIVE command tells Kermit-MS to receive a file or file  group  from  the

other  system.   Kermit-MS passively waits for the file to arrive; this command

is not to be used when talking to a Kermit server (use  GET  for  that).    You

should already have issued a SEND command to the remote Kermit and escaped back

to Kermit-MS before issuing the RECEIVE command.

If the optional filespec is provided, the first incoming file  will  be  stored

under  that  name.    The filespec may include any combination of the following

fields:

Device designator

Store  the  file  on  the  designated device.  If no device designator is

given, store it on the current default device.

Directory path

Store  the file in the designated directory.  If no path given, store the

file in the current directory.

File name

Store the file under the name given.  If no name is given, store it under

the name it was sent under, converted, if necessary, to suit DOS  conven-

tions,  and  modified,  if  desired, to avoid overwriting any file of the

same name in the same directory.

If the optional filespec was provided, but more  than  one  file  arrives,  the

first  file  will be stored under the given filespec, and the remainder will be

stored under their own names, but on the specified device and directory.

If an incoming file does not arrive in its entirety,  Kermit-MS  will  normally

discard  it  and it will not appear in your directory.  You may change this be-

havior by using the command SET INCOMPLETE KEEP, which will cause  as  much  of

the file as arrived to be saved on the disk.

The same single-character commands are available as during SEND:

^X  Request that the remote Kermit stop sending the current file, and proceed

to the next one immediately.  Since this is an optional  feature  of  the

Kermit protocol, the remote Kermit might not honor the request.

^Z  Request  that  the  remote  Kermit terminate the entire transfer; this is

also an optional feature that may or may not be supported by  the  remote

Kermit.

^C, ^E, and CR operate in the same way as they do during SEND.  In this case,

^E should always do what ^Z is supposed to do.

If the incoming file has the same name as a file that already exists, and WARN-

ING  is  set ON, Kermit-MS will change the incoming name (and inform you how it

renamed it) so as not to obliterate the pre-existing file.  If WARNING is  OFF,

the  original  file  will be overwritten; if you type ^X or ^Z to interrupt the

transfer, you'll either get a partial new file, or else both the  old  and  the

new  file of that name will be lost, depending on SET INCOMPLETE.  In any case,

MS-DOS KERMIT                                                          Page 178

when WARNING is off, files with the same name as incoming files will  not  sur-

vive.

Caution: If an incoming file's name (the part before the dot) corresponds to an

MS-DOS device name, such as NUL, COM1, CON, AUX, or PRN, output will go to that

device, rather than to a file with that name.  This is a feature of MS-DOS.

The GET Command

Syntax: GET remote-filespec

The  GET command requests a remote Kermit server to send the file or file group

specified by remote-filespec.  This command can be used only when Kermit-MS has

a Kermit server active on the other end of the connection.  This means that you

must have CONNECTed to the other system, logged in, run  Kermit  there,  issued

the SERVER command, and escaped back (e.g. ^]C) to the local Kermit-MS.  If the

remote Kermit does not have a SERVER command, then  you  should  use  SEND  and

RECEIVE as described above.

You  may  use  the GET command in a special way to specify a different name for

storing the incoming file.  Just type GET alone on a  line,  and  you  will  be

prompted separately for the remote filespec and the local filespec:

Kermit-MS>get

Remote Source File: com1 txt

Local Destination File: a:xcom1.txt

The  local  file name may contain a device field, and/or a directory specifica-

tion.  If more than one file arrives, only the first will be renamed.    Device

and  directory  specifications in the local destination file name work the same

way as in the RECEIVE command.

The remote filespec is any string that can be a legal  file  specification  for

the  remote  system;  it  is  not  parsed or validated locally.  It can contain

whatever wildcard or file-group notation is valid on the  remote  system.    As

files  arrive,  their names will be displayed on your screen, along with packet

traffic statistics and status messages.  You may type ^X to  request  that  the

current  incoming  file  be  cancelled,  ^Z to request that the entire incoming

batch be cancelled, and ^C or  ^E  to  return  immediately  to  the  Kermit-MS>

prompt, exactly as described for the RECEIVE command.

11.3.3. Commands for Controlling Remote Kermit Servers

The  BYE,  FINISH,  and  LOGOUT commands allow you to shut down a remote Kermit

server:

BYE     When communicating with a remote Kermit server, use the BYE command  to

shut  down the server, log out its job, and exit locally from Kermit-MS

to DOS.

FINISH  Like BYE, FINISH shuts down the remote server.   However,  FINISH  does

not  log  out the server's job.  You are left at Kermit-MS prompt level

so that you can connect back to the job on the remote system.

MS-DOS KERMIT                                                          Page 179

LOGOUT  The LOGOUT command is identical to the BYE  command,  except  you  will

remain  at Kermit-MS prompt level, rather than exit to DOS, so that you

can establish or use another connection.

Other functions may be also requested of a server, described under  the  REMOTE

command, immediately below.

11.3.4. Commands for File Management

Kermit-MS provides commands or managing both local and remote files.

The REMOTE Commands

The REMOTE keyword is a prefix for a number of commands.  It indicates that the

command is to be performed by the remote Kermit, which must  be  running  as  a

server.    Note  that not all Kermit servers are capable of executing all these

commands, and some Kermit servers may be able to perform  functions  for  which

Kermit-MS  does  not  yet  have the corresponding commands.  In case you send a

command the server cannot execute, it will send back a message stating that the

command  is  unknown  to  it.  If the remote server can execute the command, it

will send the results to your screen (or whatever device you have specified  in

your most recent SET DESTINATION command).

Here are the REMOTE commands that Kermit-MS may issue:

REMOTE CWD [directory]

Ask the server to Change your Working Directory  on  the  remote  host,

that  is, the default source and destination area for file transfer and

management.  You will be prompted for a password, which will  not  echo

as  you type it.  If you do not supply a password (i.e. you type only a

carriage return), the server  will  attempt  to  access  the  specified

directory  without  a password.  If you do not supply a directory name,

your default or login directory on the remote system  will  be  assumed

and you will not be prompted for a password.

REMOTE DELETE filespec

Ask the server to delete the specified file or files on the remote sys-

tem.  In response, the server may display a list of the files that were

or were not successfully deleted.

REMOTE DIRECTORY [filespec]

Ask  the  server to display a directory listing of the specified files.

If no files are specified, then the list should include  all  files  in

the the current working directory.

REMOTE HELP

Ask the server to list the services it provides.

REMOTE HOST [command]

Ask  the  server  to  send  the  command to the remote system's command

processor for execution.

KERMIT command

Send  the  command  to the remote Kermit for interpretation as a Kermit

MS-DOS KERMIT                                                          Page 180

command itself, in the remote Kermit server's own command syntax.  Most

Kermit  servers,  including  Kermit-MS,  do not yet recognize such com-

mands.

REMOTE SPACE [directory]

Ask  the  server  to  provide  a  brief  summary  of  disk usage in the

specified area on the remote host or, if none specified, the default or

current area.

REMOTE TYPE filespec

Ask the server to display the contents of the specified remote file  or

files on your screen.

The LOCAL Command

The  LOCAL keyword is a prefix for a number of commands.  It indicates that the

specified command is to be executed on the local  MS-DOS  system.    The  LOCAL

prefix  may  be  omitted.    All  file specifications may include device and/or

directory fields.  The local commands are:

CWD path

Changes  the  current  working  directory  to  the  given  path.    All

references to local file names without explicit  paths  will  refer  to

that path.

DELETE filespec

Deletes the specified file or files.  As  in  DOS,  the  names  of  the

deleted  files  are  not  listed, only the message "file(s) deleted" or

"file(s) not  found",  and  if  you  give  the  command  "delete  *.*",

Kermit-MS will prompt "Are you sure?", like DOS.

DIRECTORY [filespec]

Lists the names, sizes, and creation dates  of  files  that  match  the

given  file  specification.    If  no filespec is given, the command is

equivalent to DIR *.*.

SPACE   Performs the MS-DOS CHKDSK function by running the CHKDSK program  from

the current path.

RUN command

Passes the command line to COMMAND.COM for execution.  Thus, any  legal

DOS  operation  is  permitted: running a program (with command line ar-

guments or i/o redirection), executing a DOS command,  or  executing  a

batch  file.    COMMAND.COM  should  be in the current path.  Kermit is

suspended while the command is executed and automatically  resumes  af-

terward.    Yes,  one  may  nest  RUN KERMIT several times if memory is

available; but that just wastes memory.  If the command is a  DOS  com-

mand  (like  DIR  or  REN) it will be executed directly by COMMAND.COM.

Otherwise, the specified file will be executed, which must be  in  .EXE

or  .COM  format,  or  a  .BAT  file  containing DOS commands, from the

specified path or according to the value of the  PATH  variable  if  no

path was included in the filespec.  Example:

Kermit-MS>run more < foo.txt

MS-DOS KERMIT                                                          Page 181

TYPE filespec

Displays the specified local file on the screen.   Automatic  pause  is

not  available  at  the end of a page (but see above example for how to

accomplish this).  On most systems, Ctrl-S can be used to stop  scroll-

ing and Ctrl-Q to continue scrolling.

PUSH    Invokes   an   MS-DOS   command  processor  "under"  Kermit-MS,  either

COMMAND.COM or whatever shell  you  have  specified  with  COMSPEC  (or

SHELL,  depending  on  the  system)  in  your CONFIG.SYS file.  You can

return to Kermit-MS by typing the MS-DOS EXIT  command,  and  you  will

find  Kermit-MS  as  you  left  it, with all settings intact.  The same

function is accomplished by the CONNECT escape-level command P.

The local RUN command has various uses, one of which is to supplement the  fea-

tures  of Kermit-MS.  A common complaint about Kermit-MS, for instance, is that

it lacks login scripts, autodialer control, and phone directories.  While wait-

ing  for  these  features  to  appear in a generally useful form in some future

release, you might be able to provide the specific functions you need by  writ-

ing a program in any compiled language (assembler, Pascal, C, etc) and then in-

voking from within Kermit-MS using the RUN command.  For instance, if you  have

a  Hayes-like modem, you could write a short program to operate the dialer with

AT commands, accepting the phone number to dial on the command line.   If  this

program were called DIAL, then you could "run dial 7654321" from within Kermit.

The TAKE Command

Syntax: TAKE filespec

The  TAKE  command  instructs  Kermit-MS to execute commands from the specified

file, which may include an explicit path; if no path is specified, the value of

the  PATH  variable  is  used;  if PATH has no value, then the current disk and

directory are searched.  The command file may include any valid Kermit-MS  com-

mands,  including TAKE, but it cannot include characters to be sent to a remote

host during terminal emulation (i.e.  after a CONNECT command).

Commands within TAKE files, unlike interactive commands, may  include  trailing

comments, preceded by semicolons:

set port 2      ; Select the modem port

set speed 1200  ; Set the baud rate for the modem

Warning:  since TAKE file processing discards all characters from a line begin-

ning with the first semicolon, it is not  possible  to  include  semicolons  in

remote filespecs within TAKE files, e.g.

get dska:foo.bar;6

Commands  from the TAKE file will normally not be displayed on your screen.  If

you want to see them as they are executing, you can SET TAKE-ECHO ON.

MS-DOS KERMIT                                                          Page 182

The LOG and CLOSE Commands

Syntax: LOG filespec

CLOSE

Specifies that your terminal session during CONNECT will  be  recorded  in  the

specified  file;  the filespec may include a device specification and/or direc-

tory path.  The LOG command allows you to "capture" files from a remote  system

that doesn't have Kermit, as well as to record remote command typescripts.  The

log is closed when you EXIT from Kermit-MS or when you issue an explicit  CLOSE

command.

During  terminal emulation, the LOG command records all the characters that ar-

rive from the remote host in the specified file,  including  escape  sequences.

If  you  have  SET  LOCAL-ECHO ON, it will also record the characters you type.

And if you have also SET DEBUG ON, then  during  file  transfer  it  will  also

record the packets in the log.

You may LOG PRN to cause the logging information to be printed directly on your

printer.  Note that any escape sequences that are sent to the screen  are  also

sent to the printer.

If  you  want  to record information without imbedded escape sequences, use the

screen dump feature, invoked by the CONNECT escape-level command  F,  which  is

described in more detail in the terminal emulation section.

11.3.5. The SERVER Command

Kermit-MS  is capable of acting as a full-fledged Kermit server, providing file

transfer and management for users coming in through one  of  the  communication

ports.  To put Kermit-MS into server mode, first issue any desired SET commands

to select and configure the desired port, and then  type  the  SERVER  command.

Kermit-MS will await all further instructions from the user Kermit on the other

end of  the  connection,  which  may  be  hardwired  or  connected  through  an

autoanswer modem.  For example:

Kermit-MS>set port 1

Kermit-MS>set baud 1200

Kermit-MS>set timer on

Kermit-MS>set warning on

Kermit-MS>server

Kermit 2.29 server mode supports the following requests:

SEND           REMOTE DELETE       REMOTE CWD

GET            REMOTE DIRECTORY    REMOTE HOST

FINISH         REMOTE SPACE

BYE            REMOTE TYPE

The  REMOTE  HELP command is not implemented.  Remote CWD can be used to change

both directories and devices.

CAUTION: The method used for most of the REMOTE commands is to invoke a

task  with  the user's command line, redirect standard output to a tem-

MS-DOS KERMIT                                                          Page 183

porary file, $KERMIT$.TMP, send that file back to the remote  end,  and

then delete the file.  Sufficient space must be available to store this

file.  To service DOS commands or user tasks the boot drive must hold a

copy  of  COMMAND.COM.    PATH  will  not be searched (this can be dis-

asterous on a floppy disk based system).

FURTHER CAUTION: Any of these DOS tasks or programs  may  encounter  an

error,  and  in  that case, DOS will generally put the familiar "Abort,

Retry, Ignore?" message on the screen, and will wait for an answer from

the  keyboard.    This  will hang the server until a human comes to the

keyboard and gives a response.  The same thing  will  happen  when  any

program is invoked that interacts with the real console.  For instance,

REMOTE SPACE works by running CHKDSK; if CHKDSK finds  something  wrong

with the disk while tallying up the space, it will ask (at the console)

if you want to it to be fixed.  This, too, will hang the server.

MORAL: The MS-DOS Kermit server should probably not be used for  REMOTE

commands  unless  someone  is  around  to  take care of it when it gets

stuck.

11.3.6. The SET Command

Syntax: SET parameter value

or: SET parameter parameter value

The SET command establishes or modifies various parameters for file transfer or

terminal  connection.    You  can examine their values with the STATUS and SHOW

commands.  The following SET commands are available in Kermit-MS:

BAUD  Communications port line speed (synonym for SPEED)

BELL  Whether to beep at the end of a transaction

BLOCK-CHECK-TYPE  Level of error checking for file transfer

DEBUG  Display packet contents during file transfer

DEFAULT-DISK  Default disk drive for file i/o

DESTINATION  Default destination device for incoming files

DISPLAY  For selecting the type of file transfer display

DUMP  Screen dump file (or device) name

END-OF-LINE  Packet termination character

EOF  Method for determining or marking end of file

ESCAPE  Escape character for CONNECT

FLOW-CONTROL  Enable or disable XON/XOFF

HANDSHAKE  Half-duplex line turnaround option

INCOMPLETE  What to do with an incompletely received file

KEY  Specify key redefinitions, or "keystroke macros"

LOCAL-ECHO  Specify which computer does the echoing during CONNECT

MODE-LINE  Whether to display a mode line during terminal emulation

PARITY  Character parity to use

PORT  Select a communications port

PROMPT  Change the "Kermit-MS>" prompt to something else

RECEIVE  Request remote Kermit to use specified parameters

REMOTE  For running Kermit-MS interactively from back port

RETRY  Packet retransmission threshold

SEND  Use the specified parameters during file transfer

SPEED  Communications port line speed (synonym for BAUD)

MS-DOS KERMIT                                                          Page 184

TAKE-ECHO  Control echoing of commands from TAKE files

TERMINAL  Emulation and parameters

TIMER  Enable/disable timeouts during file transfer

WARNING  Specify how to handle filename collisions

The SET commands are now described in greater detail.

SET BAUD

Syntax: SET BAUD rate

Set the speed of the currently selected terminal communications port  (COM1  by

default)  to  300,  1200,  1800,  2400, 4800, 9600, 19200, or other common baud

rate.  Some implementations  do  not  support  this  command.    In  any  case,

Kermit-MS leaves the current communication port settings alone unless you issue

explicit SET commands to change them.  SET SPEED is an acceptable  synomym  for

SET BAUD.  Note that on certain systems, when you first run Kermit after power-

ing the system up, you may get a message "Unrecognized baud rate".  This  means

that Kermit tried to read the baud rate from the port and none was set.  Simply

use SET BAUD (if available) or the DOS MODE command to  set  the  desired  baud

rate.

SET BELL

Syntax: SET BELL {ON, OFF}

Specifies  whether  the  bell  (beeper)  should sound upon completion of a file

transfer operation.  Normally ON.

SET BLOCK-CHECK

Syntax: SET BLOCK-CHECK {1, 2, 3}

Selects the error detection method: a 1-character checksum (the normal case), a

2-character  checksum,  or  a 3-character 16-bit cyclic redundancy check (CRC).

If the other Kermit program is not capable of type 2  or  3  checking  methods,

automatic fallback to type 1 will occur.

SET DEBUG

Syntax: SET DEBUG {ON, OFF}

When  debugging is ON, Kermit will display packet traffic on your screen during

file transfer.  If the debugger is loaded, control will be  transferred  to  it

when  Ctrl-C is typed.  During terminal emulation (on the IBM PC only), control

characters are displayed in uparrow notation.  If logging is being  done,  file

transfer  packets  are  included  in  the log.  When OFF (this is the default),

debugging information is not displayed, and packets are not logged.

MS-DOS KERMIT                                                          Page 185

SET DEFAULT-DISK

Syntax: SET DEFAULT-DISK x:

Specify the default disk drive to use for file  transfer,  directory  listings,

and so forth.  Equivalent to typing the DOS command for changing disks (A:, B:,

etc).  Affects Kermit and all inferior processes, but when you exit  from  Ker-

mit, you will still have the same default disk as when you entered.

SET DESTINATION

Syntax: SET DESTINATION device

Specify  the  device for incoming files during file transfer: DISK, PRINTER, or

SCREEN.  SET DESTINATION PRINTER  will  cause  incoming  files  to  be  spooled

directly to the printer; SCREEN will send output normally destined for the disk

to the screen.  The normal destination is DISK.  SET DESTINATION  affects  only

files  transferred with SEND, GET, or RECEIVE; it cannot be used to reroute the

output from REMOTE server commands.

SET DISPLAY

Syntax: SET DISPLAY {QUIET, REGULAR, SERIAL}

During file transfer, MS-DOS Kermit's regular display  is  a  formatted  screen

whose  fields  randomly  updated with file names, packet numbers, error counts,

percent done, error messages, and so forth:

File name: FOO

KBytes transferred: 7

Sending: In progress

Percent done: 52%

Number of packets: 74

Number of retries: 2

Last error: None

Last warning: None

The items in the right-hand column are updated more or less at random.

If you wish to run Kermit-MS interactively through the back port, for  instance

after  the operator has done CTTY COM1, you must give the command SET REMOTE ON

(which, currently at least, is equivalent to  SET  DISPLAY  QUIET);  this  sup-

presses  the  file transfer display screen, so that the display won't interfere

with the file transfer itself.  You can also use this command to  suppress  the

display  in  local  mode,  in case you are using a system that allows you to do

other work while file transfer proceeds in the background.

If you have your PC connected to a  speaking  device  (a  common  practice  for

visually  impaired  people), or you are logging the display screen to a printer

(using DOS ^P or kermit > prn), the random nature of the regular  display  will

make  the  results of little use.  SET DISPLAY SERIAL is provided for this pur-

pose; it causes the program to report progress "serially" on the  screen.    In

serial mode, error messages are preceeded with the word "Error" and repeat mes-

sages with the word "Retry".  Packets are numbered as dots with every tenth be-

MS-DOS KERMIT                                                          Page 186

ing  a  plus  sign.  The packet display is automatically broken across lines at

every 70th packet.  The serial display makes much more sense when  spoken  than

does the regular display.

The  serial display does not show the percent and kilobytes transferred.  It is

the default display style for generic MS-DOS Kermit; REGULAR is the default for

all others.

SET DUMP

Syntax: SET DUMP path

On  those  systems that support this feature, change the file or device name of

the screen dump file.  The normal file name is KERMIT.SCN.  See the section  on

terminal emulation for details about screen dumps.

END-OF-LINE

Syntax: SET END-OF-LINE number

If the remote system needs packets to be terminated by anything other than car-

riage return, specify the decimal value of the desired ASCII character.

SET EOF

Syntax: SET EOF {CTRL-Z, NOCTRL-Z}

Controls how the end of file is handled.  CTRL-Z specifies a Control-Z  charac-

ter  should  be appended to the end of an incoming file, unless it already ends

with a Control-Z.  Certain MS-DOS text editors and other  applications  require

files  to  be in this format.  For outbound files, treat the first Control-Z as

the end of the local file, and do not send it  or  any  subsequent  characters.

NOCTRL-Z  is the default; incoming files are stored, and MS-DOS files are sent,

exactly as is, in their entirety.

SET ESCAPE

Syntax: SET ESCAPE character

Specify the control character you want to use to "escape" from  remote  connec-

tions  back  to  Kermit-MS.  The default is normally ^] (Control-Rightbracket).

The character is entered literally, and should normally be chosen from the  AS-

CII  control range.  It is not possible to use non-ASCII characters (e.g. func-

tion keys) for this purpose.

SET FLOW-CONTROL

Syntax: SET FLOW-CONTROL {XON/XOFF, NONE}

Specify the full duplex flow control to be done on the currently selected port.

The  options are XON/XOFF and NONE.  The specified type of flow control will be

MS-DOS KERMIT                                                          Page 187

done during both terminal emulation and file transfer.   By  default,  XON/XOFF

flow  control  is  selected.  XON/XOFF should not be used on half-duplex (local

echo) connections.  If XON/XOFF is used, HANDSHAKE should be set to NONE.

SET HANDSHAKE

Syntax: SET HANDSHAKE {CODE number, BELL, CR, LF, NONE, XOFF, XON}

Specify any half-duplex line turnaround handshake character for  the  currently

selected port.  The CODE number form allows any ASCII character to be specified

by its decimal ASCII code.  The  specified  handshaking  is  done  during  file

transfer  only.   Handshake is NONE by default; if set to other than NONE, then

FLOW-CONTROL should be set to NONE.

SET INCOMPLETE

Syntax: SET INCOMPLETE {DISCARD, KEEP}

Specifies what to do with files that arrive incompletely: discard them or  keep

them.  They are normally discarded.

SET KEY

Syntax: SET KEY key-specifier

Specifies that when the designated key is struck during terminal emulation, the

associated character string is sent.  The key-specifier is one of the  keywords

F1,  F2,  ..., or else SCAN followed by a scan code.  Systems that have a BACK-

SPACE key also include BACKSPACE as a keyword.

If SCAN is used, it is followed by a decimal number to indicate the  scan  code

of  the  key,  which  you would ascertain from your system reference manual, or

else by using the Kermit-MS SHOW KEY command.  SET KEY prompts  you  on  a  new

line  for  the definition string.  Certain characters, like ESC and CR, may not

be entered literally into the string, but can be included by  inserting  escape

codes  of  the  form \ooo, a backslash followed by a 2- or 3-digit octal number

corresponding to the ASCII value of the desired character.  If some  other  key

redefinition package, like ProKey, has been loaded, then its redefinitions will

take precedence over Kermit's.

The SET KEY command is illustrated in the  terminal  emulation  section  below.

Note  that  key  redefinitions occur only during terminal emulation, and not at

Kermit-MS command level, or outside of Kermit.

SET LOCAL-ECHO

Syntax: SET LOCAL-ECHO {ON, OFF}

Specify how characters are echoed during terminal emulation  on  the  currently

selected  port.    ON  specifies  that characters are to be echoed by Kermit-MS

(because neither the remote computer nor the communications circuitry has  been

requested to echo), and is appropriate for half-duplex connections.  LOCAL-ECHO

MS-DOS KERMIT                                                          Page 188

is OFF by default, for full-duplex, remote echo operation.

When using Kermit to connect two PCs "back to back," use  local  echo  so  that

when  you CONNECT to the other PC to send messages to its operator, you can see

what you are typing.  Depending on the system, you may have to type a  carriage

return and a linefeed at the end of each line in order to make the display look

right.

SET MODE-LINE

Syntax: SET MODE-LINE {ON, OFF}

On systems, like the IBM PC family, which are capable of displaying  a  status,

or  "mode"  line on the 25th line during terminal connection, disable or enable

this function.  Has no effect on systems that do not display a mode line during

connect.   When the mode line is enabled, it may be turned on and off using the

CONNECT escape-level command M. Refer to the terminal emulation section,  11.4,

for further details.

SET PARITY

Syntax: SET PARITY {EVEN, ODD, MARK, SPACE, NONE}

Specify  the  character parity to be used on the currently selected port.  NONE

means no parity processing is done, and the 8th bit of each  character  can  be

used for data when transmitting binary files.  This is the normal case.

You  will  need  to  SET PARITY to ODD, EVEN, MARK, or possibly SPACE when com-

municating with a system, or over a network, or through modems,  concentrators,

multiplexers, or front ends that require or impose character parity on the com-

munication line.  For instance, GTE  Telenet normally uses MARK parity.  If you

neglect  to  SET  PARITY  when  the  communications  equipment requires it, the

symptom may be that terminal emulation works (well or  maybe  only  partially),

but file transfer does not work at all.

If  you  have  set parity to ODD, EVEN, MARK, or SPACE, then Kermit-MS will re-

quest that binary files will be transferred using 8th-bit-prefixing.    If  the

other  Kermit knows how to do 8th-bit-prefixing (this is an optional feature of

the Kermit protocol, and not all implementations of Kermit have it),  then  bi-

nary  files  can  be  transmitted successfully.  If NONE is specified, 8th-bit-

prefixing will not be requested.  Note that there  is  no  advantage  to  using

parity; it reduces Kermit's file transfer efficientcy without providing any ad-

ditional error detection.  The SET PARITY command is  provided  only  to  allow

Kermit  to  adapt to conditions where parity is required, or 8-bit transmission

is otherwise thwarted.

SET PORT

Syntax: SET PORT {number, COM1, COM2}

On machines with more than one communications port, select the port to use  for

file  transfer and CONNECT.  This command lets you use a different asynchronous

adapter, or switch between two or more  simultaneous  remote  sessions.    Sub-

MS-DOS KERMIT                                                          Page 189

sequent  SET  BAUD, PARITY, HANDSHAKE, FLOW, and LOCAL-ECHO commands will apply

to this port only.  SET PORT 1 selects COM1, SET PORT 2 selects COM2.  All ver-

sions  default to port 1, except for the IBM PCjr, which uses port 2 by default

(since Kermit does not know how to control its port 1, an internal modem).

In "generic" MS-DOS Kermit, the following alternate forms allow you to  experi-

ment with device names or numbers until you find the communication port:

SET PORT {DEVICE, FILE-HANDLE}

Just  type  a  carriage  return after either of these commands, and you will be

prompted for a device name or a numeric port-handle.  Keep trying till you find

one that works.

SET PROMPT

Syntax: SET PROMPT string

This command allows you to change the MS-DOS Kermit program's prompt.

SET RECEIVE

Syntax: SET RECEIVE parameter value

At  the beginning of a protocol operation, request the remote Kermit to use the

given value specified parameter, or inform Kermit-MS  that  the  remote  Kermit

will be using it.

PACKET-LENGTH number

Ask the remote Kermit to use the specified maximum length  for  packets

that  it  sends  to  Kermit-MS.  The normal (and maximum) length is 94.

Use this command to shorten packets if the communication line is noisy;

this  will  decrease  the  probability that a particular packet will be

corrupted, and will reduce the retransmission overhead when  corruption

occurs, but it will increase the protocol overhead.

PADCHAR number

Ask the remote Kermit to use the given control character (expressed  as

a  decimal  number  0-31,  or  127) for interpacket padding.  Kermit-MS

should never require any padding.

PADDING number

Ask  the remote Kermit to insert the given number of padding characters

before each packet it sends.  This should never be necessary.

START-OF-PACKET number

If  the  remote  Kermit will be marking the beginning of packets with a

control character other  than  Control-A,  use  this  command  to  tell

Kermit-MS  about  it (the number should be the decimal ASCII value of a

control character).  This will be necessary only if the hosts  or  com-

munication  equipment involved cannot pass a Control-A through as data,

or if some piece of communication equipment is echoing packets back  at

you.

MS-DOS KERMIT                                                          Page 190

TIMEOUT number

Ask the remote Kermit to time out after the given number of seconds  if

a  packet expected from Kermit-MS has not arrived.  Use this command to

change the normal timeout interval.

SET REMOTE

Syntax: SET REMOTE {ON, OFF}

SET REMOTE ON removes the file transfer display (as if you had given  the  com-

mand  SET  DISPLAY QUIET).  It should be used when you are running Kermit-MS in

remote mode (i.e. when coming in from another PC through the Kermit-MS's  "back

port",  to  which  the console has been reassigned using the DOS CTTY command).

It is necessary to issue this command because (a) Kermit-MS has no way of know-

ing  that its console has been redirected, and (b) when the console is the same

as the port, the file transfer display will interfere with  the  file  transfer

itself.    SET  REMOTE  OFF  returns the file transfer display to its preferred

style (REGULAR or SERIAL).

SET RETRY

Syntax: SET RETRY number

Sets the number of times a packet is retransmitted before  the  protocol  gives

up.   The number of retries can be between 1 and 63, and is 5 by default.  This

is an especially useful parameter when the communications line is noisy or  the

remote host is very busy.

SET SEND

Syntax: SET SEND parameter value

PACKET-LENGTH number

Use the specified maximum  length  for  outbound  packets.    Normally,

Kermit-MS uses whatever length the other Kermit requests.

PADCHAR number

Use the specified control character  for  interpacket  padding.    Some

hosts  may require some padding characters (normally NUL or DEL) before

a packet, and certain front ends or other communication  equipment  may

need certain control characters to put them in the right modes.

PADDING number

How many copies of the pad character to send before each  packet,  nor-

mally zero.

PAUSE number

How many milliseconds to pause between sending the next packet,  0-127,

normally zero.  This helps half-duplex systems prepare for reception of

our packet.  Padding characters are sent only after the time limit  ex-

pires.

QUOTE number

MS-DOS KERMIT                                                          Page 191

Use the indicated printable character for prefixing  (quoting)  control

characters and other prefix characters.  The only reason to change this

would be for sending a very long  file  that  contains  very  many  "#"

characters (the normal control prefix) as data.

START-OF-PACKET number

Mark the beginning of outbound  packets  with  some  control  character

other than Control-A.  This will be necessary if the remote host or the

communication channel involved cannot accept a Control-A as data, or if

it  echoes back your packets.  The remote host must have been given the

corresponding SET RECEIVE START-OF-PACKET command.

TIMEOUT number

Change  Kermit-MS's  normal timeout interval; this command is effective

only if TIMER is set to be ON; it is normally OFF so  that  the  remote

Kermit can control timeouts.  When the timer is ON, the default timeout

interval is 13 seconds.

SET SPEED

Syntax: SET SPEED rate

Same as SET BAUD, q.v.

SET TAKE-ECHO

Syntax: SET TAKE-ECHO {ON, OFF}

Specifies whether screen display should occur during implicit or explicit  TAKE

operations on MSKERMIT.INI or other Kermit-MS command files, and during evalua-

tion of macro definitions.  Handy for finding errors.

SET TERMINAL

Syntax: SET TERMINAL parameter [value]

This command controls  most  aspects  of  terminal  emulation.    Most  of  the

parameters  are  only  settable  (or  meaningful) on the IBM PC family and com-

patibles.  (Programmers who are proficient on other MS-DOS systems are  invited

to  fill  in  these  functions  for  those systems and send the results back to

Columbia.)

The first group of parameters tells which kind of terminal to  emulate.    When

Kermit-MS uses its built-in software for emulation, incoming characters are ex-

amined for screen control commands (escape sequences)  specific  to  that  ter-

minal, and if encountered, the commands are executed on the PC screen.

NONE    Act  as  a  dumb terminal.  All incoming characters will be sent to the

screen "bare", as-is, through DOS.  If you have loaded a device  driver

into DOS for the CON device, such as ANSI.SYS, then that driver will be

able to interpret the codes itself.  Many non-IBM  systems  have  their

own  screen  control  code  interpreter  built into DOS or firmware, or

available as a loadable device driver.

MS-DOS KERMIT                                                          Page 192

VT52    The DEC VT-52 terminal.

HEATH   The Heath/Zenith-19 terminal (H19), which supports all  the  VT52  com-

mands,  plus  line and character insert/delete editing functions, and a

25th line.

VT102   The DEC VT102 (ANSI) terminal, which is the same as a  VT100  but  also

supports line/character insert/delete editing functions.

TEKTRONIX

A Tektronix 4010 graphics terminal.  Only available on  TI  and  Victor

PCs.

The  specific  escape  sequences supported by Kermit for each of these terminal

types are listed in section 11.10.

The remaining SET TERMINAL commands specify setup options for the selected ter-

minal:

CHARACTER-SET {UK, US}

UK displays # as a pound sterling sign, US displays # as #.

COLOR number [, number [, number]]

Several  numbers, applied in left to right sequence, separated by com-

mas or spaces:

0   Reset the colors to normal intensity white characters on a black

background  and use the "no-snow" mode on the IBM Color Graphics

Adapter (CGA).

1   High intensity foreground

10  Request fast screen updating for use on the IBM EGA or PGA,  and

some non-IBM CGAs.

3x  Foreground color

4x  Background color

where x is a single digit from 0 to 7, which is the sum of the desired

colors:

1   Red

2   Green

4   Blue

Example: 0, 1, 34, 40 on an IBM CGA would produce blue  characters  on

black  field  with  no snow.  The snow removal business has to do with

whether the program should synchronize with vertical retrace when  up-

dating  screen  memory.  This is necessary with certain color adaptors

(like the CGA) and unnecessary for others (like the EGA).

CURSOR-STYLE {BLOCK, UNDERLINE}

Sets the cursor rendition to your preference.  Note that on some early

IBM PCs and compatibles, the cursor  may  not  be  restored  correctly

after escape back from CONNECT because of a bug in the early IBM BIOS.

KEYCLICK {ON, OFF}

Turns electronic keyclick ON or OFF.  If your keyboard has a  mechani-

cal  clicker (as IBM boards do), you may not notice the effect of this

MS-DOS KERMIT                                                          Page 193

command.

MARGIN-BELL {ON, OFF}

Controls  whether  the  bell  should be sounded when the cursor passes

column 72 near the right screen margin.

NEWLINE-MODE {ON, OFF}

ON  sends  a carriage-return-linefeed combination (CRLF) when you type

carriage return (CR) during terminal  emulation;  OFF  (default)  just

sends a CR when you type CR.

SCREEN-BACKROUND {NORMAL, REVERSE}

NORMAL means dark background, light characters.  REVERSE  means  light

background, dark characters.

TAB {AT n, CLEAR AT n, CLEAR ALL}

Sets tab stops or clears one or all tab stops.  n is the numeric posi-

tion  of  the  tab to be set or cleared.  By default, tabs are every 8

spaces, at positions 9, 17, 25, etc.  Only meaningful when emulating a

terminal  that  has  settable  tabs (the VT52 doesn't).  More than one

tabstop may be specified by separating  column  numbers  with  commas,

spaces, or tabs.

WRAP {ON, OFF}

ON automatically breaks screen lines (by inserting a CRLF)  when  they

reach  the the right margin; OFF disables wrapping -- if a line is too

long, the excess characters go off the screen.

SET TIMER

Syntax: SET TIMER {ON, OFF}

This command enables or disables the timer that is used during file transfer to

break  deadlocks  that  occur when expected packets do not arrive.  By default,

the timer is ON.

SET WARNING

Syntax: SET WARNING {ON, OFF}

Specify what to do when an incoming file has the same name as an existing  file

in  the  default  directory of the default device.  If ON, Kermit will warn you

when an incoming file has the same name as an existing file, and  automatically

rename  the  incoming  file  (as indicated in the warning message) so as not to

destroy (overwrite) any existing  one.    If  OFF,  the  pre-existing  file  is

destroyed,  even  if  the incoming file does not arrive completely.  WARNING is

OFF by default.

The new name is formed by adding numbers to the part of  the  name  before  the

dot.  For instance, ABC.TXT becomes ABC00001.TXT, then ABC00002.TXT, etc.

MS-DOS KERMIT                                                          Page 194

11.3.7. The SHOW Command

Syntax: SHOW option

Most  parameters  that  may  be  altered with SET commands are displayed by the

STATUS command.  The SHOW command is used for displaying macro definitions  and

key redefinitions.

The  SHOW MACROS command displays the definitions of all currently defined mac-

ros, as well as the amount of space left for new macro definitions.

The SHOW KEY command allows you to determine the scan code produced by pressing

a  given  key, so that you can construct a SET KEY SCAN command to redefine the

key.  If the key already has a redefinition in effect, that too  will  be  dis-

played.  This can be done either interactively or in a macro command.  Refer to

the terminal emulation section for examples.

The SHOW KEY command only works on certain systems.

11.3.8. Command Macros

Kermit-MS provides a facility for combining commands into  "macros."    Command

macro  definitions  may be included in your MSKERMIT.INI file, TAKEn explicitly

from a specified file, or typed interactively.  Macros are invoked with the  DO

command.

The DEFINE Command

Syntax: DEFINE macro-name [command [, command [, ...]]]

Kermit-MS  command  macros  are  constructed  with  the  DEFINE  command.   Any

Kermit-MS commands may be included.  Example:

define telenet set parity mark, set baud 1200, connect

A macro can be undefined by typing an empty DEFINE command for it, like

define telenet

A macro definition may be no longer than 128 characters.    Longer  definitions

can be accomplished by "chaining."  Example:

define setup set port 1, set speed 19200, set parity even, do setup2

define setup2 set port 2, set speed 1200, set parity none, do setup3

define setup3 set warning on, set incomplete keep, connect

The SHOW MACROS command displays the value of all currently defined macros, and

tells how much space is left for further definitions.

MS-DOS KERMIT                                                          Page 195

The DO Command

A Kermit-MS command macro is invoked using  the  DO  command.    For  instance,

Kermit-MS  comes  with  a  predefined  macro  to allow convenient setup for IBM

mainframe line-mode communications; to invoke it, you would type

do ibm

The IBM macro is defined as "set timer on, set local-echo on, set parity  mark,

handshake xon, set flow none".  You can use the DEFINE command to redefine this

macro or remove the definition altogether.

There is no automatic way to undo the effect of a macro.  If you  need  to  ac-

complish  this  effect,  you should define another macro for that purpose.  For

instance, to undo the effect of "do ibm" so that you could connect to,  say,  a

VAX, you could:

define vax set par no, set hand no, set flo x, set tim off, set loc off

Then  you  can  "do  ibm" whenever you want to use the IBM system, and "do vax"

whenever you want to use the VAX.

If you wish to view the macro expansion whenever you issue a  DO  command,  you

can SET TAKE-ECHO ON.

11.4. Terminal Emulation

When  you  issue the CONNECT command, your PC acts as a terminal connected to a

remote computer through the currently selected port.  The characters  you  type

are  sent out the port, and characters that arrive at the port are displayed on

your screen, or interpreted according to whatever type  of  terminal  is  being

emulated.

If  you  have  not previously issued a SET PORT command, COM1 is used except on

systems (like the IBM PCjr) where some other port is the default.  If you  have

SET LOCAL-ECHO ON for the selected port, then Kermit-MS will display characters

on the screen as you type them, otherwise it will rely on the remote system  to

echo them.  XON/XOFF flow control will be done unless you have SET FLOW-CONTROL

OFF.  If you have SET PARITY to anything other than NONE,  Kermit-MS  will  add

the  appropriate  parity  to each outbound character.  While CONNECTed, you can

also communicate directly with an autodialer or "smart modem"  to  control  the

communications  line, hang it up, and the like, for instance, by typing AT com-

mands to a Hayes-like modem.

When you CONNECT, the program attempts to raise the DTR and RTS RS-232 signals,

and  it  takes no specific action to lower them unless you explicitly issue the

HANGUP command.

The IBM PC version of Kermit-MS emulates the DEC VT102 terminal by default, and

may also be instructed to emulate the DEC VT52, the Heath/Zenith-19, or no ter-

minal at all, via the SET TERMINAL command.  Emulation of each  of  these  ter-

minals  is  nearly  complete.    VT102  emulation lacks only smooth scroll, 132

column mode, and ANSI printer control.  Double-height, double-width  characters

are supported, but simulated using ordinary characters.  On color monitors, the

foreground and background colors may be  set  using  SET  TERMINAL  COLOR,  and

MS-DOS KERMIT                                                          Page 196

inverse/normal  video  display may also be selected, along with many other ter-

minal parameters.  A complete list of the commands, default key configurations,

and  escape  sequences accepted by the IBM PC Kermit terminal emulator is given

in section 11.10.

The Escape Character

The escape character is used to regain the attention of Kermit-MS  during  CON-

NECT,  i.e.  terminal emulation.  When you type the escape character, Kermit-MS

waits for you to follow it with a single character command.  For instance,  the

single character command "?" produces a list of available single character com-

mands.  This command is executed immediately; it may not  be  edited,  and  the

program  does  not wait for a carriage return to confirm it.  Here are the CON-

NECT escape-level commands available in Kermit-MS:

?   Help -- prints the available single-character commands.

0   (the digit zero) Transmit a NUL (ASCII 0).

B   Transmit a BREAK signal.

C   Close the connection and return to Kermit-MS prompt level.

F   File the current screen in the screen dump file.

M   Toggle the mode line, i.e. turn it off if it is on & vice versa.

P   Push to DOS; get back to CONNECT by typing EXIT.

Q   Temporarily quit logging the remote session.

R   Resume logging the remote session.

S   Show the status of the connection.

^]  (or whatever you have set the escape character to be)

Typing  the  escape character twice sends one copy of it to the connected

host.

Typing any other character (except the space bar, which is the "null  command")

after  the  escape character will cause Kermit-MS to beep, but will do no harm.

The  escape  character  can  be  changed  to  something  other  than   Control-

Rightbracket by using the SET ESCAPE command.

The Mode Line

When  you  first  issue the CONNECT command, a message (on some systems, an in-

verse video "mode line") will display the most important facts about  the  con-

nection you've just established, so that you can quickly diagnose any problems.

Here's what the IBM PC mode line looks like:

+--------------------------------------------------------------------------+

| Esc-chr:^] help:^]? port:1 speed:9600 parity:odd echo:rem VT102 .... PRN |

+--------------------------------------------------------------------------+

This shows that the escape character is Ctrl-Rightbracket, that you would  type

Ctrl-rightbracket  followed  by question mark (^]?) to get help during CONNECT,

that you are connected on port 1 at 9600 baud with odd parity and remote  echo,

and  that  a  VT102  terminal  is  being emulated.  The four dots represent the

VT102s LEDs (they turn into the digits 1,2,3,4 when "lit") and PRN will show up

if the printer is activated (e.g. by Ctrl-PrintScreen).

The  mode  line occupies the 25th line of those systems that have such a thing,

and is not affected by scrolling.  When emulating a VT102 or  Heath-19,  Kermit

MS-DOS KERMIT                                                          Page 197

will  allow the host to address the 25th line directly using cursor positioning

commands.  If this happens, Kermit will remove its  mode  line  and  relinquish

control  of the 25th line to the host (as if you had typed SET MODE OFF).  When

no terminal is being emulated, the 25th line (if any) is available for  scroll-

ing.  If the mode line is disabled by an application or by the command SET MODE

OFF then the only way to revive Kermit's mode line display is to give the  com-

mand SET MODE ON.

Screen Scroll

On  certain  systems,  Kermit-MS provides several pages of screen memory, which

may be scrolled up and down using keys as shown in Table 11-1.

-------------------------------------------------------------------------------

System    Screen Down  Line Down        Screen Up   Line Up

IBM PC    PgUp         Ctrl-PgUp        PgDn        Ctrl-PgDn

Rainbow   PrevScreen   Ctrl-PrevScreen  NextScreen  Ctrl-NextScreen

HP-150    Prev         Shift-UpArrow    Next        Shift-DownArrow

NEC APC   Uparrow      Ctrl-UpArrow     DownArrow   Ctrl-DownArrow

Table 11-1:   Kermit-MS Screen Scroll Keys

-------------------------------------------------------------------------------

There is no way to assign these functions to other keys.

The IBM PC also allows use of the Home key to get to the  top  of  its  display

memory  and End key to get to the bottom, and the keypad plus (+) key to toggle

the mode line on and off.

The Rainbow uses Shift-Next-Screen to get to the bottom of its display  memory,

but provides no key for moving directly to the top.

Screen Dump

The screen dump feature writes the contents of the screen to a file (KERMIT.SCN

unless another file was selected by the SET  DUMP  command)  when  the  CONNECT

escape-level  command  F is typed.  The screen dump file is appended to on each

successive screen dump, with each screen  separated  by  a  formfeed  (Ctrl-L).

This  feature may be used in conjunction with screen rollback -- a handy way to

recapture screenfuls of laboriously typed-in  text  after  a  remote  host  has

crashed without saving your work.

A  screen  dump  differs  from  a session log in two ways.  First, each desired

screen must be manually filed, and  second,  the  screen  dump  file  has  been

stripped of any escape sequences, whereas the session log records them.

MS-DOS KERMIT                                                          Page 198

Printer Control

During  terminal emulation, a locally attached printer may be controlled in the

normal manner, on most systems.  Pushing the "Print  Screen"  key  (shifted  on

some  systems)  will  cause the current contents of the screen to be printed or

spooled; holding down CTRL while depressing Print Screen will start or stop the

spooling  of  incoming characters to the printer.  On the IBM PC, the mode line

will show PRN when the printer is activated in this manner.  ^P or ^N are  sent

to  the  host during terminal emulation, and do not toggle printing, as they do

when you're talking directly to DOS.

CTRL-Print-Screen can be simulated with the Kermit-MS LOG PRN  and  CLOSE  com-

mands.

Key Redefinitions

Key  redefinitions  are  useful  for  defining  "keystroke macros" of login se-

quences, frequently issued commands, and so forth, and for setting up the  ter-

minal  for use with host resident software designed to work with terminals that

send predefined sequences from their function keys.  For instance, here's a key

redefinition  file for arranging the DEC Rainbow keyboard into the normal ASCII

keyboard layout:

; Make shift-comma send a left angle bracket

set key scan 556

<

; Shift-period sends a right angle bracket

set key scan 558

>

; Accent grave is where ESC is supposed to be

set key scan 96

\33

; Put accent grave on the ESC function key

set key f11

`

Since SET KEY is a two-line command, a special trick is necessary in  order  to

include  it in a single-line macro definition: just use a comma where you would

have typed carriage return after the first line, for instance:

define bar set key scan 261, foo

The CLEAR command may be used to eliminate all key redefinitions.  The SET  KEY

facility may be used provide the PC with a "meta" key for use with editors like

EMACS or TVEDIT that can use "meta characters" as commands.  A meta  key  is  a

shift  key  whose  effect  is to turn on the 8th (parity) bit of the character.

For instance, on the IBM PC  the  scan  codes  produced  by  holding  down  ALT

together with other keys can be determined using SHOW KEY, and then 8-bit ASCII

equivalents with the 8th bit turned on can be defined using  SET  KEY;  if  the

scan code produced by typing ALT-a, i.e. the letter "a" (ASCII 141, octal) with

the ALT key held down, is 2078 (decimal), you would set the META equivalent  to

141+200=341 (octal), or "\341" in octal SET KEY notation:

Kermit-MS>sho key

Press a key: ALT-a

MS-DOS KERMIT                                                          Page 199

Scan Code: 2078

Definition:

Kermit-MS>set key scan 2078

Definition String: \341

Whenever you type ALT-a with this definition in effect, Kermit-MS will transmit

octal 341, rather than 141.

Summary of Kermit-MS Terminal Emulation Features

Table 11-2 shows the terminal emulation options for the systems presently  sup-

ported by Kermit-MS.

-------------------------------------------------------------------------------

System         EscChar   Cabilities   Terminal Service

ACT Apricot      ^]            K      ???

DEC Rainbow      ^]      R   P K D    VT102 firmware

DECmate/DOS      ^]                   ???

Generic DOS      ^]                   Depends on system

Grid Compass     ^]                   ???

HP-110           ^]                   Dumb terminal

HP-150           ^]      R            HP-2623 firmware

IBM PC,XT,AT     ^]      R M P K D    H19,VT52,VT102 emulation

Intel 300        ^]                   ???

NEC APC          ^]      R   P K      VT100, ADM3A firmware

Olivetti M24     ^]      R M P K D    VT100 emulation

Sanyo MBC550     ^]                   ???

Wang PC          ^A                   Wang firmware

TI Pro           ^]        M P K      VT100/Tektronix

Victor 9000      Alt-]                VT100 and/or Tek4010

Zenith Z100      ^]                   Heath-19 emulation

R=Rollback, M=Modeline, P=Printer control, K=Key redefinition, D=screen Dump

Table 11-2:   Kermit-MS Terminal Emulation Options

-------------------------------------------------------------------------------

11.5. Installation of Kermit-MS

If you already have Kermit on your PC, you can use it to obtain new versions of

Kermit-MS when they appear on the central system at your site.  If you  do  not

have  Kermit  or any other reliable file capture facility on your PC, and there

is no one from whom you can borrow a floppy  disk  to  copy  Kermit,  then  you

should  read the following instructions for initially "bootstrapping" Kermit-MS

from a mainframe where it is stored onto your microcomputer.

There are at least three methods of initially getting Kermit-MS onto your PC:

1. Try again to find a copy on diskette.