DBG ASSEMBLY LEVEL DEBUGGER

David F.  Mischler

4389 Chestnut Ridge Road #5

Tonawanda, NY 14150

DEC, DECUS, PDP, and RSX

are trademarks of

Digital Equipment Corporation.

TABLE OF CONTENTS                                     Page i

CHAPTER 1  INTRODUCTION                                  1-1

1.1  What DBG does                               1-2

CHAPTER 2  EXPRESSIONS                                   2-1

2.1  Constants                                   2-1

2.1.1  Numeric Constants                         2-1

2.1.2  ASCII Constants                           2-2

2.1.3  RAD50 Constants                           2-2

2.2  Symbols                                     2-2

2.3  Operators                                   2-3

2.3.1  Character Operators                       2-3

2.3.2  Radix Operators                           2-3

2.3.3  Monadic Operators                         2-3

2.3.4  Dyadic Operators                          2-4

CHAPTER 3  COMMANDS                                      3-1

3.1  CANCEL                                      3-1

3.2  DEFINE                                      3-2

3.3  DEPOSIT                                     3-2

3.4  EVALUATE                                    3-3

3.5  EXAMINE                                     3-3

3.6  EXIT                                        3-4

3.7  GO                                          3-4

3.8  HELP                                        3-4

3.9  LOAD                                        3-5

3.10  QUIT                                       3-5

3.11  SET                                        3-5

3.11.1  SET BREAK                                3-5

3.11.2  SET HISTORY                              3-6

3.11.3  SET LOG                                  3-6

3.11.4  SET MODE                                 3-6

3.11.4.1  SET MODE (NO)LEADING                   3-6

3.11.4.2  SET MODE radix                         3-7

3.11.4.3  SET MODE (NO)SIMULATE                  3-7

3.11.4.4  SET MODE (NO)SINGLE                    3-7

3.11.4.5  SET MODE (NO)SYMBOLIC                  3-8

3.11.4.6  SET MODE (UN)SIGNED                    3-8

3.11.5  SET OUTPUT                               3-8

3.11.5.1  SET OUTPUT (NO)ATTACH                  3-8

3.11.5.2  SET OUTPUT (NO)LOG                     3-8

3.11.5.3  SET OUTPUT (NO)TERM                    3-9

3.11.5.4  SET OUTPUT (NO)VERIFY                  3-9

3.11.6  SET PROFILE                              3-9

3.11.7  SET TRACE                                3-9

3.11.8  SET TYPE                                3-10

3.11.9  SET WATCH                               3-10

3.12  SHOW                                      3-10

3.12.1  SHOW BREAK                              3-11

3.12.2  SHOW HISTORY                            3-11

3.12.3  SHOW MODE                               3-11

3.12.4  SHOW OUTPUT                             3-11

3.12.5  SHOW PROFILE                            3-12

3.12.6  SHOW SYMBOL                             3-12

TABLE OF CONTENTS                                    Page ii

3.12.7  SHOW TRACE                              3-12

3.12.8  SHOW TYPE                               3-12

3.12.9  SHOW WATCH                              3-12

3.13  STEP                                      3-12

3.14  UNDEFINE                                  3-13

CHAPTER 4  INSTALLING DBG                                4-1

4.1  Conditional Assembly                        4-1

4.2  Use of FCSRES                               4-2

CHAPTER 5  TASK BUILDING FOR USE WITH DBG                5-1

5.1  A Simple Example                            5-1

5.2  More About DBGAID                           5-1

CHAPTER 6  USING DBG                                     6-1

6.1  Getting Started                             6-1

6.2  Target Task Priority                        6-1

6.3  Directive Status                            6-2

6.4  MCR DEBUG                                   6-2

6.5  PROFANAL                                    6-2

6.6  SST Vectors                                 6-3

6.7  I/D Space Tasks                             6-3

6.8  Programs with Overlays                      6-3

6.9  Programs with ASTs                          6-3

6.10  Debugging a Task at Another Terminal       6-4

6.11  Function Keys                              6-4

6.12  Serious Problems                           6-4

CHAPTER 7  IMPLEMENTATION                                7-1

7.1  Parsing                                     7-2

7.2  User Output                                 7-3

7.2.1  Output Formatting                         7-3

7.2.2  Error Output                              7-3

7.2.3  Terminal Output                           7-3

7.3  List Handling                               7-3

CHAPTER 1

INTRODUCTION

DBG is a symbolic debugging tool for user mode programs

running under RSX-11M-PLUS (it  won't  work  under  M  until

sending  the  task  region by reference is allowed).  It was

written because DEC failed to include air sickness bags with

ODT.  DBG has the following general features:

o  Symbolic instruction display.

o  Memory and register display and modification.

o  Program   control   via  breakpoints,  tracepoints,

watchpoints and single stepping.

o  Instruction profiling and history.

o  Ability to debug I/D space tasks.

o  Ability to accept commands from a command file, and

write commands and output to a log file.

There  are  many  desirable features which DBG does not

provide, including:

o  DBG is NOT a source language debugger.

o  DBG does NOT know the types of data objects.

o  DBG  can  NOT  differentiate  between I space and D

space symbols in an I/D space task.

o  DBG does NOT provide support for debugging overlaid

programs.

o  DBG can NOT debug supervisor mode libraries.

o  DBG  does  NOT  provide  support  for  dealing with

floating point instructions and registers.

DBG Assembly Level Debugger V1.8                    Page 1-2

Introduction

o  DBG   does   NOT  provide  a  screen-oriented  user

interface.

o  DBG  does NOT come with a known bug list.  I'm sure

there are bugs here, but I don't  know  where  they

are.

o  DBG  does  NOT come with any guarantees, express or

implied.  I have had good luck in using it,  but  I

refuse  any  responsibility  for the results of any

use, misuse, or abuse of this program.

1.1  WHAT DBG DOES

To  debug  a program using DBG, the programmer includes

an extra module in the task image (DBGAID), and  causes  the

task  builder to create a symbol table file.  The programmer

invokes the task in the normal manner,  and  then  initiates

DBG.  DBG prompts the programmer for commands and allows the

programmer  to  examine  and  modify  memory  and  the   CPU

registers, and to control and profile task execution.

Breakpoints  may  be  set  in the task.  Task execution

will stop when a breakpoint is reached;  the instruction  at

that  location will not yet have been executed.  Tracepoints

are a lot like  breakpoints,  but  they  do  not  halt  task

execution.   You  will simply be notified each time the task

reaches the point in question.  Breakpoints and  tracepoints

are implemented by storing BPT instructions at the specified

points, so they can not be used  on  read  only  code  (e.g.

FCSRES).

Watchpoints   are   used   to   detect  erroneous  data

modifications.  The target task will be single stepped,  and

the   watched  location(s)  will  be  examined  after  every

instruction.  The task will  be  halted  whenever  the  data

changes.  Only data word watchpoints are provided (byte data

is not supported).  Because of the implementation  technique

execution with watchpoints is very slow.

When  profiling  is  enabled  a  group  of  counters is

created.  Each counter represents the execution  of  one  or

more  instructions.   The  task is single stepped, and every

time an instruction that is  represented  by  a  counter  is

executed  that  counter  is  incremented;   this  allows the

measurement of the instruction execution frequency within  a

task.   The data acquired by profiling can be used to aid in

the  optimization  of  program   performance.    A   program

(PROFANAL.C)  is provided to convert the instruction counter

data to instructions executed per module (areas separated by

global symbols).

DBG Assembly Level Debugger V1.8                    Page 1-3

Introduction

When  instruction  history  is enabled the task will be

single stepped, and the registers  will  be  stored  in  the

history  buffer  prior to the execution of each instruction.

The  history  buffer  may  be  displayed  as   a   list   of

instructions, registers, or both.

CHAPTER 2

EXPRESSIONS

DBG  allows  all  addresses  and  numeric  values to be

expressions.  A  valid  expression  may  contain  constants,

symbols, and operators.

2.1  CONSTANTS

Constants  may  be numeric values, ASCII characters, or

RAD50 characters.

2.1.1  Numeric Constants

Numeric  constants  may  be  in  one  of  four radices:

binary,  octal,  decimal,  and  hexadecimal.   By   default,

numeric  constants  are  always  processed  in  the  current

default radix (initially octal, and controlled  by  the  SET

MODE  command).   In  addition,  the  default  radix  may be

overridden through the use of the temporary radix  operators

(^B,  ^D,  ^O,  ^X),  or by following a decimal value with a

period.  The first digit  of  a  hexadecimal  constant  must

always be a valid (decimal) digit, even if a temporary radix

operator is used.

The  following  examples  assume  the  default radix is

octal (default).

EXAMPLES:

100     = 100 octal (64 decimal)

100.    = 144 octal (100 decimal)

^B100   = 4 octal (4 decimal)

^X0D    = 15 octal (13 decimal)

^XAF0   = Invalid (does not begin with 0-9)

DBG Assembly Level Debugger V1.8                    Page 2-2

Expressions

2.1.2  ASCII Constants

ASCII  constants  may  be  either one or two characters

long.  Single ASCII characters must be preceded by a  single

quote.   Two  characters  following  a  double quote will be

packed into a 16-bit word;   the  first  character  will  be

placed in the low byte.

EXAMPLES:

'A      = 101 octal (65 decimal)

"AA     = 40501 octal (16705 decimal)

"AB     = 41101 octal (16961 decimal)

2.1.3  RAD50 Constants

RAD50  constants  may  be  up to three characters long.

The characters to be converted to RAD50 must be preceded  by

a  percent  sign, or the RAD50 operator (^R).  If fewer than

three characters are provided, the  rightmost  positions  in

the word will be filled with RAD50 spaces (zeroes).

EXAMPLES:

%ABC    = 3223 octal (1683 decimal)

^RDEF   = 14716 octal (6606 decimal)

%A      = 3100 octal (1600 decimal)

2.2  SYMBOLS

Symbols   are  groups  of  characters  which  represent

numeric values to DBG;  they may be created  by  the  DEFINE

and   LOAD   commands.   Symbols  may  be  composed  of  the

alphanumeric characters (A-Z and 0-9), the dollar  sign  ($)

and period (.).  A symbol which begins with a digit will not

be usable in expressions.  Only the first six characters  in

a  symbol  are significant.  It is useful to note that all C

implementations  (that  I  know  of)  for  RSX  convert   an

underscore (_) to a period (.) for task building purposes.

EXAMPLES:

ABC             OK

$123            OK

FILENAME        OK (but will be truncated to FILENA)

E_ZT            Invalid (contains an underscore)

1QZ             Not recommended (starts with digit)

DBG Assembly Level Debugger V1.8                    Page 2-3

Expressions

In  addition,  the  following PDP-11 registers (and the

directive status word) may be referred to by name:  R0,  R1,

R2,  R3,  R4,  R5,  SP, PC, PS, $DSW.  These special symbols

refer to locations in the DBGAID  module  that  contain  the

register values.

2.3  OPERATORS

DBG  has  several  classes  of  operators:   character,

radix, monadic, and dyadic.   In  addition,  angle  brackets

(<>)  may be used to enclose subexpressions, and control the

order of evaluation.

2.3.1  Character Operators

Character  operators  are used to convert characters to

numeric values.  The single quote (') and double  quote  (")

characters  specify  ASCII  conversion, and the percent sign

(%) and RAD50 operator (^R) specify RAD50  conversion.   For

examples  of  these  operators see the sections on ASCII and

RAD50 constants.

2.3.2  Radix Operators

Radix  operators  are  used  to temporarily control the

conversion  of  digit  strings  to  numeric  values.   These

operators  cause  the  radix to be temporarily set to binary

(^B), octal (^O), decimal (^D) or hexadecimal  (^X).   These

operators  are  handled  by the expression parser as monadic

operators;  they normally  apply  only  to  the  next  digit

string.    If   one   of   these  operators  is  used  on  a

subexpression  (i.e.   one  that  is   enclosed   in   angle

brackets),  then  it  becomes  the  default  radix  for  the

subexpression.  For examples of the use of these  operators,

see the section on numeric constants.

2.3.3  Monadic Operators

Monadic  operators  take  only a single argument.  This

argument  may  be  a  constant,  symbol,  or   subexpression

enclosed in angle brackets.  Monadic operators are processed

left  to  right  before  dyadic  operators.    The   monadic

operators   are   the  complementation  operator  (^C),  the

indirection operator (@), and the negation operator (-).

DBG Assembly Level Debugger V1.8                    Page 2-4

Expressions

The complementation operator inverts all of the bits in

its  argument  (it  takes  the   one's   complement).    The

indirection  operator fetches the data word specified by its

argument  taken  as  an  address.   The  negation   operator

produces the two's complement of its argument.

EXAMPLES:

^C<1>   = 177776 octal (65534 decimal)

@R0     = contents of the word whose address is in

register R0.

-1      = 177777 octal (65536 decimal)

2.3.4  Dyadic Operators

Dyadic  operators  take  two  arguments,  which  may be

constants, symbols, subexpressions,  etc.   These  operators

are  evaluated from left to right.  The dyadic operators are

add (+), subtract (-), multiply (*), divide  (/),  and  (&),

and or (!).

EXAMPLES:

1+2     = 4

4-3     = 1

5*6     = 36 octal (30 decimal)

9./2    = 4 (note period after 9)

5&6     = 4

101!16  = 117 octal (79 decimal)

CHAPTER 3

COMMANDS

DBG  has  a  number  of  commands,  some  of  which are

actually useful.  All command keywords and switches  may  be

shortened  as  much  as you like, but the result must not be

ambiguous (e.g.  "EX" could represent "EXAMINE" or  "EXIT").

You  may  exit  DBG by typing control-Z, EXIT, or QUIT.  Any

command line beginning with a semicolon will be treated as a

comment.

3.1  CANCEL

The  CANCEL  command  is used to eliminate breakpoints,

tracepoints, watchpoints, and  profile  ranges.   A  keyword

specifying  the  type  of  object  to  be eliminated must be

included following the command.  If more than one object  of

a  type  is  to be eliminated then their addresses should be

separated by commas with no intervening blanks.

SYNTAX:

CANCEL[/switches] keyword [address expression(s)]

SWITCHES:

/ALL    Specifies that all objects of the specified

type are to be eliminated.

KEYWORDS:

BREAK   Eliminate the specified breakpoint.

PROFILE Eliminate all profile ranges.

TRACE   Eliminate the specified tracepoint.

WATCH   Eliminate the specified watchpoint.

EXAMPLES:

CANCEL BREAK T$RECV     Eliminate the breakpoint at

location T$RECV

DBG Assembly Level Debugger V1.8                    Page 3-2

Commands

CANCEL/ALL TRACE        Eliminate all tracepoints

CANCEL PROFILE          Eliminate all profile ranges

3.2  DEFINE

The  DEFINE  command  is used to define a symbol with a

specified value.  Both the symbol name  and  value  must  be

specified.

SYNTAX:

DEFINE symbol=value

EXAMPLES:

DEFINE BIT9=512.        Define symbol BIT9 and give

it the value 1000 octal.

DEFINE NEXT=@PC         Define symbol NEXT and give

it the value in the PC.

3.3  DEPOSIT

The  DEPOSIT  command is used to modify the target task

memory and/or registers.  This command  always  defaults  to

depositing  data  words;   it  is  not  affected by the mode

settings.  To  deposit  multiple  data  bytes  or  words  in

sequential  memory locations, separate them with commas.  To

deposit multiple instructions  they  must  be  separated  by

semicolons.   In  I/D  space  tasks  only  instructions  are

deposited into I space.

SYNTAX:

DEPOSIT[/switch] address=value[,value...]

SWITCHES:

/BYTE           Deposit data bytes

/INSTRUCTION    Deposit instructions

/WORD           Deposit data words (default)

EXAMPLES:

DEPOSIT SYMLOW=64.      Deposits 100 octal in word

SYMLOW

DEP/BY TXT='H,'I        Deposits 'H and 'I in the

DBG Assembly Level Debugger V1.8                    Page 3-3

Commands

two bytes at location TXT

DEP/I @PC=MOV (R0)+,-(SP);SOB R1,@PC    Deposits a

loop at @PC

3.4  EVALUATE

The  EVALUATE command is used to evaluate an expression

and display its value in signed decimal,  unsigned  decimal,

binary, octal, hexadecimal, RAD50, and ASCII.

SYNTAX:

EVALUATE expression

EXAMPLE:

EVAL 2*<3+6>

3.5  EXAMINE

The  EXAMINE  command  is  used  to display target task

memory or registers.  This command always remembers the last

address range displayed, and will redisplay it by default if

no address range is  specified.   The  data  type  and  mode

switches are set for the current command only;  the defaults

are set by the SET MODE and SET TYPE commands.   An  address

range  consists  of  two  address expressions separated by a

colon;  the lower address should be specified first.  In I/D

space  tasks  I space can only be displayed as instructions;

all other data types refer to D space.

SYNTAX:

EXAMINE[/switches] range[,range...]

SWITCHES:

/ADDRESS        Displays symbolic addresses

/ASCII          Displays ASCII text

/BINARY         Display in binary

/BYTE           Displays byte data

/DECIMAL        Display in decimal

/HEXADECIMAL    Display in hexadecimal

/INSTRUCTION    Displays instructions

/LEADING        Display leading zeroes

/NOLEADING      Don't display leading zeroes

/NOSYMBOLIC     Don't display symbols

/OCTAL          Display in octal

/RAD50          Displays RAD50 characters

DBG Assembly Level Debugger V1.8                    Page 3-4

Commands

/SIGNED         Display numbers as signed

/SYMBOLIC       Display symbols

/UNSIGNED       Display numbers as unsigned

/WORD           Displays words

EXAMPLES:

EXAM/WORD R0:PS         Displays all registers

EXAM @PC                '' next instruction

EXAM @PC:@PC+20         '' next few instructions

EXAM/ADDR @SP:@SP+100   '' stack symbolically

3.6  EXIT

The  EXIT  command  terminates  the  debugging session.

There are no parameters.  If a log file is open it  will  be

closed before the command takes effect.

3.7  GO

The   GO  command  initiates  or  resumes  target  task

execution.  If an address expression is specified  then  the

PC  will  be  set  to the specified address before execution

begins.

SYNTAX:

GO [address]

EXAMPLES:

GO START        Resume execution at location START

GO              Proceed with execution from @PC

3.8  HELP

The  HELP  command  displays brief help on a variety of

topics.  Type HELP without a topic to get a list of  topics.

HELP does not support subtopics.

DBG Assembly Level Debugger V1.8                    Page 3-5

Commands

3.9  LOAD

The LOAD command is used to add the symbols in a symbol

table file produced by TKB into the symbol table.  The  file

name  must be specified.  If the device, directory, and file

type are not specified then they will default to the current

device (SY:), directory, and type .STB.

SYNTAX:

LOAD filespec

EXAMPLES:

LOAD LB:[3,54]DBG       Load LB0:[3,54]DBG.STB

LOAD PROGRAM            Load PROGRAM.STB

3.10  QUIT

This command is a synonym for the EXIT command.

3.11  SET

The  set command is used to control many DBG parameters

affecting program execution and DBG output.  The SET command

must  be  followed  by  one  or more keywords indicating the

parameters that are to be controlled.

3.11.1  SET BREAK

SET  BREAK is used to set one or more breakpoints.  The

addresses of the instructions which are to  be  breakpointed

must  be  specified.   If  multiple breakpoints are set then

their addresses should be separated by commas.

EXAMPLES:

SET BREAK T$RECV        Breakpoint location T$RECV

SET BREAK MAIN+20       Breakpoint location MAIN+20

DBG Assembly Level Debugger V1.8                    Page 3-6

Commands

3.11.2  SET HISTORY

SET HISTORY is used to control instruction and register

traceback.  The number of instructions of traceback must  be

specified.   The  absolute maximum is 1820 instructions, but

prior memory commitments may reduce this.  Whenever  HISTORY

is in effect all instructions will be single-stepped.

EXAMPLES:

SET HISTORY 1820.       Set history to maximum.

SET HISTORY 0           Turn off history.

3.11.3  SET LOG

SET  LOG  is used to set the name of the log file.  The

name of  the  log  file  must  be  specified.   The  default

filespec, parts of which will be used if the filespec is not

complete, is SY:DEBUG.LOG.

EXAMPLE:

SET LOG EXAMPLE    Sets the log file to EXAMPLE.LOG

3.11.4  SET MODE

The  SET  MODE  command  allows the user to control the

formatting of numeric  values  and  other  output,  and  the

simulation of stepped instructions.

3.11.4.1  SET MODE (NO)LEADING

The  command  controls the display of leading zeroes on

numeric values.  The initial setting is NOLEADING.

EXAMPLES:

SET MODE LEADING        Display leading zeroes

SET MODE NOLEADING      Don't display leading zeroes

DBG Assembly Level Debugger V1.8                    Page 3-7

Commands

3.11.4.2  SET MODE radix

This  group  of  commands  controls  the  default input

radix, as well as the output radix.  The radix may be set to

BINARY, DECIMAL, HEXADECIMAL, or OCTAL.  The initial setting

is OCTAL.

EXAMPLE:

SET MODE DECIMAL        Set radix to decimal

3.11.4.3  SET MODE (NO)SIMULATE

Single stepping is important to some of the things that

DBG does, but stepping with the trace trap  is  horrendously

slow.  For this reason, DBG provides a module that simulates

many common instructions at a  faster  rate  than  with  the

trace  trap.  This module does not operate unless a SET MODE

SIMULATE command is issued, because it depends on  the  MFPS

and  MTPS  instructions  which are fully implemented only on

some of the newer PDP-11  CPUs  (e.g.   KDF11  and  KDJ11s).

Please  note  that  simulation could have strange effects on

system behavior if the MCR DEBUG command is used, or if  the

target task is aborted during simulation.

EXAMPLES:

SET MODE SIMULATE       Turns on simulation

SET MODE NOSIMULATE     Turns off simulation

3.11.4.4  SET MODE (NO)SINGLE

This  command  controls  the  display  of multiple data

items on a single line (as in EXAM/WORD).   I  find  that  I

only  use this mode as a switch on the EXAMINE command.  The

initial setting is NOSINGLE.

EXAMPLE:

SET MODE SINGLE         Display one item per line

DBG Assembly Level Debugger V1.8                    Page 3-8

Commands

3.11.4.5  SET MODE (NO)SYMBOLIC

This  command  controls  the  use  of  symbols  in item

addresses  and  disassembled  instruction   operands   (type

ADDRESS  is  always  displayed  symbolically).   The initial

setting is SYMBOLIC.

EXAMPLES:

SET MODE SYMBOLIC       Display symbols

SET MODE NOSYMBOLIC     Do not display symbols

3.11.4.6  SET MODE (UN)SIGNED

This  command  controls  the display of values with the

high bit set as negative.  The initial setting is UNSIGNED.

EXAMPLES:

SET MODE SIGNED         Display values as signed

SET MODE UNSIGNED       Display values as unsigned

3.11.5  SET OUTPUT

The  SET  OUTPUT  command  allows  the  user to control

terminal, log file, and command file handling.

3.11.5.1  SET OUTPUT (NO)ATTACH

This command controls whether or not DBG's TI:  will be

attached during data  output  (e.g.   EXAMINE,  SHOW).   The

initial setting is ATTACH.

3.11.5.2  SET OUTPUT (NO)LOG

This  command  controls  whether  or  not  commands and

responses (except HELP responses) will be sent  to  the  log

file.  The initial setting is NOLOG.

DBG Assembly Level Debugger V1.8                    Page 3-9

Commands

3.11.5.3  SET OUTPUT (NO)TERM

This  command  controls  whether  or  not  commands and

responses (except  HELP  responses)  will  be  sent  to  the

terminal.  The initial setting is TERM.

3.11.5.4  SET OUTPUT (NO)VERIFY

This command controls whether or not commands read from

a command file are sent to your terminal and  the  log  file

(subject  to LOG and TERM).  The initial setting is NOVERIFY

(i.e.  commands  read  from  a  command  file  are  executed

quietly).

3.11.6  SET PROFILE

The  SET PROFILE command is used to control instruction

profiling;  specifically, it is used  to  create  a  profile

range.   A profile range consists of a set of counters which

reflect execution  within  a  specific  address  range.   To

create  a  profile range an address range, and a compression

factor, must be specified.

The  address  range  specifies  the minimum and maximum

addresses which are to be profiled.  The compression  factor

specifies  the number of instruction words to be represented

by each 32-bit counter.  The default compression  factor  of

one  provides  the best possible profile resolution, but may

consume too  much  memory.   Higher  levels  of  compression

reduce resolution, but also reduce memory requirements.

EXAMPLES:

SET PROF 0:20000        Profiles from 0 to 20000,

1 word per counter

SET PR 20002:40000 2    Profiles from 20002 to 40000

2 words per counter

3.11.7  SET TRACE

SET  TRACE is used to set one or more tracepoints.  The

addresses of the instructions which are to be traced must be

specified.   If  multiple  tracepoints  are  set  then their

addresses should be separated by commas.

EXAMPLES:

DBG Assembly Level Debugger V1.8                   Page 3-10

Commands

SET TRACE LOOP          Trace location LOOP

SET TRACE MAIN+20       Trace location MAIN+20

3.11.8  SET TYPE

SET  TYPE  controls  the  default type displayed by the

EXAMINE command.  The type to be displayed must  follow  SET

TYPE.  The initial setting is INSTRUCTION.

TYPES:

ADDRESS         Displays symbolic addresses

ASCII           Displays ASCII text

BYTE            Displays byte data

INSTRUCTION     Displays instructions

RAD50           Displays RAD50 characters

WORD            Displays words

EXAMPLE:

SET TYPE ADDR   Set the default type to ADDRESS

3.11.9  SET WATCH

SET  WATCH is used to set one or more watchpoints.  The

addresses of the data words which are to be watched must  be

specified.   If  multiple  watchpoints  are  set  then their

addresses should be separated by commas.

EXAMPLES:

SET WATCH ERRCOUNT      Watch the word at ERRCOU

SET WATCH @SP+4         Watch the third word from

the top of the stack

SET WATCH R5            Watch R5

3.12  SHOW

The  SHOW  command  is  used  to  display  various  DBG

parameters.

DBG Assembly Level Debugger V1.8                   Page 3-11

Commands

3.12.1  SHOW BREAK

SHOW  BREAK  will display the symbolic address of every

breakpoint that is currently set.  If no breakpoints are set

then nothing will be displayed.

3.12.2  SHOW HISTORY

SHOW  HISTORY  will display the size or contents of the

history buffer.  If no switches are provided then  the  size

will  be  displayed.   The switches specify what information

should be displayed for each history  entry.   The  optional

count  parameter  specifies  how  many  of  the  most recent

history entries should be displayed.

SYNTAX:

SHOW HISTORY[/switches] [count]

SWITCHES:

/INSTRUCTIONS   Displays instructions

/REGISTERS      Displays registers

EXAMPLES:

SHOW HIST       Displays history buffer size

SH HIST/I/R     '' entire history buffer

SH HIST/R       '' all registers in buffer

SH HIST/I 5     '' last 5 instructions

3.12.3  SHOW MODE

SHOW MODE will display the current mode settings.  Note

that some mode parameters (e.g.  LEADING, SIGNED,  SIMULATE,

SINGLE)  are  only  displayed  when  they  are enabled (i.e.

NOLEADING will never show up).

3.12.4  SHOW OUTPUT

SHOW  OUTPUT  will  display the current output settings

(i.e.  those controlled by the SET OUTPUT command).

DBG Assembly Level Debugger V1.8                   Page 3-12

Commands

3.12.5  SHOW PROFILE

SHOW  PROFILE will display the profile ranges that have

been defined.  For each range, the low and  high  addresses,

and the compression factor, will be displayed.

SHOW  PROFILE/COUNTERS  will display all of the profile

counters.  Each line will contain the  symbolic  address  of

the  lowest  word represented by the counter, and the number

of instructions executed in the words represented.

3.12.6  SHOW SYMBOL

SHOW  SYMBOL  will  display the name and value of every

symbol known to DBG.  The symbols will be displayed in value

order.  If no symbols have been defined then nothing will be

displayed.

3.12.7  SHOW TRACE

SHOW  TRACE  will display the symbolic address of every

tracepoint that is currently set.  If no tracepoints are set

then nothing will be displayed.

3.12.8  SHOW TYPE

SHOW TYPE will display the current default type for the

EXAMINE command.

3.12.9  SHOW WATCH

SHOW  WATCH  will display the symbolic address of every

watchpoint that is currently set.  If no watchpoints are set

then nothing will be displayed.

3.13  STEP

The  STEP  command executes one or more instructions in

the target task.  If a numeric value is  provided  then  the

number of instructions specified will be executed.  The step

count is treated as an unsigned 16-bit value.  If  no  count

is specified then one instruction will be executed.

DBG Assembly Level Debugger V1.8                   Page 3-13

Commands

SYNTAX:

STEP [count]

EXAMPLES:

STEP            Execute the next instruction

STEP 3          Execute the next 3 instructions

3.14  UNDEFINE

The  UNDEFINE  command  is  used  to delete one or more

symbols from the symbol table.  The symbol name(s)  must  be

specified.   If  more  than one symbol is to be deleted then

the names should be separated with commas (no blanks).

SYNTAX:

UNDEFINE symbol[,symbol...]

EXAMPLE:

UNDEF START     Remove START from the symbol table

CHAPTER 4

INSTALLING DBG

All  source code needed for DBG and DBGAID is provided.

To assemble all of the modules, install ...MAC and type "MAC

@DBGASM".   To  generate  listings,  use command file DBGASL

instead of DBGASM.

If  your  machine supports I/D space, then you may task

build DBG by typing "TKB @DBGBLD".  Otherwise, you will have

to  task  build DBG by typing "TKB @DBGOVLY" (overlaid DBG).

I/D space DBG is faster and  has  more  space  for  symbols,

breakpoints,  profile  ranges, etc., so you should use it if

possible.  If your system does not support fast mapping then

you  will  have  to  edit out the "/FM" switches in the task

build files,  and  remove  the  definition  of  X$FMAP  from

DBGPRE.MAC.

Once  you have assembled and task built DBG, you should

copy DBGAID.OBJ to LB:[1,1], and copy DBG.TSK to your LIBUIC

(LB:[3,54]).  If you intend to modify DBG, then DBG.STB will

be useful for debugging it.

4.1  CONDITIONAL ASSEMBLY

In  some  cases  you may wish to modify DBGPRE.MAC (the

source  prefix  file)  to  change  conditionally   assembled

support.   The  following  conditional  assembly symbols are

used:

I$FIS   - Disassembly of FIS instructions

I$FPP   - Disassembly of FPP instructions

I$MTPS  - MFPS/MTPS support for simulation

M$PLUS  - Execution on RSX-11M-PLUS (default)

X$FMAP  - Fast mapping support

DBG Assembly Level Debugger V1.8                    Page 4-2

Installing DBG

4.2  USE OF FCSRES

Using FCSRES or FCSFSL with DBG will reduce the size of

the task.  If the overlaid version is used, then  this  will

also  increase  the  virtual  address  space  available  for

symbols, etc.  If your system does not support  FCSRES  then

you   will   have  to  remove  the  line  specifying  it  in

DBGOVLY.CMD.

CHAPTER 5

TASK BUILDING FOR USE WITH DBG

In  order for DBG to be used with a program, the DBGAID

module must be linked in as a debugging aid when the task is

built.   This module, which is normally located in LB:[1,1],

must always be placed in the root of an overlaid task.

5.1  A SIMPLE EXAMPLE

In  the following example, DBGAID is linked with a task

which consists of a single module, TEST.  The  symbol  table

file  produced  will  contain  all global symbols defined in

TEST.  These may be used to aid  in  debugging  after  being

loaded into the symbol table with a LOAD TEST command.

TKB TEST,,TEST=TEST,LB:[1,1]DBGAID/DA

5.2  MORE ABOUT DBGAID

The  DBGAID module contains several locations which may

be of interest to advanced (i.e.  desperate) users.

Location  $DBG$S  (instruction space) may be patched to

contain an RTI instruction (octal 000002).  This will  cause

DBGAID to begin task execution immediately.  If an error SST

occurs (the MCR DEBUG  command  could  also  be  used)  then

DBGAID  will  attempt  to  communicate  with the appropriate

debugger task.  This allows a production task to be debugged

if  and  when  a  problem  is  discovered  (without  re-task

building).  DBG is built this way (see DBGBLD.CMD).

Location $DBG$T (data space) contains the RAD50 name of

the task which DBGAID will communicate with.  The first word

(3  characters)  of  this  value  may be patched during task

building (or later) so that a name other than "DBGTxx"  will

be  used.  This is especially useful for debugging more than

one task from a single  terminal,  or  using  DBG  to  debug

DBG Assembly Level Debugger V1.8                    Page 5-2

Task Building for use with DBG

itself  (see  DBGBLD.CMD).  The second word of this value is

determined at startup from  the  TT:   unit  number  of  the

target  task's TI:  (e.g.  a task running at TT13:  will set

this word to RAD50 "T13").

Location  $DBG$V  (data  space) contains the 8-word SST

vector  table.   This  table,  which  is  described  in  the

Executive  Reference  Manual  under  the SVDB$ directive, is

used to vector the SSTs that DBG  is  interested  in.   Some

language  run  time systems, particularly DEC's Fortran OTS,

use the TRAP instruction.   Normally  DBGAID  would  receive

this   SST,  but  by  changing  the  TRAP  SST  vector  word

($DBG$V+14) to a zero you can allow Fortan to  handle  TRAPs

in  its  own way.  The other SST vector words may be treated

similarly, except that DBG will cease to be  useful  if  you

zap  the  T-bit  and BPT trap vector.  Please note that your

task must never issue an SVDB$ directive (SVTK$ is OK).

On  RSX-11M-PLUS  V3.0,  DBGAID  requires  562 bytes of

instruction space, and 526 bytes of data space  (1088  bytes

total).

CHAPTER 6

USING DBG

6.1  GETTING STARTED

DBG  only seems to work right when it is installed as a

utility (i.e.  ...DBG), so make sure that this is done prior

to  attempting to use it.  After having built your task with

DBGAID, run it  in  the  normal  manner.   Unless  you  have

patched  DBGAID  location  $DBG$S,  your program will not do

anything much.  Now type DBG followed by the  name  of  your

task  (if  you  leave off the task name, DBG will prompt for

it).  If all is well, then DBG will come up with  a  message

that includes DBG's version and the name of the target task,

and a prompt.  If you created a symbol table  file,  now  is

the  time to LOAD it.  The EXAMINE command will let you look

around (try "EXAM @PC" or "EXAM/W R0:PS"), and  the  GO  and

STEP  commands will let you begin to execute your task.  Set

a breakpoint with SET BREAK (clear it  with  CANCEL  BREAK).

The HELP command just might.

6.2  TARGET TASK PRIORITY

If  you  must  do a lot of stepping (e.g.  profiling or

using watchpoints) and cannot use  simulate  mode  then  you

should  make  sure  that  your  target  task runs at a lower

priority  than  DBG.   This  will  result  in  much   faster

execution.   This  is  because of the way DBG and DBGAID use

the   Wait   for   Significant   Event    directive    while

communicating.   Organizing  your  task  priorities  in this

manner could also improve the speed of other operations.

DBG Assembly Level Debugger V1.8                    Page 6-2

Using DBG

6.3  DIRECTIVE STATUS

The  executive expects the directive status word ($DSW)

to be at location 46.  DBGAID,  however,  would  modify  the

target task directive status if it didn't save it.  For this

reason you should always refer to  $DSW  by  name  for  both

examination  and  modification.   This  will  cause  you  to

reference the target task directive status that  DBGAID  has

squirreled away, and which will be restored before execution

proceeds.  If you "EVAL $DSW", you will notice it is not  at

location 46 (it is stored right after the PS in DBGAID).

6.4  MCR DEBUG

The  MCR DEBUG command is used to force a trace trap in

a task which has  a  debugging  aid.   DBG  recognizes  this

condition,  with  the possible exception of when the program

is being stepped via  the  trace  trap  (try  again  if  DBG

doesn't seem to notice).

If  you  are  stepping, and simulation mode is enabled,

then the MCR DEBUG command will not be recognized  until  an

instruction  that  is  not  simulated  is encountered.  This

might take a little while.  If this is a  real  problem  for

you,   then  you  should  reduce  the  value  of  MAXSIM  in

DBGPRE.MAC.

6.5  PROFANAL

PROFANAL.C is a supplied C program that can help you to

use the data from instruction profiling.  To  use  PROFANAL,

compile  and  task  build  it,  and  run  it  with the STDIN

redirected to a file  containing  the  output  of  the  SHOW

PROFILE/COUNTERS  command.  You can create a file containing

this information by logging this information to a file  (see

SET   OUTPUT  LOG)  and  editing  the  log  file  to  remove

everything but the results of this command.   PROFANAL  will

add  up  all counters between global symbols and display the

sums, as well  as  a  grand  total,  on  STDOUT.   For  many

programs,  this  list  represents the relative time spent in

each module, therefore it indicates which modules,  if  any,

could profit from optimization.

DBG Assembly Level Debugger V1.8                    Page 6-3

Using DBG

6.6  SST VECTORS

As   discussed  in  the  section  on  DBGAID,  DBGAID's

debugging SST vectors begin  at  location  $DBG$V.   If  any

vectors  must  be  disabled  (e.g.   TRAP  for  use with the

Fortran OTS) they may be modified at run time.   The  target

task must not issue an SVDB$ directive.

6.7  I/D SPACE TASKS

DBG  is  perfectly happy debugging I/D space tasks.  It

doesn't know that you may get confused because your data and

instruction  symbols  will  overmap the same areas.  Even if

TKB indicated whether symbols are in I or D space, a lot  of

DBG  would  have  to  change to take advantage of it (symbol

storage, expression handling, symbol selection, etc.).

6.8  PROGRAMS WITH OVERLAYS

It  is  possible  to debug programs with overlays using

DBG, but I don't recommend it.  Overlays make  things  nasty

because  DBG  doesn't  know  where  things are anymore.  You

should make sure that no breakpoints are set in  an  overlay

area  if another overlay might be loaded;  otherwise DBG may

modify your program or data for you.

It  is useful to know that when you set a breakpoint on

a global routine that is actually in an  overlay,  DBG  will

set  the  breakpoint  on  the  autoload  vector  entry point

(assuming you are not loading overlays yourself).  From this

point you can step your way into the overlay...

6.9  PROGRAMS WITH ASTS

Debugging  a  program  with  ASTs  can  be  complicated

because of the multiple execution streams through the  task.

DBGAID protects itself by disabling AST recognition while it

has control.  Before DBGAID relinquishes control it  enables

AST  recognition  only  if  it  was  enabled when DBGAID was

entered.

You  can set a breakpoint within an AST routine and DBG

should work as expected.  If you  are  single  stepping  the

program at task level, an AST can occur between the time the

trace trap occurs and when DBGAID disables AST  recognition.

When  the  AST  routine is entered the PSW will not have the

DBG Assembly Level Debugger V1.8                    Page 6-4

Using DBG

trace bit set, so the AST routine  will  execute  unimpeded.

It may not be obvious that this has occurred.

6.10  DEBUGGING A TASK AT ANOTHER TERMINAL

It  is  sometimes  necessary  to run DBG on a different

terminal from the target tasks TI:  (e.g.  the  target  task

attaches  the  terminal).   This  is  most easily handled by

running the target task in the normal manner, and initiating

DBG  execution  via  the MCR RUN command on another terminal

and specifying the task name as if DBG were running  on  the

same terminal as the target task.  This works because DBGAID

communicates with the DBG task using RCVD$ and SREF$ calls.

EXAMPLE:

RUN $DBG/TASK=DBGT13

6.11  FUNCTION KEYS

DBG  is  very  nice  with  programmable  function keys.

Unfortunately, I didn't implement them.  I have  a  terminal

which  allows  the  user to program the function keys inside

the setup menus.  I typically put  in  things  like  "STEP",

"GO",  "EXAM/W  R0:PS",  etc.   When  I  want  to  implement

something that can't be done in  one  command,  I  have  the

function  key  invoke  a  command  file.   A good example is

OVER.CMD (below, invoked as "@OVER").

;

;   OVER.CMD "steps over" a subroutine call.

;   Note that this is not foolproof, but still useful.

;

SET BREAK @PC+4 ! Set break at return point

GO              ! Execute the subroutine

CAN BREAK @PC   ! Clear the breakpoint

6.12  SERIOUS PROBLEMS

Because  of  the  way  DBGAID  is  implemented,  it  is

possible to write a program that is very difficult to debug.

A  program  that  modifies DBGAID's code and/or data area(s)

could cause you to tear your hair out.   Assuming  that  the

damage  is accidental, it may help to change the ordering of

modules (changing DBGAID's PSECT names could  work),  or  to

insert  guard  areas  before  and/or after DBGAID's code and

data  areas,  and  set  watchpoints  on  the  guard   areas.

DBG Assembly Level Debugger V1.8                    Page 6-5

Using DBG

Understanding  how  DBGAID  works,  along with some creative

thinking, should get you through most anything.

CHAPTER 7

IMPLEMENTATION

DBG  is  a  non-privileged program, and operates on the

target task by mapping an address window to it.  The  DBGAID

module  facilitates the process by saving and restoring task

registers, and by sending mapping context  information,  and

other task areas, by reference.  The DBGAID module is rather

large because of its internal data requirements, but  it  is

smaller  than  ODT.   An  earlier  version  of  this program

operated entirely by send and receive data calls, but I  got

fed up with it because of the poor performance.

DBG  requires  the  extended instruction set (EIS).  On

KDF11 and KDJ11  processors  DBG  can  simulate  instruction

execution  (the  MFPS  and  MTPS instructions are required);

this is faster than stepping with the trace trap.

DBG  uses  CSI,  GCML,  and  FCS.  DBGAID disables ASTs

while it executes, and does not use any LUNs or event flags.

DBG  uses fast mapping when it is available.  DBG and DBGAID

use the following executive directives:

ALUN$                           GTSK$S

CRAW$                           MAP$S

DSAR$S                          QIO$

DTRG$                           QIOW$

ENAR$S                          QIOW$S

EXIT$S                          RCVD$

EXST$S                          RREF$

EXTK$S                          SREF$S

GIN$ (subfunction GI.TSK)       SVDB$

GLUN$                           SDAT$S

GMCR$                           WSIG$S

GMCX$

DBG is composed of the following files:

ASSEMBLE.MAC    Assembles instructions for DEP/INS

CANCEL.MAC      CANCEL command

CONVERT.MAC     Numeric/symbolic conversion

DBGAID.MAC      DBG support module for target task

DBG Assembly Level Debugger V1.8                    Page 7-2

Implementation

DBGPRE.MAC      DBG and DBGAID prefix file

DEBUG.MAC       DBG main program (many global variables)

DEFINE.MAC      DEFINE and UNDEFINE commands

DEPOSIT.MAC     DEPOSIT command

DISASM.MAC      Instruction disassembler

ERROR.MAC       Error message display and definition

EVALUATE.MAC    EVALUATE command

EXAMINE.MAC     EXAMINE command

EXECUTE.MAC     GO and STEP commands, associated stuff

EXPRESS.MAC     Expression parser

HELP.MAC        HELP command and text

INIT.MAC        One-time initialization

LOAD.MAC        LOAD command

MEMORY.MAC      Target task memory routines

MODTBL.MAC      Mode keyword table for SET and EXAMINE

OPCTBL.MAC      Opcode table for ASSEMBLE and DISASM

SET.MAC         SET command

SHOW.MAC        SHOW command

SIM11.MAC       PDP-11 instruction simulator

SYMBOL.MAC      Symbol handling routines

TSKCOM.MAC      Routines to talk to DBGAID

UTILITY.MAC     Utility routines (some bizarre)

PROFANAL.C      C program to analyze profile data

DBGASL.CMD      MAC command file to assemble DBG

DBGASM.CMD      Same as DBGASL but makes no listings

DBGBLD.CMD      Builds DBG for I/D space

DBGMAN.CMD      Produces DBG manual (via RUNOFF)

DBGOVLY.CMD     Builds overlaid DBG

DBGOVLY.ODL     Overlay description for DBGOVLY.CMD

DBGFRONT.RNO    Cover sheet for DBG manual (RUNOFF)

DBGMAN.RNO      DBG manual (RUNOFF)

DBG.DOC         Human readable DBG manual

7.1  PARSING

Parsing  takes  place  at  several  levels  within  the

debugger.   Keywords  are  parsed  by  recognizing  them  in

keyword  tables.   Expressions  are  parsed  by  a recursive

descent expression parser.  The following routines (modules)

are useful for parsing command lines.

U$DCOD (UTILITY)    Looks up a keyword in a keyword table.

The keyword table is composed of KEYWRD

macro calls, and terminated by a zero.

U$FNXT (UTILITY)    Finds the next command line element.

U$FTRM (UTILITY)    Finds a command element terminator.

XPRESS (EXPRESS)    Evaluates an expression.

DBG Assembly Level Debugger V1.8                    Page 7-3

Implementation

7.2  USER OUTPUT

7.2.1  Output Formatting

The following routines (modules) are provided to aid in

the formatting of output.

C$ASCB (CONVERT)    Convert a byte to an ASCII character.

C$ASCI (CONVERT)    Convert a word to 2 ASCII characters.

C$NUMB (CONVERT)    Convert a byte to a numeric value.

C$NUM  (CONVERT)    Convert a word to a numeric value.

C$R50  (CONVERT)    Convert a RAD50 word to ASCII.

C$SYMB (CONVERT)    Convert a word value symbolically.

C$VALU (CONVERT)    Converts a word value according to the

default format (mode and symbol flags).

U$ADDR (UTILITY)    Buffer an address to start a data line.

U$RMTB (UTILITY)    Strip trailing blanks from a string.

7.2.2  Error Output

Routine  ERROR  (module  ERROR) is used to output error

messages.  All of the errors that can be displayed are in  a

table  in this module.  A single text argument can be passed

for inclusion at a user-defined point in an error message.

7.2.3  Terminal Output

Routine  LINOUT (module DEBUG) is used to output a line

of text to the terminal, and the  log  file  if  logging  is

enabled.  Locating the data to be presented in TRMBUF allows

optimal data handling.

Routines TRMATT and TRMDET (also module DEBUG) are used

to attach and  detach  the  terminal  for  potentially  long

output  sequences.   This allows the use of control O on the

output.

7.3  LIST HANDLING

All  lists  managed  by  DBG are stored in value order.

This  simplifies  many  of  the  list  searching  algorithms

although   it   frequently   increases  search  times.   The

following lists are stored in this manner.

BRKLST (DEBUG)      Breakpoint list (elements are P.xxxx)

DBG Assembly Level Debugger V1.8                    Page 7-4

Implementation

PROLST (DEBUG)      Profile range list (elements are PR.xxx)

REGSYM (DEBUG)      Register symbols (elements are S.xxxx)

SYMTBL (DEBUG)      Symbol table list (elements are S.xxxx)

TRCLST (DEBUG)      Tracepoint list (elements are P.xxxx)

WCHLST (DEBUG)      Watchpoint list (elements are P.xxxx)

The following routines are provided for managing symbol

tables.

S$DEFN (SYMBOL)     Define a symbol.

S$LNAM (SYMBOL)     Locate a symbol by name.

S$LVAL (SYMBOL)     Locate a symbol by value.