DECUS C LANGUAGE SYSTEM

Tool Library

Reference Manual

by

Martin Minow

This  document  describes a number of tools, written in C.  Most

of these do not require any knowlege of,  or  interest  in,  the

language C itself.

DECUS Structured Languages SIG

Version of 11-Nov-83

NOTE

This  software  is  made available without any support

whatsoever.    The   person   responsible    for    an

implementation of this system should expect to have to

understand and modify the source code if any  problems

are  encountered  in  implementing  or maintaining the

compiler  or  its   run-time   library.    The   DECUS

'Structured  Languages  Special Interest Group' is the

primary focus for communication among  users  of  this

software.

UNIX  is  a  trademark  of  Bell  Telephone  Laboratories.  RSX,

RSTS/E,  RT11  and  VMS  are  trademarks  of  Digital  Equipment

Corporation.

1.0  General Considerations

With few exceptions, all Decus C tools use a Unix-like method of

passing parameters to the program, allowing for  redirection  of

the  "standard"  input-output  devices.   This section describes

overall tool installation and the parameter passing mechanism.

1.1  Installing the Tools

The  tools  are compiled and linked with the C run-time library.

The resulting executable program (.TSK, .SAV, or  .EXE)  may  be

copied to a convenient directory.  The following conventions are

used:

o  On RSTS/E, the tools are copied to the account named in

the system-wide logical "PUB:".  In order  to  use  the

supplied  command  files  without  editing,  the system

manager  must  have   defined   this   account   as   a

"system-wide logical".

Decus  C  programs  can be run using the RSTS/E consise

command (CCL) facility.  If the system manager does not

wish  to install all programs as CCL commands, the CRUN

program, written in  Basic-Plus  may  be  compiled  and

installed.   You  may  then  run a tool (or any Decus C

program) by a command such as

CRUN Disk:[P,Pn]file argument list

where "Disk[P,Pn]" indicates the disk and account where

the executable program is stored.  CRUN chains  to  the

program,  passing  the  argument list in a common area.

It is very specific to RSTS/E.

o  On  VMS,  the  tool build process writes the executable

image to the user-defined logical  "BIN:".   Tools  are

"installed" by the foreign command definition:

$ TOOLNAME :== $BIN:TOOL.EXE

o  On  RSX, tools may be installed as known images.  There

is  no  equivalent  of  the  CRUN   program   currently

available.

o  On RT11, tools are RUN.  There is no (supported) way to

pass a command line to C programs.

When  specifically  RUN  (rather  than  executed via a command),

(most) tools will prompt for a command line, using a prompt such

as:

Argv>

The  user  types  the command line which is parsed and passed to

the tool's main program.  The specific prompt varies from system

to system.  If known, the name of the tool will be used.

1.2  Argument Lists

If a tool is invoked using an argument list as in:

tool arg1 arg2 "argument three"

the  main  program  will receive the arguments as strings.  Note

the following:

o  Non-quoted arguments are converted to lower-case.  This

is compatible with Vax-11 C.

o  Argument  strings  may  be quoted using "..." or '...'.

Within a quoted string, the other quote  character  may

be used.

o  Within  an  argument,  the backslash '\' character will

quote the next character:

tool don\'t ...

o  As will be described, the '<' and '>' characters may be

used to specify I/O redirection.

1.3  I/O Redirection

Normally,  tools  read  from  and  write  to  the user's command

terminal.  Most tools allow the specific file to  be  redefined,

allowing  tools  to  be  combined  into  larger  entities.   I/O

redirection is done as follows:

o  If  any  (non-quoted)  argument  begins  with  the  '<'

character, the argument will define a file to  be  used

for command input.

o  If  any  (non-quoted)  argument  begins  with  the  '>'

character, the argument will define a file  to  receive

normal  output.   If  the  argument  begins  with '>>',

output will be appended to the file  (if  supported  by

the operating system).

Note  that  I/O  redirection  is  not  supported by the Vax-11 C

run-time library.   A  few  tools,  though  not  all,  implement

redirection as described above.

################################################################################################################