JENS-PETER JENSEN                                  20-JUL-81

Institut f. Nachrichtentechnik

TU Braunschweig                                 Tel:391-2489

Schleinitzstr 23

3300 Braunschweig

RT-11 SUBSYSTEMS

or

How to get up to eight RT-11 Systemdevices (almost) for free

Abstract     ________

This article describes in colloquial form how the MRRT-11 V01 Software

Development  System  (DEC  order  no.   AA-J118A-TC)  can  be  used to

bootstrap a standard RT-11 monitor on the target computer and  how  it

can  be  used to emulate standard RT-11 systemdevices for use by up to

eight front-end processors.  A  Subsystem  Utility  Program  (SUP)  is

introduced, which allows the free exchange of files between subsystems

(or MRRT-11 .DSK-files) and the host processor.

Let's say you own a typical RT-11 development system consisting of:

- PDP11 minicomputer (11/34) or LSI11 microcomputer (11/23)

- Console terminal

- 32KB (and up) of RAM

- Disk subsystem (RL01/02)

- Lineprinter

- Serial I/O ports (e.g. DLV11-J)

- RT-11 V04 operating system with systemprograms

(e.g. MACRO,FORTRAN,EDIT etc.)

and you have to write  application  programs  for  a  SB-11  or  VT103

microcomputer  as target system.  The following will also apply to any

LSI11-based system with the following features:

- LSI11/2 or LSI11/23

- Video terminal

- 32KB RAM

- 2 Serial line units

- Bootstrap ROMs with TU58 bootstrap

RT-11 SUBSYSTEMS                                      PAGE 2

- any other interfaces required by your application

(e.g. IEC-Bus interfaces, A/D or D/A converters etc.)

You could start writing your  programs  for  the  target  system  from

scratch  using  assembly language (without using programmed requests -

SHUDDER!) and transfer them maybe in LDA - format  on  paper  tape  to

your  target system.  To get started you will need a paper tape reader

and punch and a bootstrap loader for the tape.

A better approach would be  to  write  your  programs  in  Stand-alone

FORTRAN  (an option of RT-11 FORTRAN IV V2.5).  But still you have the

problem of how to transfer your programs to the target system.

A good choice in this situation is the  MRRT-11  software  development

system which consists of two parts:

1.  Your standard RT-11 V04 development system

with all its tools like editors, language processors, linker,

debugger etc.  and

2.  The MRRT-11 run-time system.

This is a memory resident version of the RT11SJ monitor

If you have loaded MRRT-11 into your target  system  your  application

program  may  use  nearly  all  the facilities of a proven monitor via

SYSLIB - calls or Programmed Requests.  This includes I/O  via  device

handlers plus a large library of RT-11 oriented routines and programs.

Also you can write your application program in any language  supported

by RT-11.

MRRT-11 offers two ways to load a program into your target processor:

1.  You can save  your  application  program  together  with  the

run-time  system  and  device  handlers on a TU58 tape drive.

Then you carry the tape drive  to  your  target  machine  and

bootstrap the tape via the TU58 bootstrap in ROM...  or

2.  You can down-line load your program.  In order to do  so  you

have  to  establish  a serial link (DLV11 or MXV11-A) between

the host system and  your  target  processor.   The  baudrate

should  be  set to >= 9600 Baud to accomplish reasonably fast

loads.

Next you have to generate a special .DSK-file by running the  BUILD  -

program (included in the MRRT-11 package).  The .DSK-file resides as a

normal RT-11 file  on  a  random  access  device  (preferably  a  fast

hard-disk  system  e.g.  RL01/02) and contains anything a normal RT-11

system device  would  contain:   e.g.   bootstrap  blocks,  directory,

monitor,  handler,  the  application program file, data files and even

empty areas where your application program could put files if  it  was

to generate any.

Now, how is the down-line loading accomplished?

Well, the MRRT-11 system provides  two  essential  programs  for  this

RT-11 SUBSYSTEMS                                      PAGE 3

purpose:

1.  The DLLOAD - program

2.  The XT - handler

Before you can switch on your target processor you have to INSTALL and

LOAD  the  XT - handler and run the DLLOAD - program *).  The DLLOAD -

program is the link between the  XT  -  handler,  which  on  its  part

communicates  with  the  target  computer  via the serial line and the

device handler of the masstorage device  which  holds  the  .DSK-file.

This configuration makes the .DSK-file on the host appear identical to

a TU58 connected directly to the target system.  Therefore the  target

system  can  be bootstrapped at power on by using the TU58 boostrap in

ROM.  For  the  same  reason  the  application  program  will  see  no

differences  in  file I/O (via a DD - handler) between files on a TU58

or files in a .DSK-file on the host.  Figure 1  illustrates  structure

and operation of the DLLOAD - facility.

Figure 1

As  elegant  as  the  MRRT-11  concept  may  seem  there  are  a   few

shortcomings:

1.  The BUILD - process is a little awkward

If your application program contains an error  and  you  have

corrected  it  on  the host system, you have to BUILD a whole

------------

*) DLLOAD can be run as foreground job if you use the FB  or

XM monitor on the host system.  I recommend to do so, if you

don't want to block your main  system,  especially  if  your

application  program  needs access to the .DSK-file after it

has been bootstrapped.

RT-11 SUBSYSTEMS                                      PAGE 4

new  .DSK-file,  instead  of  just  replacing  the  erroneous

program.

2.  You don't have access to the files of the target system

You  have  no  chance  to  return  any  result   files   your

application program might have generated (for example if your

application program is a data aquisition program).

3.  You can't inspect the directory of a .DSK-file

Neither from the host nor from the  target  machine  you  can

inspect  the  contents  of  .DSK-files,  so  that you have to

document their contents very elaborately.

4.  You can't inspect any data or result files

Neither from the host nor from  the  target  system  you  can

inspect  ASCII data files unless you include code for this in

your application program.

5.  You can run only one application program

MRRT-11  provides  a  program  called  APPLP.SAV  which,   if

included   in   the  .DSK-file  together  with  one  or  more

application programs, will query you which  of  the  programs

you  want  to  run.   This  is  fine,  but  on  exit  of  the

application program you only have the choice of rebooting the

MRRT-11 system or rerunning the same program.

6.  MRRT-11 does not include the KMON

This means that all the nice  features  of  RT-11  which  are

initialized  by  keyboard  commands,  like  for  example  SET

options to device handlers, TIME and DATE  setting,  indirect

command files, ASSIGNment of logical device names, RUNning of

programs etc.  are not availlable to the MRRT-11 user.

7.  The MRRT-11 monitor is a single job (SJ) monitor

Some  applications  really  may  require  the  use   of   the

foreground/background monitor, especially if your application

is real-time process control.  In this  case  you  can't  use

MRRT-11 at all.

SO...  Wouldn't it be nice to run a  standard  RT-11  monitor  on  the

target  system?  Then you could RUN arbitrary programs, TYPE your data

files on the  target  processors  terminal,  list  the  DIRECTORY  *),

execute indirect command files, etc.

------------

*) To accomplish some of these tasks, utility programs  like

DUP, DIR or PIP need to be included in the .DSK-file.

RT-11 SUBSYSTEMS                                      PAGE 5

BUT...  Unfortunately your target system doesn't have a system device,

so we can forget this idea, can't we?

NO !  After all, doesn't the HOST/DLLOAD/XT/.DSK-file combination look

like  a system device to the MRRT-11 monitor?  Wouldn't it be possible

to boostrap a normal RT-11 monitor in the  same  way  as  the  MRRT-11

system?

This is indeed the case and if you own the  MRRT-11  software  package

you can try this yourself because it is very easily accomplished.

I will assume, that you have established a serial  communication  link

between the host and the target processor, as described in the MRRT-11

Software User's Guide, and that you have installed the  XT  -  handler

propperly  (see  appendix A of the above manual), so that you are able

to down-line load MRRT-11 systems.

Besides the MRRT-11 system (on device DK:  ) you will need  access  to

the  files  of  a normal RT-11(SJ) system and the handler files TT.SYS

and DD.SYS which should have the same SYSGEN options as  the  monitor.

( presumably on device SY:  )

Then start the BUILD program and answer the questions  like  shown  in

the following dialogue:

RUN BUILD

BUILD V01.00

Several questions will ...

.

.

.

question again.

Do you want to down-line load this system [Y or N] (N) ?  Y

Output file name (MRRT.DSK):  dv#:MRRT.DSK[100]

{ this will generate the .DSK-file on device dv#:   ;   change

this  to  be  the  fastest and largest random access device in

your system.  Make  sure,  that  this  device  has  about  400

contiguous  free blocks.  The final file will contain 100 free

blocks for you to to exercise maybe the EDITor.  }

MRRT monitor file name (MRRT.MRT):  MRRT1.MRT

Boot device handler file name (DD.MRT):  <CR>

Do you want to retain the boot device handler [Y or N] (Y) ?  <CR>

Names of other device handlers:

*<CR>

User application program file name (APPLP.SAV):  <CR>

RT-11 SUBSYSTEMS                                      PAGE 6

Names of other application programs:

*SY:SWAP.SYS,SY:RT11SJ.SYS,SY:DD.SYS,SY:TT.SYS

Names of any data files:

*SY:PIP.SAV,SY:DUP.SAV,SY:DIR.SAV,SY:EDIT.SAV

{ here you may enter 2 additional programs for a total  of  6,

which you would like to try on your new RT-11 system }

?BUILD-I- System size is XXXXX.  bytes

Next type:

LOAD XT:

RUN DLLOAD

{ or FRUN DLLOAD/BUFFER:1000 }

Then DLLOAD will prompt:

How many XT units do you want to assign?  (1-8) 1

*XT0:=dv#:MRRT

Now turn on the target system to start the bootstrap procedure.  After

a while the system should come up with:

MRRT-11 V01.00D

Enter program to be run

*

Type DUP.SAV<CR> in response to the prompt.  After a few  seconds  DUP

should  prompt  with a * .  Type <CR> and DUP should print its version

number.  Now type the following command:

*DD0:A=DD0:RT11SJ.SYS/U

This will copy the bootstrap information from DD.SYS and RT11SJ.SYS to

the  correct  blocks  of  the  .DSK-file.   When the * reappears, type

CTRL-C.  The MRRT-11 system will probably halt the processor.   Switch

off  the  target  machine  and  on  again.  After a while the standard

RT11SJ - monitor should come up with it's monitor  ID  and  a  message

that it can't find the start-up file.

Now you are ready to go.  Type any RT-11 command to  verify  that  the

system  works correctly.  Don't get impatient if some commands seem to

take too long time (especially if you are using a  low  baudrate).   I

would  recommend  to  try  to sucessively increase the baudrate of the

serial link, starting with 9600 Baud, until the system fails to boot.

Now that you have verified  that  it  is  indeed  possible  to  run  a

standard  RT-11  System  on  your target processor using the DLLOAD/XT

RT-11 SUBSYSTEMS                                      PAGE 7

software, only two problems remain to be solved:

1.  It still isn't possible to  copy  files  into  the  .DSK-file

without  recreating  a  whole new .DSK system (which could be

quite embarrassing if  you  have  unique  data  in  your  old

.DSK-file).

2.  You can't copy files from the .DSK-file to the host system in

order to access aquired data.

To  overcome  these  limitations  I  have  written  a  program  called

Subsystem  Utility Program (SUP).  This is a program which runs in the

host's background (while DLLOAD runs in the foreground).  It  is  able

to  create and maintain subsystemfiles (.DSK-files) which are accessed

by the DLLOAD - facility.  SUP can be used to COPY files which  reside

on  a  device  of the host system to any subsystemfile and vice versa.

It is also possible to  COPY  files  from  one  subsytem  to  another.

Further  you  may  delete or rename files in subsystems from the host.

With  a  DIRECTORY  command  you  can  inspect  the   directories   of

.DSK-files.   In  order  to totally eliminate the need for the BUILD -

program and the  awkward  bootstrap  procedure  described  above,  SUP

includes  commands  to create a new subsystemfile of specified size *)

and to copy the bootstrap information from any RT-11  monitor  to  the

bootstrap  blocks  (  V4.0 and V3.0 bootstrap copies supported) of the

subsystemfile.  Besides SUP you won't need any  part  of  the  MRRT-11

system  exept DLLOAD and the XT - handler.  If you insist on using the

original MRRT-11, SUP still provides you  with  the  ability  to  copy

files into and out of .DSK-files and to inspect the directories.

DLLOAD  can  support  up  to  8  different  target  machines  (with  8

.DSK-files).   So,  theoretically  you  could  run 9 independent RT-11

systems (if DLLOAD runs in the foreground,  that  is)  with  only  one

system device.  Figure 2 shows this configuration.

But practically you should consider that for  each  additional  system

DLLOAD  needs  256  words of buffer space in the hosts memory and that

the XT - handler will be larger.  Further subsystemfiles of reasonable

size  (e.g.   1024  blocks)  will  consume  your masstorage very fast.

Finally,  if  several  target  computers   are   requesting   disk-I/O

simultaneously  you  may  get  data  overrun errors in the serial line

units, so that you have to reduce the baud rate, which again makes the

already long load times larger.

Thus I would recommend not to install more  than  one  or  two  target

systems.

But if you accept these limitations, RT-11 Subsystems really may be  a

cheap  solution  to  your  data aquistion - , process control - or any

------------

*) This size ,however, is limited to a maximum of 512 blocks

by  the device size in the DRDEF - MACRO of the XT - and the

DD - handlers.  if you need  larger  subsystemfiles,  change

these  MACRO  -  parameters to the maximum needed size, then

reassemble and link the handlers.

RT-11 SUBSYSTEMS                                      PAGE 8

Figure 2

other problem requiring a front-end processor.

############################################################################################