(RSX) RATFOR DOCUMENTATION

VERSION 20  FEBRUARY, 1980

A.      DESIGN

RATFOR attempts to  retain  the  merits  of  FORTRAN  (universality,

portability,   efficiency),   while   hiding   the   worst   FORTRAN

inadequacies.  RATFOR is FORTRAN except  for  two  aspects.   First,

since  flow  control  is  central  to any program, regardless of the

application, the primary task of RATFOR is to conceal this  part  of

FORTRAN  from  the user by providing decent flow control structures.

Second, it is possible at the same time to  clean  up  many  of  the

"cosmetic"  deficiencies of FORTRAN, thus providing a language which

is easier to read and to write.

Beyond these two aspects, flow control and  cosmetics,  RATFOR  does

nothing  about  other  weaknesses of FORTRAN, except that the STRING

data type is added.  The design philosophy which has determined what

should  be in RATFOR and what should not has been that RATFOR should

not know much FORTRAN.  Further, that RATFOR should  be  as  machine

and operating system independent as possible to facilitate transport

of RATFOR programs from one machine to another.

I strongly recommend "Software Tools" (Addison-Wesley) by  Kernighan

and  Plauger  for  more  detail and not a little philosophy on which

RATFOR is based.

David P. Sykes,

American Management Systems, Inc.

1515 Wilson Blvd.

Arlington, VA 22209

(703) 841-6086

(RSX) RATFOR DOCUMENTATION                                    PAGE 2

VERSION 20  FEBRUARY, 1980

B.      THE RATFOR LANGUAGE

B.1     Statements

(a)     DO Statement

Syntax:

DO limits

RATFOR statement

becomes:

DO label limits

FORTRAN statement(s)

label   CONTINUE

The  "limits"  can  be  any  Control=Initial,Final,Increment  string

accepted  by the local FORTRAN compiler, since it is copied into the

FORTRAN code directly.  RATFOR supplies  the  appropriate  statement

number.   BREAK  and  NEXT  statements  may  be  used within "RATFOR

statement".

(b)     FOR Statement

Syntax:

FOR (initialize; condition; increment)

RATFOR statement

becomes:

initialize FORTRAN statement

label1  IF (.NOT.(condition)) GOTO label2

FORTRAN statement(s)

increment FORTRAN statement

GOTO label1

label2  CONTINUE

The "initialize" statement is executed, then "condition" is  tested,

if  it  is true, "RATFOR statement" is executed, then "increment" is

executed and "condition" is tested again.  If  "condition"  is  ever

false, the loop is broken and control passes to the next statement.

"Initialize" is any single FORTRAN statement, which is  to  be  done

just once before the loop begins;  "increment" is any single FORTRAN

statement to be executed at the end of each loop, before  the  test;

"condition"  is  anything that is legal in a logical IF to the local

FORTRAN compiler.

Any of the three parts may be omitted, although the semicolons  must

remain.   A null condition is treated as always true, so an infinite

(RSX) RATFOR DOCUMENTATION                                    PAGE 3

VERSION 20  FEBRUARY, 1980

loop results.  BREAK and NEXT statements are allowed within  "RATFOR

statement";  NEXT jumps immediately to "increment".

Example:

FOR (I=80; I > 0; I=I-1)

IF (CARD(I) != blank)

BREAK

Note that the IF and the statement  controlled  by  it  (BREAK)  are

considered to be a single RATFOR statement, even though not enclosed

in braces.

(c)     IF Statement

Syntax:

IF (condition)

RATFOR statement1

ELSE

RATFOR statement2

becomes:

IF (.NOT.(condition)) GOTO label1

FORTRAN statement1(s)

GOTO label2

label1  CONTINUE

FORTRAN statement2(s)

label2  CONTINUE

The ELSE part is optional.  The "condition" is anything legal  in  a

logical  IF  to the local FORTRAN compiler.  If "condition" is true,

statement1 is executed, if it is false and there is an ELSE  clause,

statement2 is executed.  IF statements can be chained to provide the

equivalent of the CASE statement:

IF (condition 1)

RATFOR statement1

ELSE IF (condition 2)

RATFOR statement2

ELSE IF (condition 3)

RATFOR statement3

ELSE

RATFOR statement4

(d)     REPEAT Statement

Syntax:

REPEAT

RATFOR statement

(RSX) RATFOR DOCUMENTATION                                    PAGE 4

VERSION 20  FEBRUARY, 1980

UNTIL (condition)

becomes:

label1  CONTINUE

FORTRAN statement(s)

IF (condition) GOTO label1

or

label1  CONTINUE

FORTRAN statement(s)

GOTO label1

The UNTIL part is optional;  if omitted, the loop  is  infinite  and

must  be broken some other way.  The "RATFOR statement" is executed,

then "condition" (if any) is tested;  if true, "RATFOR statement" is

executed  again.   "RATFOR  statement"  is  always executed at least

once.  BREAK and NEXT statements are allowed within the REPEAT, with

NEXT  jumping immediately to the UNTIL (if any, otherwise, repeating

the loop).  "Condition" can be anything legal in a logical IF to the

local FORTRAN compiler.

(e)     RETURN (expression) Statement

Syntax:

RETURN (expression)

becomes:

function = (FORTRAN statement)

return

where "function" is the name of a FORTRAN function subprogram  which

RATFOR remembers from the last "FUNCTION" statement it saw (ie., the

one that  started  the  function  subprogram  in  which  the  RETURN

statement appears).

The parentheses are required (and are passed to the  FORTRAN  code).

This  statement  makes it very clear exactly what value the function

is returning as its value.  "expression" can  be  any  valid  single

FORTRAN  statement  that  the local compiler allows to be equated to

the function's name.  The normal rules for  FORTRAN  RETURNs  apply:

this  statment  is  valid only within a FORTRAN function subprogram;

it may occur several times within a  function;   the  normal  RETURN

without the expression may occur anywhere, as always.

(RSX) RATFOR DOCUMENTATION                                    PAGE 5

VERSION 20  FEBRUARY, 1980

(f)     WHILE Statement

Syntax:

WHILE (condition)

RATFOR statement

becomes:

label1  IF (.NOT.(condition)) GOTO label2

FORTRAN statement(s)

GOTO label1

label2  CONTINUE

As long as  "condition"  is  true  (which  may  be  never),  "RATFOR

statement"  is  repeated;   if it is ever false (including the first

time) the loop is broken  and  execution  continues  with  the  next

statement.   BREAK  and  NEXT  statements are allowed within "RATFOR

statement";   with  NEXT  going   directly   to   the   "condition".

"Condition"  can  be  anything  legal  in  a logical IF to the local

FORTRAN compiler.

(g)     BREAK and NEXT Statements

Syntax:

BREAK           and

NEXT

Allowed in REPEAT, WHILE, DO, and FOR loops to either  proceed  with

the  next  iteration,  or to break out of the loop.  The BREAK jumps

out of only one loop, regardless of how deeply nexted the BREAK is.

B.2     RATFOR Language Features

(a)     Since symbols are clearer than the .EQ., .GT., etc.  used by

FORTRAN, RATFOR allows the use of conventional mathematical symbols,

which it converts into the equivalent FORTRAN.

(RSX) RATFOR DOCUMENTATION                                    PAGE 6

VERSION 20  FEBRUARY, 1980

> or >>                 for .GT.

< or <<                 for .LT.

>=                      FOR .GE.

<=                      for .LE.

==                      for .EQ.

!, ^, or ~              for .NOT.

!=, ^=, ~=, <>, or ><   for .NE.

\ or |                  for .OR.

\\ or ||                for .XOR.

&                       for .AND.

&&                      for .EQV.

(b)     All characters between a pound sign (#) and the end  of  the

line are treated as comments.  This is equivalent to the exclamation

point comment feature in DEC FORTRAN.  The pound sign may  occur  in

the  first  column,  if desired, replacing the FORTRAN "C" in column

one (which is not allowed in RATFOR source code).  In this case, the

entire  line  (with  the "#" replaced by a "C"), will be copied into

the FORTRAN code as a comment (unless the /CO switch is in effect).

(c)     If the input file(s) contain more than  one  program  module

(delimited  by  the FORTRAN "END"), each new program will be started

at the top of a new page in the listing file (if any)  and  will  be

separated by a formfeed in the FORTRAN output file.

(d)     RATFOR source code lines are automatically continued if:

- The statement is obviously incomplete at the end  of

the  line,  as in the middle of the conditional part

of a FOR or IF statement,

- When the line ends with a comma,

- When the line ends with an underline character, (the

underline character is deleted).

(e)     Statements  may  be  placed  anywhere  on  the   line.    By

convention,  blocks of statements are usually indented three spaces.

Multiple  statements  may  appear  on  one  line  if  separated   by

semicolons (no semicolon is required at the end of a statement which

is on a line by itself).  If a statement begins with an all  numeric

token,  it  is assumed to be a FORTRAN statement label and is placed

in colums 1-5 of the FORTRAN output.

(f)     RATFOR source  lines  may  be  up  to  90  characters  long.

FORTRAN  code  is  generated  with  a  maximum  line  length  of  72

characters.  Continuation lines are created if necessary.

(g)     Any group of one or more statements (within a single program

and  a single file) can be enclosed in braces or square brackets and

then treated as a single RATFOR statement and used anywhere a single

statement  may  be used.  Braces ({ }) and square brackets ([ ]) are

synonymous.

Example:

(RSX) RATFOR DOCUMENTATION                                    PAGE 7

VERSION 20  FEBRUARY, 1980

IF (I == J) [

J=1

I=2

]

causes both statements to be executed if I equals J, whereas

IF (I == J)

J=1

I=2

causes only J=1 to be executed conditionally;  I=2 would be executed

in  any  case  because  only  one  statement  or  bracketed group of

statements is controlled by the IF.  "RATFOR statement"  in  any  of

the examples above could be a bracketed group of several statements,

or a single un-bracketed statement.

(h)     Strings in double quotes (e.g.  "foo")  are  converted  into

the  equivalent  Hollerith  string  (e.g.  3hfoo), since that is the

only legal form in ANSI standard  FORTRAN.   Single  quotes  strings

(e.g.   'A')  are passed to the FORTRAN code unchanged.  Both single

and double quoted strings are treated as single tokens;  this  means

that  symbolic  constants within them are not processed and that the

string must be less then "maxtok" (currently 70) characters long.

Two apostrophes (single quotes) next to each other are passed to the

FORTRAN  code  as  a single apostrophe.  Therefore, in direct access

READ/WRITE  statements,   use   two   apostrophes   together   (e.g.

READ(1''23)).

(i)     Imbedded blanks  are  significant  to  RATFOR  because  they

separate  tokens  and  are  used  to  break  input  code  down  into

recognizable pieces (along with special  characters).   Leading  and

trailing blanks are not significant;  in particular, the indentation

of blocks of code is meaningless to the pre-processor;   it  is  for

visual  information  only.   The pre-processor does no formatting of

the RATFOR source code on the  RATFOR  listing;   you  must  provide

indentation and other formatting in the source code file the way you

want it to appear on the listing.

(RSX) RATFOR DOCUMENTATION                                    PAGE 8

VERSION 20  FEBRUARY, 1980

C.      PRE-PROCESSOR FEATURES

Keywords for pre-processor features must,  like  language  keywords,

appear at the beginning of a line, or after a semicolon.

C.1     INCLUDEs

When a line of the form:

INCLUDE RSXfilespecification

is encountered in the RATFOR source  code,  the  specified  file  is

opened and the input of source code continues from the start of that

file until its end is reached, at which time input  reverts  to  the

next  line  of  the  original  file.   The  INCLUDEd file may itself

contain INCLUDEs;  they can be nested three deep.   The  format  for

the file specification is the standard for RSX, optionally including

UIC and device specifications, (the defaults are the user's UIC  and

SY:);   an extension (the default is RAT), and a version (default is

the latest version).

If the symbol "defaultopen" is defined (in RATRSX.RAT), and  if  the

include  file  is not found based on the file specification as given

in  the  INCLUDE  statement,  then   the   defaults   specified   in

"defaultopen"  are  prefixed to the specification in the INCLUDE and

another attempt to find the file is made.  However, this occurs only

if  the  original  specification  in  the INCLUDE contains neither a

device  nor  a  UIC.   Note  that  the  default  string  defined  in

"defaultopen"   MUST   be  in  quotes;   it  can  contain  a  device

specification, a UIC specification, or both.

Example:

DEFINE (DEFAULTOPEN="LB:[1,1]")

INCLUDE FOO

causes an attempt to open using  "FOO.RAT".

If that fails, another attempt to open using   "LB:[1,1]FOO.RAT"

is made.

If you do not want the contents of the INCLUDEd file to  be  printed

in the listing file, a "no-list" switch is allowed in the INCLUDE:

INCLUDE/NL RSXfilespecification

Any INCLUDEs within the specified file will not  be  listed  either,

regardless of the switch settings of their INCLUDE lines.  (Only the

slash is actually checked for.)

Statements  and  lines  cannot  be  split  across  file  boundaries;

(RSX) RATFOR DOCUMENTATION                                    PAGE 9

VERSION 20  FEBRUARY, 1980

otherwise,  the  INCLUDEd code is processed just like it had been in

the original input file.

In order to set the INCLUDEd code apart from the normal code,  lines

of  code  being  taken  from an INCLUDEd file are preceded by one or

more asterisks on the listing (if they  are  listed).   First  level

INCLUDEs  are  preceded by one asterisk, INCLUDEs within INCLUDEs by

two asterisks, and third level INCLUDEd code by three asterisks.

The INCLUDE feature is especially handy where COMMON blocks  are  to

be  used  by  several different programs, since only one copy of the

COMMON block and all associated declarations  need  be  kept  up  to

date,  but  subroutines, definitions, or any other code can be in an

INCLUDEd file.  They are also very useful  for  files  containing  a

standard  set  of DEFINEs, which can then be INCLUDEd in a series of

related programs easily.

C.2     DEFINE (Symbolic Constants)

The  pre-processor  contains   a   simple   text-replacement   macro

capability  that  allows  the  user  to  create  symbolic  constants

(similar to FORTRAN variables, but not limited to  six  characters),

and  specify a "meaning" or definition for each, which can remain in

effect across several programs.  When the RATFOR code  is  converted

into  FORTRAN  code,  all  occurrences  of symbolic constants in the

RATFOR code are replaced by their defined equivalents in the FORTRAN

code.   The  definition itself may, in turn, be a symbolic constant.

DEFINEs may not appear in the middle of other RATFOR statements.

Example:

if      DEFINE (character=BYTE)

and     DEFINE (maxlin=80)

then    character LINE (maxline)

is converted into the equivalent FORTRAN code:

BYTE LINE (80)

The symbolic constant ("character" in the first line above) and  its

definition ("BYTE") must be enclosed in parentheses and separated by

either a comma or an equals sign.

Currently, the symbolic and its definition can  each  be  up  to  70

characters  long  (their length is DEFINEd in the pre-processor) but

the entire DEFINE statement must fit on one line.

The definition of  a  symbolic  constant  can  be  another  symbolic

constant.   Once  a  symbolic  constant  is  defined,  it  cannot be

redefined within the files  processed  by  a  single  command  line.

Symbolic  constants  must  be single, unique alphanumberic character

(RSX) RATFOR DOCUMENTATION                                   PAGE 10

VERSION 20  FEBRUARY, 1980

strings beginning with a letter;  no special  characters  or  blanks

allowed.   Definitions  can  be  any  character  string less than 70

characters long but may not contain dollar signs.  If the definition

contains parentheses, they must be balanced pairs.

The definition, either in whole or in part, can consist of a  simple

integer  mathematical  relationship  between  one  or  more symbolic

constants and/or integers enclosed  in  less-then  and  greater-than

signs  (<  >).   Addition, subtraction, multiplication, and division

are allowed.  Evaluation of  the  expression  is  strictly  left  to

right;   there  is  no  precedence  of operators.  If the expression

contains previously defined symbolic constants, they are replaced by

their  definitions  prior to mathematical evaluation.  The result of

the <MATH> part is included in the definition in place of the <MATH>

expression  (it  may  be  the  entire  definition).  Numbers and the

definitions of symbolic  constants  must  be  positive  or  negative

integers.

if      DEFINE (foo=7)

then    DEFINE (blatz=<foo+3*foo-1>)

makes "blatz" equal "69" and is equivalent to

DEFINE (blatz=69)

and

DEFINE (dumb=VERSION <foo+1>)

makes "dumb" equal "VERSION 8"

If the /SC switch is specified in the command line, a table  of  all

current  (as  of  the  end  of the processing for that command line)

symbolic constants and their definitions is included in the  listing

file.

DEFINEs may occur anywhere in a RATFOR program.  Definitions  remain

in  effect  for  the  remainder of processing of the command line in

which the file containing them occurred.  Normally, all  definitions

are cleared before processing the next command line, but this can be

prevented by specifying  the  /RE  (RETAIN)  switch  in  the  second

command  line.  A symbolic constant cannot be DEFINEd twice in files

in the same command line (or any INCLUDEd files used by that command

line), or a subsequent command line if the /RE switch is used, since

that would consitute redefinition, which is  not  allowed.   DEFINEs

and MACROs can be, and often are, in INCLUDEd files.

C.3     MACROs

The MACRO facility is exactly like the DEFINE facility (in fact, the

keywords  are  really  synonymous),  except  that  MACROs  allow the

(RSX) RATFOR DOCUMENTATION                                   PAGE 11

VERSION 20  FEBRUARY, 1980

passing of a single argument to be included in the definition at one

or  more  places  at  pre-processing  time.   The  place(s)  in  the

definition where the argument is to be inserted are specified in the

definition  with  a special flag character (a dollar sign).  A MACRO

is distinguished from a simple DEFINE by the presence  of  at  least

one  dollar  sign  in  the definition.  MACROs may not appear in the

middle of other RATFOR statements, but may appear anywhere  else  in

the program.

Example:

MACRO (foo,$=$+1)

then    foo(I)          in the RATFOR code becomes

I=I+1           in the FORTRAN code.

because each "$" in the definition is replaced by the

argument "I".

if      DEFINE (star=46)

and     MACRO (flag=ECOL($)=star)

then    flag(7)                         in the RATFOR code

becomes ECOL(7)=46                      in the FORTRAN code.

C.4     IFDEF / IFNOTDEF / ENDIFDEF (Conditional processing)

Sections of RATFOR code (one  or  more  lines)  can  be  selectively

processed into FORTRAN or ignored, depending upon the current define

status of a specific symbolic  constant.   When  "IFDEF(symbol)"  is

encountered  in  the  RATFOR  source code, a check is made to see if

"symbol" has previously appeared in a DEFINE or MACRO statement;  if

it  has,  the  source code up to the balancing ENDIFDEF statement is

processed;  if not, the source code is skipped until  the  balancing

ENDIFDEF  is  found.  The "IFNOTDEF(symbol)" is similar, except that

the RATFOR source code up to the balancing ENDIFDEF is processed  if

"symbol" has NOT been previously defined.

The symbolic constant can be given a null definition, if it is being

defined  only  for  use  with  the  IFDEF/IFNOTDEF  statements (e.g.

DEFINE(foo=) is sufficient).  IFDEFs (and IFNOTDEFs) can be  nested;

if  an  outer conditional is unsatisfied, all inner conditionals are

skipped,  just  like  all  other   code   within   the   unsatisfied

conditional.

(RSX) RATFOR DOCUMENTATION                                   PAGE 12

VERSION 20  FEBRUARY, 1980

Example:

DEFINE (foo=3)

DEFINE (foo1=)

IFDEF (foo)

I=9

IFNOTDEF (foo1)

I=72

ENDIFDEF

ENDIFDEF

I=9 is processed;  I=72 is ignored.  Because the IFNOTDEF is  nested

within  the  IFDEF,  if  "foo"  were  not defined, I=72 would not be

processed regardless of the status of "foo1".

Undefined conditional code (that  not  processed  into  FORTRAN)  is

normally  printed  in  the  RATFOR  source listing, but will have no

source code line numbers on the lefthand side of the page.   It  can

be  deleted  from the listing entirely if a /NOIF switch is included

in the command line.

C.5     Literal Lines (%)

Occasionally, a line of code must be included in a program which  is

modified in some undesirable way by the pre-processor.  If a percent

sign (%) occurs in column 1 of the  RATFOR  source  code  line,  the

entire  line,  except  for  the  percent sign, will be passed to the

FORTRAN code without  ANY  modification  whatsoever.   Don't  forget

enough leading blanks to put the start of the resulting FORTRAN line

at or past column seven, as required by FORTRAN.

C.6     Debug Lines (?)

If a question mark occurs in column one  of  a  RATFOR  source  code

line, it is considered to be a debug line and will be processed into

FORTRAN (minus the question mark) only if the /DE (DEBUG) switch was

specified in the command line.

Multiple levels of debug statements can  be  specified  by  a  digit

(1-9) in the second column (after the "?").  Debug lines whose level

is equal to or greater than the level specified in the /DE:n  switch

are  processed,  but lines with a lower level are not processed into

FORTRAN.  Lines with no level specified (blank in  column  two)  are

always  processed if the /DE switch is specified.  A /DE switch with

no value causes all debug lines to be processed.

Example:

?5      WRITE (5,1)

?5      1  FORMAT (" DEBUG LINE")

(RSX) RATFOR DOCUMENTATION                                   PAGE 13

VERSION 20  FEBRUARY, 1980

would be processed by specifying /DE or /DE:5 or /DE:4, etc,

but not be processed by specifying /DE:6 or /DE:9.

C.7     STRINGs

Since character processing frequently requires the  use  of  strings

(collections  of  characters of arbitrary length), the pre-processor

adds the STRING data type to FORTRAN.  In FORTRAN, a STRING  becomes

a  character  array with one character per element, plus one element

for the terminator (end-of-string character,  in  DEC-land,  a  zero

byte).

Example:

STRING FOO "BLATZ"

becomes

character FOO(6)

DATA FOO /1hB, 1hL, 1hA, 1hT, 1hZ, eos/

The string must be delimited by double quotes, single quotes are not

allowed.   Single  quotes,  may,  however, appear within the string.

Null strings are allowed, e.g.  STRING FOO "".

Note that the STRING function requires that the  symbolic  constants

"character"  and  "eos"  be defined (usually as "BYTE" and "0") when

the STRING keyword is first encountered;  otherwise, "character" and

"eos"  will  be  passed  to  the  FORTRAN  code  as is and upset the

compiler.  (The standard define file, DEFIN.RAT does this.)

Since ANSI standard FORTRAN requires that all DATA  statements  must

be  grouped  together and placed in the FORTRAN code after all other

specification statements,  but  before  any  executable  statements,

STRING  statements must be grouped together and appear in the RATFOR

source code after all other specification statements but before  any

DATA  statements.   The pre-processor holds the DATA statement parts

until it has output the "character" statement parts for all STRINGs,

then  outputs  the  DATA  statements  as a group.  The pre-processor

really dosen't care if STRING statements are grouped in this  manner

or  not.  DEC FORTRAN IV doesn't care either, but FORTRAN 4 PLUS and

the ANSI standard do.

Currently, there is a limit of 20  string  specification  statements

with  a  total of 300 characters (combined total of the string names

and the characters in  the  strings)  in  any  one  program  module.

(DEFINEd as 'numstr' and 'maxstr' in RATDEF.RAT).

(RSX) RATFOR DOCUMENTATION                                   PAGE 14

VERSION 20  FEBRUARY, 1980

D.      PRE-PROCESSOR USE UNDER RSX

D.1     FORTRAN is normally installed with the task name "...RAT" so

that in can be run as an MCR task or RUN independently:

MCR>RAT command line

or

MCR>RUN RATFOR

RAT>command line

In the first example, the specified cmmand line  will  be  processed

and the MCR will return;  in the second example, RAT will return for

another command line until given a CTRL/Z (EOF).

Regardless of the method of running RATFOR, an indirect command file

specification  may  be given by preceeding it with the usual at sign

(@).  Only one level of indirect  command  files  is  allowed.   The

default extension is CMD.

The RATFOR output file (default  extension:   FTN),  RATFOR  listing

file  (default  extension:   LST),  compiler output file for the /GO

switch (default extension:  OBJ), and up to six input files (default

extension:   RAT)  may  be  specified in a single command line.  The

default device for all files is currently SY0:.  The default UIC  is

the user's current UIC.  If an explicit version number is specified,

an explicit extension is required.  The FORTRAN output file and  the

listing  file  are  optional, but one must be present.  The compiler

OBJ output file specification is optional.

If only a single file name is specified on a command  line  with  no

equals  sign  (with switches, if desired), that name is taken as the

name for the output file, listing file, compiler output file (if /GO

is used), and (only) input file.

Example:

MCR>RAT FOO/SP          is equivalent to

MCR>RAT FOO,FOO=FOO/LC          or

MCR>RAT FOO,FOO,FOO=FOO/GO

since this is the most common form of the command line.

Everything from a "!" to the end of the  command  line  is  ignored,

thus  trailing  comments can be used either when the command line is

input by the operator  or  from  a  indirect  command  file.   Lines

beginning  with a ";" or a "!" in column one cause the whole line to

be ignored, but are allowed in indirect command files only.

When more then one input file is specified,  they  are  concatinated

together  into  single  FORTRAN  output  and  listing files, and all

command line switches  and  symbolic  constant  definitions  are  in

effect for the entire command line.

A single RATFOR input file may also contain more  than  one  program

(RSX) RATFOR DOCUMENTATION                                   PAGE 15

VERSION 20  FEBRUARY, 1980

module.   The  end of the RATFOR program module is identified by the

FORTRAN "END" statement, and each will  start  a  new  page  in  the

listing file.

The RATFOR source listing contains the RATFOR line number, which  is

consecutive  for  all programs (and files) in a single command line.

It also includes (in parentheses, on  the  extreme  left)  the  line

number   of  the  FORTRAN  code  being  generated  for  that  RATFOR

statement.  These numbers  should  match  (approximately)  the  line

numbers the compiler will come up with for the FORTRAN code which is

being generated by that RATFOR line.  These numbers are very  useful

for  debugging,  since  compiler  and runtime errors are reported by

line numbers.  Almost always, you can find the offending code in the

RATFOR  listing by looking up the matching (or almost matching) line

number.  You should almost never have to look at a compiler listing.

These  numbers  should  be  taken  with  a  grain  of   salt;    the

ralationship  of  RATFOR  to  FORTRAN  code  is  neither  simple nor

straighforward.   Many  RATFOR  statements  cause  several   FORTRAN

statements to be generated;  some generate no FORTRAN at all;  still

others generate some FORTRAN  now,  some  later.   In  general,  the

numbers listed correspond to the first significant FORTRAN statement

generated by  that  RATFOR  statement.   For  example,  an  ELSE  IF

generally   generates   a   GOTO   and  a  CONTINUE  before  the  IF

corresponding to the RATFOR IF.  In such cases, the  listed  FORTRAN

statement number is for the IF.  Complex RATFOR constructs may cause

some of the listed FORTRAN numbers to be off by  1  or  2,  but  the

numbers   always  resyncronize  after  a  couple  of  simple  RATFOR

statements (that can be passed directly to the FORTRAN output).  The

FORTRAN  line  numbers  are  correct  for  either  DEC FORTRAN IV or

FORTRAN 4 PLUS (which differ in the way they number IF's), depending

on the status of symbol "decf4p" (in RATRSX.RAT).

D.2     Command Line Switches

The following command line switches are  available  to  control  the

actions  of  the  pre-processor.   They may be placed after any file

specification on the command line and apply to  the  entire  command

line.   Where  appropriate, a switch can be negated by /NOsw or /-sw

in the RSX style, however, all of the default  switch  settings  are

NO.

/DE:n   debug

Causes all lines beginning with a question  mark  in

column  one  to  be processed into FORTRAN code;  by

default such lines are ignored.  If n is  specified,

only  debug  lines  with an equal or higher value in

column two will be processed.

/FO     FORTRAN

Causes the generated FORTRAN code to be included  at

(RSX) RATFOR DOCUMENTATION                                   PAGE 16

VERSION 20  FEBRUARY, 1980

the  end of the listing file (if one was specified).

Each program module starts a new page.  Line numbers

on  the  listing  agree  with  DEC  FORTRAN  or  F4P

compiler line numbers.  By default, the FORTRAN code

is  not  listed.   For this switch to work, you must

also  specify  an  FTN  output  file  or  else   the

pre-processor  must have been built with "openclose"

support, so it can create a scratch output file.

/CO     compress

Causes   the   FORTRAN   code   generated   by   the

pre-processor  to  be  compressed  for faster I/O by

eliminating all comments and unnecessary  blanks  in

the  generated FORTRAN code.  This makes the FORTRAN

essentially unreadable and is not recommended except

for well debugged programs.  The default is /NOCO.

/GO     go

Causes the FORTRAN or F4P  compiler  to  be  spawned

with  a command line of the form "FOO=FOO/SW", where

FOO is from the RATFOR command line, and  /SW  is  a

string  of  switches  for  the compiler which can be

defined in the module SPAWNF.  Thus,  if  you  don't

want  a fancy compile, this allows RATFOR to produce

a finished object module.  This switch is  available

only  of RSX11M V3.2 systems which support the SPAWN

directive and the symbolic constant  "SPAWNIT"  must

be  defined  (in  RATRSX.RAT).  RATFOR does not wait

for the compiler to finish, but goes on to the  next

command  line  at  once.   If  a  third  output file

specification is specified, it is used on the output

side  of  the  compiler command line.  If not given,

the device, UIC, and name from the first output file

specification  (for  the  FTN file) are used for the

OBJ file as well.  The compiler input file is always

the FTN file created by RATFOR.

/HE     help

Causes a  brief  description  of  the  command  line

switches  to be printed on the user's terminal.  The

rest of the command line, if any, is ignored.

/IF     ifdef non-listing

Causes  RATFOR  source   code   within   unsatisfied

conditionals   (IFDEFs   that  are  not  defined  or

IFNOTDEFs that are defined) NOT to be printed in the

listing  file, except for the IFDEF or IFNOTDEF line

itself.  The default  is  /NOIF  (print  unsatisfied

conditionals).

/LC     lower case

Causes RATFOR generated FORTRAN code to be output in

lower case characters (code that is just transferred

(RSX) RATFOR DOCUMENTATION                                   PAGE 17

VERSION 20  FEBRUARY, 1980

from the RATFOR source is in upper case)  making  it

easy  to  identify.   Also, RATFOR comments are also

converted to lower case when printed on  the  RATFOR

listing,  making it easier to separate comments from

code.  The default is /NOLC.

/RE     retain

Causes the symbolic constant definitions (if any) in

effect  at  the  end of the previous command line to

remain in effect for the current command  line.   By

default, any previous definitions are removed at the

start of each command line.

/SC     symbolic constant table

Causes RATFOR to  print  a  table  of  all  symbolic

constants   defined   in  a  program  (or  group  of

programs) at the  end  of  the  RATFOR  source  code

listing  (only if a listing file is specified in the

command line).  The table shows each symbol and  its

character string definition.

/SP     spool

Causes the listing file, if one is specified, to  be

spooled  to  the  line  printer.   The /SP switch is

useful only if the listing is  directed  to  a  disk

file;   if  directed  to  LP:   the operating system

convention  applies  (spooled  for  RSX11/D,  direct

output  for  RSX11/M  v3.1, failure for v3.2).  This

switch works  only  if  the  symbol  "openclose"  is

defined in the pre-processor.

/VE     version

Causes the RATFOR pre-processor to print its current

version  identification on the user's terminal.  The

rest of the command line, if any, is ignored.

(RSX) RATFOR DOCUMENTATION                                   PAGE 18

VERSION 20  FEBRUARY, 1980

E.      CONVENTIONS AND SUGGESTIONS

While not required for proper pre-processor operation, I have  found

the following helpful:

Start   pre-processor   features   (INCLUDEs,   IFDEFs,   IFNOTDEFs,

ENDIFDEFs,  DEFINEs,  and MACROs) in column one of the RATFOR source

code lines.  Start the first level of code in column four and indent

each logical level three more spaces.

If your line printer, editor, and terminal  will  print  lower  case

characters,  make  all  symbolic  constants lower case in the RATFOR

source code to make them easy to distinguish  from  variables.   The

case  of symbolic constants is not significant to the pre-processor,

but lower case makes them easy to locate in the listing.

If your line printer and FORTRAN compiler  will  handle  lower  case

characters,  use  the  /LC switch, having the RATFOR-created FORTRAN

code in lower case makes it easier to read and debug the FORTRAN  if

you  have to.  Also, having comments printed in lower case makes the

RATFOR listing easier to read.

On the listing, line numbering of pre-processor feature  lines  will

not be exactly correct if the keywords (IFDEF, INCLUDE, DEFINE, etc)

are not in upper case.  However, processing of such keywords will be

correct regardless of case.

Symbols will be added to the symbolic constant table  somewhat  more

quickly  if DEFINEs and MACROs are encountered in alphabetical order

by symbolic name.

A "standard" set of DEFINEs is contained in the file  DEFIN.RAT  and

is  useful  for  most  RATFOR  programs as well as the pre-processor

itself.  A shorter  version,  without  definitions  for  lower  case

characters  is  in DEFINS.RAT.  Such symbolic constants as EOS, YES,

NO,  CHARACTER,  and  STDOUT  and   macros   as   INCREMENT($)   and

DECREMENT($) should be standardized for all your RATFOR programs.

If you like the IF...THEN, ENDIF, ENDDO, etc.   convention  used  by

some other structured programming languages better then the brackets

used by RATFOR, you can DEFINE  such  keywords  as  open  and  close

brackets;  e.g.  DEFINE(THEN=[), DEFINE(ENDIF=]).

Leave the blank out of the FORTRAN DEFINE FILE statement, so  it  is

not confused with the RATFOR DEFINE;  FORTRAN doesn't care.

You need to be  careful  of  some  DATA  statements.   For  example,

DATA foo /"""/  or  DATA foo /"#"/ will both be misunderstood by the

pre-processor.  In such cases, you can protect the lines with  %  or

use  the  decimal  value  of  the  character  instead of the literal

character.  Note that RATFOR does not allow the  DEC  convention  of

double  quotes for Octal constants unless the line is protected with

a %.

(RSX) RATFOR DOCUMENTATION                                   PAGE 19

VERSION 20  FEBRUARY, 1980

The pre-processor uses a homebrew  command  line  interpreter  whose

calling  sequence  and structure are based on the RT11 routine ICSI.

Command line syntax and style are identical to the  RSX  conventions

with  the  following  limitations on switch arguments:  (1) Limit of

one argument per switch, (2) Character arguments  are  not  allowed,

(3)  Arguments  are always decimal numbers;  neither "." nor "#" are

allowed to specify radix.

The pre-processor works under F4P  on  processors  without  floating

point  processors,  since it generates no FPP instructions, provided

that symbol "realcode" (in RATRSX.RAT) is undefined.

To include an apostrophe in a string  or  format  statement,  use  a

double  quoted  string,  e.g.  FORMAT ("here's Johnny").  For direct

access READs and WRITEs, use two  apostrophies  together,  they  are

converted   to   a  single  apostrophe  in  the  output  code,  e.g.

READ (1''1).

When creating FORTRAN statements, the pre-processor  uses  statement

numbers  starting  at 20000 and going upward.  Therefore, you should

avoid using statement numbers in this range.  The pre-processor will

issue a warning message if it suspects a statement number conflict.

Study the RATFOR code of the pre-processor itself, it provides  some

good examples of what can be done with the language.

I find that I virtually never have  to  look  at  the  FORTRAN  code

generated  by  RATFOR.   I  think  the  success  of  RATFOR  depends

considerably on the concept that the RATFOR code IS the program, NOT

the generated FORTRAN code.

I find the most common RATFOR language error to be improperly nested

IF...IF...ELSE statements.  ELSE statements always apply to the most

recent IF that does not already have an ELSE.  Thus, if you have  an

IF...IF...ELSE  sequence, the ELSE applies to the second IF.  That's

fine if that's what you want, but if you want the ELSE to belong  to

the  first  IF,  you  must put brackets around the second IF to show

that it is a complete statement by itself;  the ELSE will then  fall

back to the first IF.

Example:

IF (statement)                  IF (statement) [

IF (statement)                  IF (statement)

statement                       statement

]

ELSE                            ELSE

statement                       statement

Assuming the indentation relfects the programmer's  intent  (ELSEing

the  first  IF),  the lefthand example is not correct, the righthand

one is.  Note that if the following statement were  anything  except

an    ELSE,    both    examples    are    equivalent,    since   the

IF (statement)/statement is  really  one  RATFOR  statement  and  is

(RSX) RATFOR DOCUMENTATION                                   PAGE 20

VERSION 20  FEBRUARY, 1980

therefore  controlled  as  a single unit by the first IF without the

need for brackets.

(RSX) RATFOR DOCUMENTATION                                   PAGE 21

VERSION 20  FEBRUARY, 1980

Here are some rough timing data for version 19:

The RATFOR input in all cases is the file RATRSX.RAT  (version  19).

It contains 800 RATFOR source lines, including 157 symbolic constant

definitions.  It produces 731  lines  of  FORTRAN.   The  data  were

collected on a dedicated 11/70 running RSX11/M,V3.1.  All files were

on the same RP06 disk.

...PREPROCESSOR BUILT WITH COMPILER OPTIONS....

FOR        FOR      FOR      FOR        F4P

/CD:THR   /CD:EIS  /CD:EIS   V2.2      V2.51

V2.1    /OP:SPD   /OP:SPD   /CD:EIS

ASCII    ASCII    NOTASCII   ASCII      ASCII

----------------------------------------------------------------

COMPILE TIME

(F,F=RATRSX)               :28                 :27       1:30

----------------------------------------------------------------

RATFOR TASK

SIZE(WDS)       28,352.   30,560.   31,776.   30,720.   28,800.

----------------------------------------------------------------

RAT F=F           1:38      1:06      1:49      1:10      1:00

----------------------------------------------------------------

RAT ,F=F          1:56      1:18      2:17      1:20      1:09

----------------------------------------------------------------

RAT F,F=F         2:04      1:25      2:38      1:28      1:17

----------------------------------------------------------------

RAT F,F=F/LC      2:07      1:26      2:38      1:28      1:18

----------------------------------------------------------------

NOTES:

The first line is the time  needed  to  compile  the  731  lines  of

FORTRAN  output.   The  second line is the size of the pre-processor

task if all pre-processor routines are compiled with  the  specified

compiler  options.   The last four lines are the elapsed time (m:ss)

require by the pre-processor to process  the  800  lines  of  RATFOR

source, given the command line specified.  For the task sizes above,

the symbols "openclose" and "spawnit" were  not  defined.   See  the

last  page  for  a  subroutine  time breakdown which shows where the

(Version 19) pre-processor spends its time.

Overlaying (in Version 20) saves about 4kw off the task sizes  given

above.   In  addition, other changes save about 3kw and speed up the

pre-processor about 8%.

If the symbol ASCII is defined  when  the  pre-processor  itself  is

processed,  the  external  character  set (ASCII on PDP-11's) is not

converted to the internal character  set  (which  is  ASCII).   Such

conversion  is not required on any processor which uses ASCII as its

character set, but would be on, say, IBM machines.  Relative to  the

second  column,  the third column shows the overhead associated with

this mapping.  Clearly, mapping every character on input and  output

is very expensive, although the mapping routines could be speeded up

somewhat.

(RSX) RATFOR DOCUMENTATION                                   PAGE 22

VERSION 20  FEBRUARY, 1980

F.      BUILDING RATFOR

A complete set of indirect command files is  provided  for  RSX11/M,

but  should  work under RSX11/D as well.  Under M+, D or IAS you may

want to edit the compile and taskbuild files to create a  multi-user

version.   The  task  build  command files must be edited to use the

correct resident libraries (if any) and device assignments for  your

system.   All  command  files  assume  RATFOR and STRLIB are in your

current UIC, and that the device is XX0:, you will  have  to  assign

XX0:   before  starting.   You  may  also have to edit the ODL file,

which assumes you have a FOROTS.OLB.  Note that  the  build  command

file puts all modules into RATFOR.OLB for the task builder.

When first installing  RATFOR,  build  it  from  the  FORTRAN  files

provided.   This  will  provide a basic version of RATFOR.  Edit the

RATFOR source code to configure it for your needs (most  things  are

in RATDEF.RAT) and pre-process it using the basic version of RATFOR.

Compile  and  task  build  this  customized  version  to  get   your

production version.

The pre-processor code is in four main files.  RAT1, RAT2, and  RAT3

contain the computer-independent parts.  All parts which may have to

be changed when moving RATFOR to a different environment are  lumped

into  RATRSX.RAT.   The whole thing is designed to be as portable as

possible  without  losing  too  much  speed  or   capability.    The

pre-processor  also  needs  STRLIB.RAT (string library) and ICSI.RAT

(command line interpreter),  and  several  COMMON  block  definition

files which are INCLUDEd all over the place.

A variety of symbolic  constants  are  available  in  RATDEF.RAT  to

configure  the  pre-processsor  for  specific  environments.   Also,

device assignments for all files  can  be  set  in  the  task  build

command files.

The internal character set used by the pre-processor is ASCII.  When

used  on  a  machine  whose  external  character  set it also ASCII,

PDP-11's, for example, much time is saved  by  not  converting  each

external  character  to  the equivalent internal character (which is

the same anyway).  However, if RATFOR is to be  used  on  a  machine

whose  external  character  set  is  not  ASCII  (IBM  for example),

conversion to and from the internal character set can be provided by

undefining  the  symbolic constant "ascii" in RATDEF.RAT because the

internal-external   conversion    code    is    all    conditionally

pre-processed.   You  must  edit  the ODL file to include OUTMAP and

INMAP.

If  defined  in  RATRSX.RAT,  the  symbol  "decf4p"  configures  the

pre-processor  for use with DEC F4P, if undefined, the pre-processor

is configured for DEC FORTRAN IV.  The  primary  difference  is  the

statement numbering on the FORTRAN code listing.  It also determines

which compiler is spawned by the /GO switch.

If defined, the symbol "decwriter" makes listing on  a  slow  device

(RSX) RATFOR DOCUMENTATION                                   PAGE 23

VERSION 20  FEBRUARY, 1980

faster  by  reducing  the  width  of  lines, especially the symbolic

constant table listing.

If symbol "strings" is undefined, the STRING data type and all  code

needed  for  it  are deleted from the pre-processor.  No ODL changes

are necessary.

Symbol "pagelength" can be adjusted for diferent line printers.

The symbol "openclose" (in RATRSX.RAT and ICSI.RAT) must be  defined

if  you  want  support  for  spooling of listing files included (the

default is still /NOSP).  If "openclose" is defined, OPENs are  used

everwhere  instead  of  ASSIGN/FDBSET.  This also allows for the odd

case where you specify /FO without creating a  FORTRAN  output  file

(by  creating  a  scratch  output  file).   This  also specifies the

correct carriage control so PIP does not get confused by the FORTRAN

carriage  control  which  otherwise  applies  to  the  output files.

However, DEC's FORTRAN IV version 2.2 has a bug (feature?) and  does

not  properly  handle  the  ERR=  transfer out of the OPEN statement

unless the reason for the ERR is "no such file".  Any  other  error,

such as a bad device name or non-existant UIC, causes an odd address

trap (error 3).  Therefore, be advised, if  you  define  "openclose"

and use FORTRAN IV v2.2 and get odd address traps, don't panic, it's

just DEC.

Another FORTRAN IV v2.2 bug causes the compiler  to  print  comments

following  an  END  statement  as  part of the just-finished program

instead of part  of  the  next  program  (in  a  file  with  several

modules).

The symbol "spawnit" (in RATRSX.RAT) must be  defined  if  you  have

RSX11/M v3.2 and want to spawn the compiler to do the OBJ file.  You

can define a standard set of switches to be applied to the  compiler

command line in module SPAWNF (in RATRSX.RAT).

The maximum nesting depth of INCLUDEs is one less then the value  of

"nfiles".   If  this  is  increased, logical unit number assingments

("stdout", etc) must be changed, since the input file LUNs are  also

the nesting level (LUN 1 being the basic source code file).

The maximum length of a line of RATFOR source code input is  defined

by "maxcard";  the maximum length of a DEFINE or MACRO definition by

"maxdef";  and the maximum  length  of  a  token  (including  quoted

strings) by "maxtok".

The maximum number of DEFINEs  and  MACROs  allowed  is  defined  by

"maxpts"  and  the  total number of characters in all definitions is

set by "maxtbl".

(RSX) RATFOR DOCUMENTATION                                   PAGE 24

VERSION 20  FEBRUARY, 1980

THIS IS TIMING FOR SYKES' RATFOR, VERSION 19.

(COURTESTY OF STEVE LAZARUS).

SINCE THIS TEST WAS RUN, SCOPY,TYPE,SPAD, AND SCOMPR HAVE BEEN

REWRITTEN TO SPEED THEM UP.

PROGRAM RUN ON 01/02/80  AND FINISHED AT 09:40:45.

122.53 SECONDS ELAPSED TIME      7229. SAMPLES      56 ROUTINES

STRPUT  28.94%    2092.0

STRGET  26.75%    1934.0

GTOK     6.47%     468.0

SLEN     4.14%     299.0

SCOPY    3.83%     277.0

GETTOK   2.70%     195.0

SEQL     2.45%     177.0

NGETCH   2.24%     162.0

DEFTOK   1.90%     137.0

LOOKFR   1.87%     135.0

SCOMPR   1.80%     130.0

TYPE     1.69%     122.0

OUTSTR   1.47%     106.0

SHELL    1.34%      97.0

.MAIN.   1.16%      84.0

SPAD     1.13%      82.0

SITOC    1.07%      77.0

OUTCH    0.79%      57.0

PRTLIN   0.73%      53.0

LEX      0.73%      53.0

UNFOLD   0.66%      48.0

RATLST   0.62%      45.0

PUTBAK   0.59%      43.0

INDEX    0.58%      42.0

EATUP    0.58%      42.0

EQLS     0.54%      39.0

GETLIN   0.48%      35.0

PBSTR    0.41%      30.0

PUTLIN   0.33%      24.0

OPENI    0.26%      19.0

ALLDIG   0.24%      17.0

PARSE    0.24%      17.0

STRIM    0.14%      10.0

BALPAR   0.14%      10.0

OUTTAB   0.12%       9.0

RELATE   0.10%       7.0

OTHERC   0.10%       7.0

OUTDON   0.08%       6.0

ADDDEF   0.06%       4.0

FORCOD   0.06%       4.0

LRPAR    0.06%       4.0

UNSTAK   0.06%       4.0

#####################################################################################################################################################################################################################################################################################################################################################################################################