SUPERCOMM

IBM PC Communications drivers

(C) Copyright 1986

Doctor Debug

Pittsburgh, Pa

All Rights Reserved

****************************************************************

*                                                              *

*         As  with all the Doctor's software,  SUPERCOMM       *

*         has  been tested and by all indications  works       *

*         as described under all reasonable  conditions.       *

*         However, the good doctor cannot be responsible       *

*         for damages,  heartbreak,  outbreaks of war or       *

*         acts  of  God  caused  by  the  use  of   this       *

*         software.    As   it   is   FreeWare,   *free*       *

*         distribution  is  encouraged  so long  as  the       *

*         programs  and  documentation  are   unaltered.       *

*         Please  read  the contribution information  at       *

*         the end of this document.                            *

*                                                              *

****************************************************************

Anyone  who has attempted to use the BIOS Int 14  to  access

the  IBM's Communications ports knows that these routines are the

pits.   They  allow  absolutely  no  flexibility,   and  also  no

interrupt-driven  means  of reading the ports.  SUPERCOMM  is  an

answer to that problem.

Running  the  program  SUPER.COM  is  necessary  to   access

SUPERCOMM.  This  program  can easily be placed in your  AUTOEXEC

file if you wish.  Part of the program stays resident,  replacing

Interrupt 14.

After  SUPER.COM has been run,  the following functions  are

available  to the new interrupt 14.  As with the BIOS  interrupt,

select the function by loading AH with the function number.

Function 0 - set Communications status

Unchanged from the BIOS Int 14. Does not activate SUPERCOMM.

This is really just kept for compatability sake.

Function 1 - Transmit Byte

A  bug  which  caused this function not to  work  with  some

modems has been fixed, but otherwise this function is the same as

the BIOS function.

Function 2 - Receive Character

Removes  a  character  from the circular  input  buffer  and

returns  it in AL.  If no character is available a timeout occurs

(bit 7 of AH will be set.)

Function 3 - Port Status

Unchanged.

Function 4 - Clear Buffer

Empties  Communications input buffer.  (The buffer is 4K  in

length)

Function 5 - Block Receive

Requests a block of data be read from the comm port.

ENTRY:    DX   =    Port (0 or 1)

CX   =    Size of block in bytes

ES:BX=    address to which data will be written

RETURN:   CX   =    16 bit checksum of data read

AL   =    Error codes (as in original Int 14)

Function 6 - Block Send

Sends a block of data over the comm line

ENTRY:    DX   =    Port (0 or 1)

CX   =    Size of block in bytes

ES:BX=    Address of block

RETURN:   AL   =    Error codes (as in original Int 14)

Function 7 - Buffer Status

Number of bytes waiting in the buffer returned in BX

Function 8 - Get Character

Equivalent  of  INKEY from keyboard.  Gets a byte  from  the

input  buffer if one is waiting.  Byte is returned in AL.  If  no

byte  was waiting,  AL will have 0.  Don't use this function  for

data which might be 0.

Function 9 - Set Port

Port in DX (0 or 1) will be buffered and used by  SUPERCOMM.

The other will be handled by the old BIOS routines.

Function 12 - Open SUPERCOMM port

Load  registers  as in old Interrupt 14's Function  0.  Port

specified in DX becomes active SUPERCOMM port.  System  interrupt

speed  is kicked up to 4550 times/sec for goof-proof buffering of

incoming  data.  This function must be called before any  of  the

input  or output functions are.  While the SUPERCOMM port is open

overall system speed may lag as much as 35%,  so be sure to close

the  port (using function 13) when you are through.  Leaving  the

port  open  at the end of a user program may cause a  crash  when

another program is loaded.

Function 13 - Close SUPERCOMM port

Simply load the port (0 or 1) into DX and call this routine.

System  interrupt  speed is returned to normal and  SUPERCOMM  is

deactivated until the next Function 12.

Function 14 - Peek buffer

This  handy function returns the next character in the input

buffer in AL,  but does not remove the character from the buffer.

This  means you can look ahead one char into the  buffer.  If  no

characters are waiting AL will return with a 0.

SUPERCOMM  returns error codes in AH and AL (unless they are

returning other information).  These error codes will be the same

as  those returned by the standard Input/Output routines  of  the

old Interrupt 14.

All this is pretty complicated, which is why I have included

the file SUPERCOM.PAS, which contains turbo Pascal routines which

access  these functions.  DUMBTERM.PAS is a demonstration program

which  is  essentially a plain vanilla  comm  program.  Examining

these  programs  will  provide some insight  into  how  SUPERCOMM

works. Be sure to compile any Turbo programs using SUPERCOMM to a

.COM file before using!  DUMBTERM.COM has been included for those

out  there  without  a Turbo Pascal Compiler.  Also  included  is

DUMB.ASM  and DUMB.COM,  which are essentially the same thing  as

DUMBTERM.PAS only in 8086 assembler.

Pascal Routines

(Found in SUPERCOM.PAS)

These routines are meant to be called by user programs. The

SUPER.COM Interrupt 14 driver must have been installed to

use any of these routines. Use of these routines without proper

installation of SUPER.COM will produce unpredictable results.

It should be relatively simple to translate these turbo routines

into C or most any other language.

The integers InError and OutError will always contain the error

conditions after every receive or transmit. The bits of these

values are defined as:

Bit 7 (128)        Timeout

Bit 3   (8)        Framing Error

Bit 2   (4)        Parity Error

Bit 1   (2)        Overrun Error

If the value of InError[port] is 0, then you can be sure that the

last character was received without error.

The value or Port is always 1 or 2.

Procedure InitPort(port,Baud,Parity,data_bits,stop_bits)

Baud: integer 300-9600

Parity: char, E(ven),O(dd),N(one)

Data_bits: integer, 7 or 8

Stop_bits: integer, 1 or 2

This routine initializes the communications port

to the parameters specified and activates SUPERCOM

for that port. All of the following functions will

use the port specified here.

Function PortStatus(port)

This function returns the line status and modem control

status of the comm port specified. The bits returned are

defined as:

Bit 15 (negative)  Time out (no device connected)

Bit 14 (16384)     Transmission shift register empty

Bit 13 (8192)      Transmission holding register empty

Bit 12 (4096)      Break detect

Bit 11 (2048)      Framing error

Bit 10 (1024)      Parity error

Bit 9  (512)       Overrun error

Bit 8  (256)       Data ready

Bit 7  (128)       Received line signal detect

Bit 6  (64)        Ring indicator

Bit 5  (32)        Data set ready

Bit 4  (16)        Clear to send

Bit 3  (8)         Delta receive line signal detect

Bit 2  (4)         Trailing edge ring detector

Bit 1  (2)         Delta data set ready

Bit 0  (1)         Delta clear to send

Procedure XmitCh(ch)

This Procedure sends the character in ch out the port

specified.

Procedure XmitBlk(string)

This procedure sends the entire string out the comm port.

Procedure XmitLn(string)

This is identical to XmitBlk, but adds a CR/LF to the

end of the block.

Procedure RecCh(ch)

This procedure waits until a character is available over

the comm line and then returns it in ch. If the system times

out ch will contain a nul (Ascii 0).

Procedure RecLn(string)

This is the equivalent of ReadLn over the comm port.

Be sure to check the InError variable to make sure the

operation did not time out (no CR was received.)

Procedure RecBlk(number,String)

The number of characters specified by number will be

placed into the string. Be sure to check the InError

variable to assure that the operation did not time out

before sufficient characters were received.

Procedure GrabCh(ch)

If a character is waiting in the receive buffer it will

be returned in ch otherwise ch will contain a nul character.

Procedure PeekBuff(ch)

Identical to GrabCh but the character is not removed

from the buffer.

Procedure ClearBuff

Empties the receive buffer

Procedure ClosePort

Closes the comm port and deactivates SUPERCOMM until the

next InitPort.

Function Rlen

Returns the number of characters currently available in the

receive buffer.

Summary

(How to use SUPERCOMM in a few semi-easy steps)

1)  Make  sure that SUPER.COM has been run so that  SUPERCOMM  is

resident. It is very difficult for software to work properly when

in is not in the computer's memory.

2)   Your program should first issue an Interrupt 14 with AH  set

to 12 (opens the port with SUPERCOMM) and the other registers set

as  in the old BIOS Interrupt 14 Function 0.  This  assures  that

SUPERCOMM is humming away,  busily checking comm activity on  the

port specified in DX  some 4500 times a second.

3)    Now  you can use functions 1 through 8 to input  of  output

all the happy data over that comm port.

4) Before exiting the program,  issue an Interrupt 14 with AH set

to  13 (close SUPERCOMM port).  This assures that all is back  to

normal for any programs which you run later.

Believe  me,  using  it  is a great deal  more  simple  than

writing it was!

If you have questions,  comments, suggestions or criticisms,

you can reach me on the Neverboard,  (412) 733-4842 1200/8/N.  If

you find SUPERCOMM useful, a donation of, say, $15 may be sent to

the doctor's office:

Doctor Debug

1 Windsor Road B-6

Pittsburgh, PA  15239

If you use these routines in a commercial product, the above

donation  is **not** optional.  The donation will make sure  that

you  are  informed of upgrades and other pieces of  the  doctor's

software.