WHAT IS FAST2 AND HOW DOES IT WORK?

-----------------------------------

What  is  FAST2?  It is a new transient program for the CP/M  (*)

disk  operating system.    It was originally written by by  Roger

van  Valzpah and released by the CP/M user group as  FAST.   This

version  was  modified by David Bennett of the  Alfred  Hospital,

Melbourne to be compatible with CP/M version 2.0.

FAST2  is used in conjunction with other transients to  speed  up

their execution.    Of course,  nothing but a hardware change can

increase the rate at which your CPU executes instructions.   But,

most  transients are "disk bound",  that is,  they spend a  large

percentage  of  their total execution time just waiting  for  the

information they need from the disk.

FAST2 uses this fact to increase execution speed by reducing  the

time spent waiting for the disks.   This is accomplished by using

two different types of buffering.   Firstly, portions of the disk

which  are  frequently  accessed (the directory) are  held  in  a

buffer,  eliminating  the need to seek to read them.    Secondly,

disk  accesses are grouped together in time to reduce the  amount

of time lost to rotational latency and head load delay.

HOW TO USE FAST2

----------------

The  normal  CP/M  command  to assemble  a  file  with  the  name

MUMBLE.ASM is:

A>ASM MUMBLE

To perform the same function under FAST2, simply type:

A>FAST2 ASM MUMBLE

Thus,  in its simplest form, FAST2 can be used as a prefix to any

normal  CP/M  command.    This  will load FAST2 which  will  link

itself  to your operating system and allocate its  disk  buffers.

Then  the  transient ASM.COM is loaded and given  control.    The

fact  that FAST2 is in the system is totally transparent  to  the

transient, except for the reduced memory size.   Messages will be

printed  just  before  and  after the transient  is  executed  to

indicate that FAST2 is in control.    After execution, you should

see something like this on your terminal:

A>FAST2 ASM MUMBLE

Alfred Hospital FAST 2.01

CP/M ASSEMBLER - VER 1.4

1234

056H USE FACTOR

END OF ASSEMBLY

FAST terminated

A>

Where N.NN is the version number of FAST2 that you are using.

COMMAND LINE OPTIONS

--------------------

The  buffering  action of FAST2 may be altered with  the  use  of

command  line  options.    This is accomplished by following  the

FAST2  command with an option string enclosed in square  brackets

("["  and "]").    A valid option string must consist of a  drive

specification   followed   by   one  or   more   buffering   mode

specifications.    This  group may be repeated for  buffering  of

multiple  drives.   Examples  of typical option strings  will  be

given below.

Explicitly  specifying  the buffering mode is desirable to  reach

the  best trade-off between speed of execution and memory  usage.

In  general,  as  more buffering is used,  execution  speed  will

increase.    However,  more  buffering means more  memory  usage.

The  type  of buffering selected should be tailored to  the  disk

usage  patterns of the transient being executed.    For instance,

write buffering will offer no speed improvement when a  transient

only  reads the disk.

One  thing  that  FAST2 must know is which disk drive  is  to  be

buffered.    This  is  communicated to FAST2 simply by using  the

single  letter  name of the desired drive (e.g.    "A"  for  disk

drive A).    As a convenience,  the current default drive may  be

specified  with the commercial at sign ("@").

The second thing which FAST2 must know is which mode of buffering

is to be used.    FAST2 supports three different buffering modes.

Each  mode may be used individually or in combination with  other

modes to offer the fastest execution.   The three modes available

are:

Seek buffering

Read buffering

Write buffering

Yes (all of the above)

The  type  of  buffering desired is communicated to  FAST2  by  a

single  letter in the option string.    The letter is simply  the

first  letter  of the word which describes the type of  buffering

(capitalized above).   The operation and application of each mode

will  be discussed below.    Thus,  a typical FAST2 command  line

might look like this:

B>FAST2 [ASWBSR] ASM MUMBLE.BAA

In this example,  the .ASM source file is on drive B so read  and

seek buffering have been specified for that drive.   The A drive,

on the other hand,  is to receive the result of the assembly (the

.HEX and .PRN files), so it gets write and seek buffering.

BUFFERING MODE DESCRIPTIONS

---------------------------

Seek buffering causes the disk directory to be read into a buffer

the  first time a drive is accessed.    From then on,  all  reads

from  and  writes  to the directory can be  carried  out  without

moving   the  disk  head  from  its  current  position.     Thus,

transients which access the directory frequently will be sped  up

considerably by seek buffering.   Transients which fall into this

category   are  those  which  deal  with  many  different   files

simultaneously,  perform  operations  with  temporary  files  and

rename them, read or write large (more than 16K) files, and those

which perform random disk I/O.   In particular, ASM.COM, MAC.COM,

PIP.COM,  and  ED.COM  are examples of  such  transients.

Read  buffering causes an entire track from the disk to  be  read

into  a buffer the first time any sector is read from that track.

This increases execution speed because CP/M typically reads  most

of one track before going on to the next one.   Additionally, the

time  required to read a whole track is a fairly small percentage

increase  over the time required to read a single  sector.    The

net effect is that less time is spent waiting for the  rotational

latency of the disk.    This mode of buffering is most beneficial

to  transients which read disk files sequentially.    As a matter

of fact, it may slow down transients which read files in a random

access mode.    Fortunatlly, the vast majority of CP/M transients

read  the disk sequentially.    Examples of such  transients  are

ASM.COM,  MAC.COM, PIP.COM, and BASIC-E.COM (if the BASIC program

doesn't do random I/O!!).

With write buffering,  sectors that are to be written to disk are

held in a buffer for a time, instead of writing them immediately.

Sectors  for  any given track are held in the buffer  until  CP/M

tries to write to a track that is not buffered.   When it is time

to  change  tracks,  only those sectors which  were  changed  are

actually written before clearing the buffers to make room for the

new  data.    This improves execution speed for the same  reasons

that read buffering does,  i.e.   a whole track can be written in

about  the time it takes to write a single  sector.    Transients

which benifit from this type of buffering typically write to disk

sequentially.   Examples include ASM.COM, MAC.COM, and ED.COM.

DEFAULT PHILOSOPHY

------------------

Two of the qualities generally associated with a good program are

versatility and ease of use.    FAST2 has been written with these

two qualities in mind as the primary design goals.    Many times,

however,  these  two  goals can be contradictory.    A  versatile

program is one which is capable of a wide variety of tasks or one

that  fits many different FAST2 Default philosophy  applications.

This  usually means a program with many user selectable  options.

Forcing  the user to type the same frequently used options  every

time  a program is invoked or committing him to remember  a  long

list  of  options  is contradictory to the goal of ease  of  use.

This conflict is resolved by the use of default mechanisms within

FAST2.

A  default  mechanism is simply a rule that can be used  to  make

assumptions  about the user's wishes when he has not stated  them

explicitly.   Thus, versatility has been retained by allowing the

user the ability to specify options explicitly, while the default

mechanism frees the user from this tedium much of the time.    In

short,  defaults allow versatility and ease of use to  peacefully

co-exist in the same program.

.cp 3

DEFAULT MECHANISMS IN FAST2

---------------------------

There  are two levels of defaults built into FAST2.    The  first

default mechanism is used when either the drive or buffering mode

is omitted from an option string.   If a drive name is not given,

the  default drive for CP/M is used (this is equivalent to  "@").

If  no buffering mode is specified,  read and seek buffering  are

used.   Thus, the following two commands would be equivalent.

B>FAST2 [BRW] LOAD MUMBLE

B>FAST2 [RW] LOAD MUMBLE

And so would the following two commands.

A>FAST2 [BRS] PIP B:THIS=B:THAT

A>FAST2 [B] PIP B:THIS=B:THAT

The  second  default  mechanism comes into play  when  no  option

string  is given on the command line.    Instead of executing the

transient with no buffering,  FAST2 uses a default option  string

to specify the buffering used.   This string is [@RS], giving the

user  read and seek buffering on the default drive when no option

string  is present on the command line.    The user may,  at  his

option,  alter  the default option to one which is more  suitable

for his typical uses.    For instance, if you typically use FAST2

for  assembling  and  don't have to worry about  running  out  of

memory,  a good default string would be [@RWS].    Conversely, if

you are running in a small memory system,  a good choice would be

to change the default option string.

HOW TO CHANGE THE DEFAULT OPTION STRING

---------------------------------------

The  default string is stored in FAST2 at address 0130H.    Thus,

to  change  the  option string,  use SID or  DDT  to  change  the

existing string.   Assuming you have SID this is very easy:

A>SID FAST2.COM

SID VERS 1.4

NEXT PC END

0A00 0100 94FF

#S130

0130 5B "[@RWS]

0136 20 .

#^C

A>SAVE 9 FASTX.COM

If you do not have SID, you'll have to use DDT and figure out the

hex  for  the ASCII string you wish to patch in.    Then use  the

Substitute command to patch the file as above.    In either case,

test  the  patched  file before killing  the  old  FAST2.COM  and

renaming the new file to FAST2.COM.

.cp 3

WARNINGS N-E-V-E-R CHANGE DISKS WHILE EXECUTING UNDER FAST2!!!

And in case you missed that one I'll say it again...

YOU  MUST

**********  N-E-V-E-R   N-O-T    E-V-E-R  **********

CHANGE  DISKS

WHILE EXECUTING UNDER FAST2!!!

(not even if you think you know what you are doing)

Wait  until  you see the message "FAST2 terminated"  before  even

thinking about switching disks.    A rather nasty side effect  of

seek  buffering  is  that it disables the disk  change  detection

mechanism built into CP/M.   This means that you will NOT get the

warning  message "Bdos error on A:  R/O" if you forget and change

disks   without  booting.    Instead  you  will  get   wild   and

unpredictable  effects  possibly including destroying  your  disk

directory.

This  also  gives  rise  to  a  minor  incompatibility  with  any

transients  which  use the BDOS reset function  (BDOS  call  13).

This  function  is  used in transients which allow  the  user  to

change  disks  without  rebooting.    Some examples  are  MBASIC,

CBASIC,  WORDSTAR,  and others.   Fortunately they only this BDOS

function  when  an explicit command is used  to  permit  changing

disks.   FAST2 is compatible with transients of this sort as long

as no attempt is made to change disks.

One  way  to  avoid  trouble is to avoid  using  FAST2  with  any

transient  which requires user input.    Any such  transient,  by

waiting  for your reply,  would leave you with the opportunity to

change disks.   To minimise the risk of your running into trouble

with  such programs,  FAST2 watches out for any requests for user

input.  When such a request is detected, FAST2 immediately writes

all  modified  blocks  back  to  disk.   While  this  gives  some

protection  against  lost data,  it fails with programs  such  as

Wordstar  which  monitor console status and  only  request  input

after a key is struck.  Again, DON'T change disks.

COMPATIBILITY

-------------

FAST2  is fully compatible with the following transients and  can

be expected to provide a significant reduction in execution time.

CP/M utilities:LOAD, SUBMIT, DUMP, MAC

Compilers:     ASM, MAC, RMAC, M80, FORTRAN, F80, COBOL-80,

PL/I, etc.

FAST2 is fully compatible with the following transients,  but due

to the fact that they require user input,  some caution should be

exercised  when they are used with FAST2.    As mentioned  above,

stopping  for  input gives the user the chance  to  inadvertently

change  disks  which CP/M would not be able to detect.    If  the

user is able to guard against this occurrence,  he may use  FAST2

with  these  transients  and expect a  significant  reduction  in

execution time.   In particular,  the improvement in the speed of

PIP is little short of unbelievable.

CP/M utilities:PIP, DDT, SID, ZSID

Editors:       ED, WORDMASTER, WORDSTAR, ED-80

Interpreters:  MBASIC, RUN, CRUN, CRUN2, PRUN

Other:         SELECTOR, ADVENTURE

FAST2 is fully compatible with the following transients,  but due

to  the  way in which they access the disk,  little or  no  speed

improvement can be expected.

Special utilities: SYSGEN, MOVCPM, FAST2, COPY

Due to an unfortunate memory dependency,  STAT (version 1.4) does

not  correctly report the number of bytes remaining on  the  disk

when it is executed under FAST2.    This is no great loss because

FAST2 can do little to speed up the execution of STAT.

Best  results  are  obtained  in situations  where  the  disk  is

accessed  sequentially,  especially if large files are read  from

beginning  to end.   The following examples indicate some of  the

results which can be expected using standard single-sided single-

density 8 inch diskettes.

A>FAST2 [AYBY] PIP B:=*.*

This command can be expected to copy a 241k diskette full of

data, file by file, in about 80 seconds.  With verification,

it takes perhaps 100 seconds.

A>FAST2 [ARBY] WS

Use this command on your Wordstar utility disk to edit  text

on drive B: (but don't change disks!) and you will marvel at

the  new version of your editor.   It will load overlays  in

the  blink  of an eye and save files (e.g.  ^KS) in  nothing

flat.

A>FAST2 [ARBY] COBOL CMUMBLE

The  compilation for a small 100 line program  drops  from

about 90 seconds to less than 40.   The biggest gains are in

loading  overlays of the compiler.

A>FAST2 [ARBY] CBASIC BMUMBLE $BF

One of the most irritating features of this compiler is that

if  you switch off the listing,  you don't know which  lines

contain  any errors which are detected.   Solution:  save  a

listing  on disk and with FAST2 the compilation  will  still

take no longer than before.

FAST2 MESSAGES

--------------

During  the course of execution,  FAST2 may print any of  several

different  messages.    Each  of these messages is  listed  below

along with conditions under which it is printed.

Alfred Hospital FAST 2.01

This  message is printed after the transient to be  executed

under  fast  has been successfully loaded,  but just  before

control is transfered to it.   This is simply an informatory

message  and does not signal an error condition.    In  this

message,  N.NN is the version number of FAST2 which you  are

using.

FAST terminated

This  message  is printed after the transient  has  finished

execution and all disk buffers have been emptied.   This too

is  simply  an  informatory  message  printed  under  normal

conditons.

NO COM FILE

This message is printed when the transient filename given on

the  command  line cannot be found in  the  directory.    No

recovery  action  can be taken,  so the  execution  complete

message is printed and FAST2 re-boots.

OUT OF MEMORY

This message is printed when the size of the TPA is exceeded

while  allocating  buffers or while loading  the  transient.

In  either  case,  no recovery action can be taken,  so  the

execution  complete message is printed and  FAST2  re-boots.

There  are  several solutions to the out of memory  problem:

use less buffering,  use smaller transients,  or, of course,

buy more memory.

DISK WRITE ERROR

This message is printed when FAST2 attempts to write data to

disk and gets the unsuccessful completion flag back from the

BIOS.    The  only  recovery  action  taken  is  simply  the

printing of the message,  then execution continues normally.

This  usually  indicates a serious error (like  a  protected

disk) and the user should boot and take corrective  measures

as soon as possible.

INVALID OPTION

This message is printed whenever an error is detected in the

option  string  being  scaned by FAST2.    This may  be  the

option  string supplied on the command line or  the  default

option  string  in memory in the unlikely event that it  has

been  incorrectly  modified.     Some  examples  of  invalid

options are the null option ([]), options containing invalid

characters ([Q]),  or options which specify buffering of the

same type for the same drive twice ([ABA]).

MEMORY HIT

This message is printed whenever FAST2 attempts to write  to

a  memory  location  and cannot read the data  it  has  just

written.   The recovery action taken is to print the message

and  ignore  the  error.    It is usually  an  indicator  of

hardware  problems in your system and should be looked  into

immediately.    If  MEMORY HIT errors are persistent and you

have a ROM monitor in your system, you can use FAST2 to help

find the memory at fault.    Create a special FAST2.COM with

a  jump to your monitor patched into location  521H.    This

location is called when a memory hit is detected.    The  HL

register  contains the address of the bad byte and a bit  in

the A register is set for each bad bit in memory.

MEMORY ALLOCATION

-----------------

When  FAST2 is loaded,  it automatically relocates itself to  the

top  of the TPA (just like DDT and SID).    This action  overlays

the  CCP.    A  side  effect  of this is  that  transients  which

normally  return  to  the  CCP without  booting  will  boot  when

executed under FAST2.   An example of such a transient command is

LOAD.

As  mentioned earlier,  the only way a transient can tell that it

is  being  executed under FAST2 is that the size of  the  TPA  is

decreased.    The  amount of memory taken out of the TPA by FAST2

is dependant on the number of buffers which are allocated.

.cp 4

The  formulas  used to determine the amount of  memory  used

are:

buffers = NTB * (3 + 131 * SPT) + NSB * (3 + 131 * SID)

SID = NDE / 4

mem used = 768 + [buffers]

Where:  NTB is the Number of Track Buffers  allocated

SPT is the number of Sectors Per Track

NSB is the Number of Sector Buffers allocated

SID is the number of Sectors in the Directory

NDE is the maximum Number of Directory Entries

[ ] indicate rounding up to the next multiple of 256

For  a normal 8 inch IBM compatible version of  CP/M,  all  these

computations can be replaced by a simple estimating rule:   Allow

1024  bytes for FAST2 code overhead,  3409 bytes for  each  track

buffer (R or W), and 2099 bytes for each seek buffer (S).   As an

example, FAST2 executed without a command line option string will

use 1024 + 3409 + 2099 bytes.    If this sum is rounded up to the

next multiple of 256, the actual memory usage figure becomes 6656

bytes.   The maxmimu on a two drive system is the command  option

[AYBY]  which  costs 1024 + 4x3409 + 2x2099 or a total  of  18.5K

(18944 bytes).

####################################################################################################