Mycroft Labs SEND/RECV Utilities v2.0

The SEND and RECV programs included on this disk are useful for

transferring files between two CP/M systems with incompatible disk

formats, over a communications line. To use them, type in the RECV

program on the new system, using ED. Comments may be ignored at this

time. Modify the port addresses, and the routines INIT, RACL and WACL

as necessary. Assemble using ASM or MAC, then load using LOAD.

NOTE; The SEND.COM and RECV.COM are already patched for your Kaypro-II

serial port at 9600 baud.  They even set the baud rate for you.  If

you want to use them to send to another cp/m computer, you'll have to

know enough assembly language to patch the SEND.ASM and RECV.ASM files

and assemble them and make them work on the other computer.  You'll

also have to make a cable to connect the Kaypro's serial port to the

other computer and get the handshaking working.

At this point, it is possible to send the full, documented source

of the RECV program over, make the same changes in it, assemble and

load it. The SEND program can then be sent over as well, and modified

for the communications port in use. Now files can be transferred in

either direction.

To use these programs, execute RECV (no parameters) on the system

which is to receive the new files, and then execute SEND on the system

from which the files are to be sent. The argument of the SEND command

may be a single file name, or an ambiguous file name (e.g. *.ASM).

Files will be sent with checksums and automatic re-transmission, if

needed, one sector at a time. Use the highest baud rate your hardware

is capable of for minimum transfer time.

Note that these programs are the property of Mycroft Labs, and have

been released to Microsoft Inc. for inclusion on their Apple CP/M disk.

They are not to be sold or included in a hardware or software package

without the express permission of Mycroft Labs. For permission, contact

Larry Hughes at (904) 385-2708, P.O. Box 6045, Tallahassee FL 32301.

##########################################################################################################################