BANK SWITCHING  - Tony Drake    1/83

Bank switching is a hardware/software technique used in the Kaypro

(and other microcomputers) to allow additional memory to be

accessible. Under usual conditions, the Z80 microprocessor can

directly address 64K (65,536) bytes of memory. When the

bank-switch capability has been built in to the computer, as it

has in the Kaypro, new segments of memory, called "banks", can be

switched in over the top of old segemnts, used for a while, then

the old segments can be returned without "forgetting" anything.

The Kaypro has one bank of 16K bytes that can be switched in and

out. The normal, unswitched bank is called BANK 0. The optional,

switched bank is called BANK 1.

Non-Linear Systems (NLS) did not put user memory in BANK 1. The

first 2K bytes consists of preprogrammed, permanent memory (ROM)

containing special system routines that are used to control the

keyboard, the screen, the disk drives, and the serial and parallel

ports. The CP/M operating system uses these routines extensively

in running the computer. A programmer can use these routines to

his advantage if he wishes to learn how.

NLS did not put any memory in the next 10K of this bank. A program

written to "read" any of this 10K would find it empty.

The last 4K of this bank contains the video display memory. Any

character codes written in this memory automatically show up on

the screen in a corresponding position. This is called

memory-mapped video. Normally, system routines (in BANK 1) are

used to control screen displays, but again, a programmer can write

directly to the screen memory if he wishes.

Switching BANK 1 with BANK 0 is easy to do, but the programmer

must not forget which bank is currently "on". The switch is made

by turning on bit 7 of the system port. Bits are numbered from

right to left, starting with bit 0, so the bank-switch bit is the

"highest", or most significant bit (MSB) of the system port. The

other bits of the system port are unrelated to bank switching and

must not be altered when making the switch. The system port is

found at port address 1C (hexidecimal). Using INTEL 8080 Assembly

language, the switch to BANK 1 can be made with three

instructions:

IN   1CH         ;INPUT CURRENT SYSTEM PORT DATA

ORI  80H         ;TURN ON HIGH BIT

OUT  1CH         ;OUTPUT NEW SYSTEM PORT DATA

Similarily, in ZILOG Z80 Assembly language:

IN   A,(1CH)     ;INPUT CURRENT SYSTEM PORT DATA

SET  7,A         ;TURN ON HIGH BIT

OUT  (1CH),A     ;OUTPUT NEW SYSTEM PORT DATA

To switch back to "normal" BANK 0 using INTEL 8080 Assembly

language:

IN   1CH         ;INPUT CURRENT SYSTEM PORT DATA

ANI  7FH         ;TURN OFF HIGH BIT

OUT  1CH         ;OUTPUT NEW SYSTEM PORT DATA

To switch back to "normal" BANK 0 using ZILOG Z80 Assembly

language:

IN   A,(1CH)     ;INPUT CURRENT SYSTEM PORT DATA

RES  7,A         ;TURN OFF HIGH BIT

OUT  (1CH),A     ;OUTPUT NEW SYSTEM PORT DATA

The programmer must be sure that his bank switch instructions are

not in the first 16K of memory, since this memory is switched

"out" when BANK 1 is switched "in". (With the instructions

switched out, the machine won't know how to get back to BANK 0.)

The program SYSCOPY uses bank switching to copy the system ROM

routines out of BANK 1 into normal memory. Here they can more

easily be studied by disassembling with DDT.

The program VIDEO uses bank switching to write directly to the

video memory. All hexidecimal codes from 00 to 7F are written to

demonstrate the entire displayable character set.