(Version 2.0, December 5, 1985)

INT13 DOCUMENTATION

The file INT13V20.ARC contains the following:

INT13.ASM - Source file for INT13.COM

INT13.COM - Resident interrupt 13h handler

INT13.DOC - This documentation file

I. Function of INT13.COM

INT13.COM is a resident "front-end" program which intercepts ROM

BIOS interrupt 13h calls to perform disk functions on specific sectors.

INT13.COM prints out the following information about each interrupt

13h call:

Information sent to Interrupt 13h processing-

1. Requested disk drive (program currently reports only calls to

drive A:)

2. Side number

3. Track number

4. Sector number

5. Number of sectors requested

6. Operation type (0=reset 1=read status 2=read 3=write 4=verify)

Information received from interrupt 13h processing-

7. Return code (result of interrupt 13h call, 00 = no error)

8. Address immediately after point where call was made

*      9. ST0 - see XT Technical Reference Manual pps. 1-164-166

*     10. ST1    "  "      "         "       "     "      "

*     11. ST2    "  "      "         "       "     "      "

*     12. C  (track found by controller)

*     13. H  (side found by controller)

*     14. R  (sector found by controller, not reliable)

*     15. N  (sector size, 0=128, 1=256, 2=512, 3=1024, etc.)

* = new features of version 2.0

II. Purpose of INT13.COM

INT13.COM was written as a tool to help unravel copy protection

schemes, virtually all of which use interrupt 13h to look for weirdly-

formatted tracks on the original "key" diskette.  This utility allows

you to find out - without searching through a disassembly of the program

- what the copy protection scheme is looking for on the key diskette.

The return address printed shows where (or, more accurately, the point

immediately after) the call to interrupt 13h was made.  This can be

especially useful where the copy protection scheme trys - as many do -

to disguise the interrupt call so as to thwart casual attempts to defeat

the protection.

I will not attempt to provide detailed information on the various

codes associated with interrupt 13h processing since this information is

available in any book on IBM Assembly Language or in the IBM Technical

Reference manuals.  In general, however, a few things to look for in a

protection scheme are:

a.  Sector numbers outside the normal range of 1 through 9.

b.  Sectors deliberately formatted with CRC errors.  The pro-

tection scheme either "reads" (operation code 02) or "veri-

fies" (operation code 04) the sectors.  The program then

expects to get a return code of 10, indicating the sectors do

indeed contain CRC errors.  (CRC errors can occur in the data

field and the id fields.  The return code field does not

distinguish between the two; the type of CRC error is shown

by the values in ST1 and ST2).

c.  Multiple calls for the same sector.  Some schemes format a

track with two sectors with the same number.

d.  Missing sector numbers on an otherwise normal track.

e.  Sectors with "N" sizes other than the normal 2 (512 bytes).

III. Using INT13.COM

Before executing the program to be examined, simply run INT13.  A

short message will be displayed indicating that the program has been

installed.  Your printer will display the information described above,

one line for each interrupt 13h call.  When you are finished, execute

INT13 one more time, and it will un-install itself, displaying a message

to this effect.

With the amount of information displayed for each int 13h call,

appoximately 130 characters are printed on each line.  The default setup

is for INT13 to use the Epson "compressed" mode of printing to accomodate

all the data on a single line.  If your printer does not have a

"compressed" mode, you may force 80-column printing by entering INT13 /8.

If you are not using DEBUG, then simply execute your program.  A log

of the disk activity will then be printed.  Program loading and execu-

tion will be slowed due to the time required for printing.

If you are using DEBUG, then proceed as you would normally.  The

addresses reported on the disk activity log will correspond to the

locations in the program being debugged so you may immediately see where

the calls are being made.

It is advisable to run INT13 with your program more than one time

since some protection schemes select different tracks for checking on a

random basis.

Note: Since copy protection schemes rarely, if ever, check other

than the "A" diskette drive, INT13 is set up to log only calls to that

drive.  If you wish disk activity on all drives to be logged, remove the

instructions listed in the source code and reassemble the program.

IV.  Interpreting the results

This is the hardest part.  If you are going to become an accom-

plished "unprotector" you will have to have at least a passing knowledge

of assembly language.  INT13 will point you to the part(s) of the pro-

gram doing the protection verification and will show you what the pro-

gram expects to find.  It's up to you to figure out how to bypass the

protection.  This can be quite easy, sometimes only requiring a branch

around the code doing the protection verification, or it can involve

hours of single-stepping through the program for really devious

protection schemes.  Either way, INT13 should speed up things.

V.  Caveats

Since INT13 functions under DOS's control, this utility will not

always function with some game programs which have their own operating

system on the disk.

INT13 also doesn't work with Prolok- and Softguard-protected

diskettes because these systems access the diskette adapter directly and

do not use interrupt 13h.

VI.  How INT13 works

When you run INT13, the vector in low memory which normally points

to the ROM BIOS routine for interrupt 13h is changed to point to the

front-end processing contained in INT13.  When your program issues an

interrupt 13h call, control goes first to INT13 and the required

information is stored for later printout.  Control is then passed to the

normal interrupt 13h routine.  When the I/O processing is finished,

INT13 regains control and prints the log data, including the return code

passed back.  Finally, control returns to the calling program.  If you

are interested in further details, examine the source code.

VII.  How some protection schemes attempt to disguise interrupt 13h

calls

If you are attempting to unprotect a program, the usual course of

action is to search for occurences of "CD13", which is machine language

for interrupt 13h.  One way or another, the protection scheme will have

to use this interrupt to check for the special sectors on the disk.  If

you examine a cross section of programs, however, you will find programs

which do not have "CD13" in their machine code, but which clearly are

checking the key diskette for weird sectors.  How is this being done?

There are several techniques which can be used to camouflage the

protection scheme from prying eyes.  I'll describe below two such

techniques I have come across:

a.  The following section of code is equivalent to issuing a INT 13

command to read one sector from: drive A, side 0, track 28h,

sector ffh, and then checking for a status code of 10h:

XXXX:1000 MOV AH,02                  ;read operation

XXXX:1002 MOV AL,01                  ;1 sector to read

XXXX:1004 MOV CH,28                  ;track 28h (40d)

XXXX:1006 MOV CL,FF                  ;sector ffh (255d)

XXXX:1008 MOV DX,0000                ;side 0, drive A

XXXX:100B XOR BX,BX                  ;move 0

XXXX:100D MOV DS,BX                  ;       to DS register

XXXX:100F PUSHF                      ;push the flags onto stack

XXXX:1010 PUSH CS                    ;push the CS register

XXXX:1011 CALL 1100                  ;push addr. of next instr.

; onto stack and branch

XXXX:1014 CMP AH,10                  ;check for CRC error

XXXX:1017 rest of verification code

.

.

.

XXXX:1100 PUSHF                      ;push the flags onto stack

XXXX:1101 MOV BX,004C                ;addr. of int 13h vector

XXXX:1104 PUSH [BX+02]               ;push CS of int13h routine

XXXX:1107 PUSH [BX]                  ;push IP of int13h routine

XXXX:1109 IRET                       ;pop IP,CS and flags

Notice that there is no "INT 13" command in the source code, so

if you had simply used DEBUG to search for "CD13" in the machine

code, you would never have found the protection routine.

b.  Another technique is to put in a substitute interrupt instruc-

tion (such as INT 10, which looks harmless enough), and have the

program change the "10" to "13".  A search for "CD13" would turn

up nothing.