RSTS/E PERFORMANCE

AND OPTIMIZATION

Martin Minow

Software Services

Digital Equipment Corporation

Maynard, Mass.

June 1978

This document  discusses  RSTS/E  system  performance.   It

includes  information  on  the design of the job scheduler,

facilities  for  obtaining  performance   statistics,   and

documentation  of  PEEK sequences for obtaining information

stored within monitor tables.

PAGE 2

The information in  this  document  is  subject  to  change

without  notice and should not be construed as a commitment

by  Digital  Equipment  Corporation.    Digital   Equipment

Corporation  assumes  no responsibility for any errors that

may appear in this document.

The software described in this document is furnished  under

a license and may be used or copied only in accordance with

the terms of such license.

Digital Equipment Corporation assumes no responsibility for

the use or reliability of its software on equipment that is

not supplied by DIGITAL.

Copyright (C) 1978 by Digital Equipment Corporation

Maynard, MA, U.S.A.

RSTS/E Performance and Optimization                     PAGE 3

PREFACE

This document should be entitled "everything you always wanted  to  know  about

RSTS/E  performance  but were afraid to ask." When you have read through it for

the first (and second time), you will undoubtedly have a good understanding  of

how sensible your fears indeed were.

The information here is the result of a  considerable  body  of  knowledge  and

experience   transmitted   to   the   author  by  a  great  number  of  people.

Unfortunately, it is mixed in with predudice, rumour and a variety of old wives

tales.  It is for you, the reader, to determine what is true and what is not by

using what is presented here to extend your own knowledge of RSTS/E.   Finally,

it  is  your  responsiblity to transmit your knowledge to others so that future

versions of this document will be more usable by the RSTS/E community.   "Share

RSTS/E  with  a  friend"  is not a just a slogan -- it is for many an important

part of their professional work.

When writing this document, it was considered essential to get the  information

down  on  paper.   Form,  structure,  and  readability  were  lesser  concerns.

Unfortunately, accuracy was also a lesser concern.

The lack of an overall organization  means  that  information  is  repeated  in

several  places, but not necessarily present where it is needed.  It also means

that information needed to understand one part of the document may be  in  some

other   section,   or  some  other  manual,  or  non-existant.   This  document

presupposes extensive knowledge of the RSTS/E operating system and  its  formal

documentation (especially the Programmer's Manual).

The information here has not been extensively checked for accuracy.   (This  is

especially  true  in  Chapter  5, PEEK Sequences.) Throughout the document, the

reader will see many suggestions for  system  optimization.   These  should  be

taken  as  suggestions  and  the  reader should verify that they work (or don't

work) before accepting them as valid.  Above all, it must  be  remembered  that

all  installations  are  different.   Techniques  that improve performance on a

CPU-bound system may well slow down a system that is I/O-bound.

What follows is a sermon, it is not a Gospel.

RSTS/E Performance and Optimization                     PAGE 4

CHAPTER 1

RSTS/E System Performance Optimization

This document discusses the performance  of  RSTS/E  systems  and  describes  a

number  of  techniques  for "tuning" systems.  The suggestions in this document

must not be applied blindly.  If you want to improve  a  system's  performance,

you  must  first  understand  what that performance currently is, and what your

goals are.  In almost all cases, the techniques discussed here will yield -- at

most  --  only  slight  improvement.   Major  improvement  can  only be made by

modifying your application programs.  I.e., don't say "My system does  20  disk

accesses per second.  How can I tune it to do 25?" Say instead, "My system does

20 disk accesses per second.  How can I improve my application to require  only

15?"

This chapter is, in its current form, little more than a collection  of  facts,

opinions,  and  fairly educated guesswork.  Unfortunately, things are all mixed

together, for which apologies are in order.  The author feels, however, that it

is  more  important  to  get the information distributed first, and see what is

true later.

It is sincerely hoped that this document will be edited  and  extended  into  a

true  optimization  document which will give RSTS/E users guidance in improving

the performance of existing systems, as well as aid in the initial  design  and

configuration of new systems.

RSTS/E has considerable performance.  While much of the performance is designed

into  the  system  in  such  as  manner  as  to  be  totally transparent to the

programmer and terminal user, there are a number of things that can be done  to

enhance  performance  which  are  documented throughout this document.  Also, a

number of things that should not  be  done  if  the  system  is  to  have  high

performance will also be discussed.

NOTE

During the  development  of  this  document,  the

author   and   others  wrote  down  a  number  of

questions to be  answered.   Until  the  document

reaches a more stable form, the questions will be

RSTS/E System Performance Optimization                                 PAGE 1-2

included  (along  with   whatever   answers   are

available).

1.1  Memory Disposition

Memory in RSTS/E is used for the  monitor  (including  Run-Time  systems),  the

extended  buffer  pool (XBUF), and for User programs being run.  Since programs

must be in memory to execute, proper management of memory is  the  single  most

important task facing the system optimizer.

1.1.1  Monitor Size

Device configuration (don't configure everything if  you  don't

need  it).   Small  Buffer allocation and usage.  What parts of

the monitor to make resident (and  the  trade-offs).   INIT.SYS

and  small  buffers.   Making  enough  space available for user

programs.(Information from the  configuration  article.)  Space

versus performance tradeoffs?  Fip disk reads trade off against

monitor size.  Note cache  effect  on  small  buffers.   Memory

layout with fences.

When the monitor is configured during System Generation, it is possible to vary

the  size of the monitor by selecting optional monitor features and by defining

the size of the system being generated.

1.1.1.1  Monitor Memory Usage -

This section briefly describes the way in which the  RSTS/E  monitor  uses  the

PDP-11  memory  management hardware.  The presentation is only intended to give

the reader an understanding of the basis for some of the system's  limitations.

Only monitor mapping is discussed.  None of this is visible to User programs.

The PDP-11  contains  hardware  to  map  physical  memory  into  eight  logical

segments.    The   monitor   dynamically   controls  the  logical  to  physical

correspondence during processing.  Because only  eight  mapping  registers  are

available,   their   use  is  standardized  within  the  RSTS/E  system.   This

standardization is one of the main contributing factors to RSTS/E's efficiency.

It is also the source of several configuration limitations.

The first five registers (0-4) are permanently mapped.   These  define  dynamic

system  information  (device  control  tables, certain I/O buffers, user status

RSTS/E System Performance Optimization                                 PAGE 1-3

information, etc.) and central monitor code.  The limitation on the  number  of

registers  limits  the  size  of  the  permanently mapped portion of the RSTS/E

monitor to 20 K-words (less one 256 word disk block used  as  a  buffer  during

system  startup).  If too many I/O devices and/or small buffers are configured,

the monitor will grow beyond the 20 K-word limit and must be  regenerated  with

something left out.

Register  5 (20-24 K-words)  is  dynamically  mapped  among   various   monitor

processes -- device drivers, monitor service routines, resident file processing

routines, DECnet, and/or the 2780 emulator.  This is done to cut  down  on  the

amount  of  code that must be permanently mapped, thus increasing the amount of

buffer space available.

Register 6 (24-28 K-words) is a dynamically mapped read-write register used  by

monitor  routines  to  access  information  in  user  programs or to access the

extended buffer pool XBUF.  (This use of XBUF further minimizes  usage  of  the

permanently-mapped  small  buffer pool).  For example, if a program prints data

on a terminal, the terminal service (temporarily mapped using Register 5) moves

data  from  the  user I/O buffer (temporarily mapped using Register 6) into the

small buffer pool (permanently mapped).  It then starts  the  transfer  on  the

device.   The  user job may then be swapped out while the device transfer takes

place.

Register 7 is normally mapped to the PDP-11 hardware address space in order  to

control  periferial  devices.   There are some special cases whereby register 7

maps "somewhere else" in order to move data efficiently between a user  program

area and a data buffer.

The above is not intended to make the reader a PDP-11 hardware expert,  nor  an

expert  on  RSTS/E  internals.   It  is  presented  only  to  give a superfical

understanding of the space limitations inherent in the operating system, and of

the  effort  that  has  gone into increasing the amount of dynamic buffer space

available for use by timesharing programs without  unduly  comprimizing  system

throughput.

1.1.1.2  Optional Monitor Functionality -

The monitor can be configured to  include  or  exclude  several  device  driver

options  as  well  as  optional file processing functionality.  In general, the

tradeoffs are space versus monitor size:

o  The terminal service may be configured to  include  optional  functionality

such  as  multi-terminal  service  and  echo control.  If these modules are

included, they will increase the size of the terminal  service  code  which

decreases  the  amount  of  main  memory  available  to  User programs.  In

addition, additional permanently mapped control information will be  needed

for each terminal line.  This adds up if many terminals are configured.  If

RSTS/E System Performance Optimization                                 PAGE 1-4

IBM 2741 support is  needed,  the  installation  should  configure  as  few

conversion tables as possible.

o  Device drivers for disks may be configured for overlapped or non-overlapped

seek   processing.   Overlapped  seeks  will  give  considerable  disk  I/O

performance improvements (on the order of 30%  better  than  non-overlapped

seek  drivers).   They  are  also  slightly  larger.   Unless the system is

exceptionally tight for space, it should use overlapped seek drivers.

o  The file processor contains considerable code for  processing  disk  files.

If  this code is made permanently main-memory resident, the system will run

much faster.  This gain of speed must, however, be balanced against  memory

size   requirements.    In   general,  systems  should  make  disk  service

permanently  resident  if  the  two   largest   programs   may   still   be

simultaneously main memory resident (as will be discussed below).

If disk file service is non-resident, it must be loaded  from  the  monitor

overlay  file  whenever a file is opened or extended.  In addition, overlay

segments are needed whenever the directory must be accessed when processing

an  open  file.   This  will put a large "overlay" disk load on the system.

The load may, in some cases, require one disk access for  every  User  disk                                                              _____

access.

o  When the system is generated, certain other system  services  may  be  made

permanently  resident.  This should be done if the system has enough memory

or if the functionality will be used often.  These  services  are  slightly

less  important  than  disk  service.  Note that code to process attributes

(which is optionally permanently resident) is needed whenever an  RMS  file

is opened.

o  The amount of dynamic buffering is also specified during system generation.

The  maximum  (generally  over  600 16-word small buffers) is perhaps a bit

small for full-blown (63 user) systems.  Given the  design  of  the  PDP-11

processor, it is impossible to squeeze in many more buffers.  (Note that 64

small buffers equals 1 K-word.)

The actual number of small buffers on a system is  generally  greater  than

the  number  specified  when  the system is generated.  This is because the

number is rounded up so that the small  buffer  area  ends  at  a  1 K-word

boundary.   Thus,  specifing  a slightly different number of buffers during

system generation may have no effect on the actual number configured.

On most systems, the default answer will prove satisfactory.  Systems  with

many high Baud-rate terminals or fast lineprinters may want to increase the

value somewhat.  Note that one 9600 Baud terminal can empty  (or  fill)  30

small  buffers  per  second.   If  this  is multipled by 127 simultaneously

active terminals,...

o  Finally, note that RSTS/E systems may be configured to include support  for

RSTS/E System Performance Optimization                                 PAGE 1-5

hardware (or users) that is not currently needed.  This just wastes space.

o  "If you don't use it, don't configure it."

All RSTS/E systems are generated to include code for all PDP-11  processors  on

which  RSTS/E  may  be run.  When timesharing starts, the system initialization

program (INIT.SYS) determines the specific hardware configuration  (which  CPU,

etc.)  and  selects specific code modules for that configuration.  For example,

all RSTS/E systems contain floating-point exception processors  for  the  11/40

FIS  unit  as  well  as for the 11/34 (etc.) FPU unit.  A given system may have

either of these (or it may have none).  When the system is started,  the  space

used  by  the  unneeded  exception processor(s) is merged into the small buffer

pool.  Thus, the number of small buffers actually on the system may be slightly

greater (or less) that that generated.

1.1.2  Run-Time System Size

Locking in memory (from which  end).   Naming  TKB.TSK  against

BP2COM.

A Run-Time system is a segment of code that may be  shared  among  many  users,

thus  lowering the amount of memory needed for adequate system performance.  It

is generally a good idea to make heavily-used Run-Time systems permanently main

memory resident (assuming enough space is available).  [More?]

1.1.3  User Program Area

Small system:  get two (three if possible) biggest programs  in

memory.   Know  when to lock in memory.  Watch out about memory

fragmentation.  "Bob Spence" memory interlock?

Once the Monitor and Run-Time  systems  have  been  configured,  the  only  (!)

problem  remaining  is determining the proper balance between user programs and

the extended buffer pool XBUF.  Unfortunately, this is a difficult topic.

The best advice is to try different combinations to determine the best  balance

for each specific installation.  It is hoped that the following guidelines will

help:

o  Above all else, try to have enough User memory to  allow  the  two  largest

jobs   to   be  main  memory  resident  sumultaneously.   This  allows  one

"partition" to execute while the other  swaps.   Exceptions  to  this  rule

should  only be made for systems in which large programs are seldom needed.

Managers of such systems should see to it  that  such  large  programs  are

indeed seldom used.

RSTS/E System Performance Optimization                                 PAGE 1-6

o  The manager should next strive to have enough memory to  make  three  large

programs  simultaneously  main  memory  resident.  If this is done, one can

execute, one swap, and the third perform some other I/O operation.

o  If more memory is available, a balance must be struck between monitor size,

XBUF  size  and user memory size.  (In fact, the manager may well choose to

increase the size of XBUF instead of allowing  a  third  large  program  to

become simultaneously main memory resident.) The manager should be aware of

the average user program size in order to establish a swap  ratio  for  the

system.   This is the ratio of the number of programs which are main memory

resident to the total number of programs which are active on the system.

The proper swap ratio depends  on  the  particular  hardware  configuration

(especially  on  whether a seperate fixed-head "swapping" disk is avaiable)

and on the type of processing done by the system.  If no seperate  swapping

disk  is  available,  swap ratios of 2.5:1 or 3:1 seem appropriate.  A swap

ratio of 3:1 means that there is sufficient main memory available for  user

programs  that  one-third of the programs may be simultaneously main memory

resident.  If a seperate swapping disk is available, swap ratios of 3:1  or

4:1 are generally recommended.

Note that these recommendations are very much "seat of the pants" guesswork

and  are heavily dependent on the particular processing done by the system.             _______

Classical timesharing systems in which terminal users spend most  of  their

time  deciding what to do next can get by with relatively high swap ratios,

while systems which are I/O bound (or which  have  professional  data-entry

personnel)require  lower  swap  ratios  for satisfactory performance.  Note

that, if the system is already I/O bound, decreasing the amount of swapping

makes  more device time available for other disk processing.  It also means

that there is a greater chance for some program in memory to be able to use

the CPU while other programs access disk files.

o  The system manager has one other technique available that may  --  in  some

cases  -- improve performance.  Unfortunately, it is a two-edged sword:  it

can make the system quite unusable if misapplied.  This technique  is  that

of  explicitly  locking  user programs in main memory.  For example, if the

system has exactly one COBOL program which is 28 K-words  long,  the  total               _______

running  time  of  this  program  will  be  shortened by preventing it from

swapping.  (Swapping 28 K-words in and out requires  about  1/3  second  on

most disks.  During most of this time, no other program can access any disk

of the same type as the one on which the swap is taking place.)

RSTS/E does not allow specification of the main memory address at  which  a

job  is  locked.  Thus, on systems with limited amounts of main memory, the

COBOL job can lock itself in the middle of memory so that no other  job  on

the  system can run.  The monitor will not prevent this from happening, nor

will it take "evasive action" if it does occur.  The net result is that the

COBOL job runs to completion while all other jobs on the system wait for it

to relinquish main memory.

RSTS/E System Performance Optimization                                 PAGE 1-7

1.1.4  XBUF Size

32K per spindle.  10-15% of physical memory.  Effect of  having

less  than  8  K of cache.  Use of cache by 2780, DECNET, local

send-receive.  Effect of XBUF size on  CPU  performance.   XBUF

useless if you have no directory I/O.

The amount of main memory available to user programs must be  balanced  against

the amount dedicated to the extended buffer pool, XBUF.  XBUF is used as a data

buffer by DECnet and the 2780 emulator.  It is also used as  a  buffer  by  the

local  Send-Recieve  monitor  function.  Its primary use, however, is to retain

directory information in main  memory.   This  gives  a  noticable  performance

improvement since, on most system, the system must access the directory (about)

every other User data access.   By  retaining  directory  information  in  main

memory,  most  --  but  not  all  --  directory processing need not require any

physical disk transfers.

Note that this is an optional feature of RSTS/E and is  included  by  answering

"yes" to the "FIP-buffering" question during System Generation.  It is strongly

recommended that FIP-buffering should be included  if  enough  main  memory  is

available   that   the   two  largest  programs  can  be  main-memory  resident

simultaneously.

When the system is started, the INIT START  or  DEFAULT  options  are  used  to

define  the  size  and  location  of XBUF.  If the manager configures less than

8 K-words of XBUF,  the  directory  cache  code  will  attempt  to  use  unused

permanently-mapped temporary storage (Small Buffers) for directory information.

Note, however, that small  buffers  used  for  directory  information  will  be

allocated to "normal" buffering whenever needed.

Four 256 word disk directory blocks may be stored in one K-word  of  XBUF.   An

additional  16 word  block is also needed to retain information about each disk

block stored.  The following formula may  be  used  to  determine  the  maximum

number of directory blocks that may be stored in XBUF:

MAX.DISK.BLOCKS.IN.CACHE = INT(K.WORDS.OF.CACHE * (3 + 15/16))

It is recommended that the cache be made as large as possible (consistent  with

other  system  usage).   One  "seat of the pants" formula that has been used to

estimate XBUF size for relatively large (11/70  with  512 K-byte  main  memory)

systems  is  to  allocate  10-15%  of main memory to XBUF.  Smaller systems may

instead prefer to allocate some percentage of User memory (perhaps  20%).   The

"best"  cache size can only be determined by experimenting with different sizes

and their affect on total system throughtput.

Different types of systems will have different  patterns  of  directory  usage,

thus  requiring  different  cache  size  allocations.   For  example,  a stable

production system which does almost no file  processing  (and  which  has  only

contiguous files) does not need a large cache.  On the other hand, an open-shop

RSTS/E System Performance Optimization                                 PAGE 1-8

timesharing system will need a relatively large cache  as  the  user  community

will tend to do a large amount of file processing (in an unpredictable manner).

The number of disk drives (and their sizes) is also  important  in  determining

XBUF  size.   If  the  drives  are  heavily used, cutting down on the number of

physical disk accesses will improve performance.  Also,  large  disks  tend  to

have  more  files than small disks -- another reason for increasing the size of

the cache.

Finally, note that XBUF is also needed for other system  processing.   If  many

local or DEC-net messages are being processed, XBUF should be relatively larger

in size.  When a job is declared as a message receiver, the maximum  number  of

messages  that  may  be  buffered  should be set as low as possible, to avoid a

program that processes messages slower than they are  sent  from  filling  XBUF

with messages in transit.

On relatively large systems, the system  manager  should  periodically  monitor

XBUF  usage  by  use  of  the  STATUS program and system display programs.  The

directory cache "hit rate" should be monitored  to  determine  that  sufficient

XBUF  is  allocated to attain a reasonable reduction in physical disk accesses.

Also, the amount of XBUF allocated to message processing should be monitored so

that  it  is as low as possible, consistent with other system throughput goals.

Note also that directory cache processing consumes CPU time.  This  is  counter

productive if the system is CPU-bound, rather than disk-bound.

1.2  Disk Optimization

The vast majority of RSTS/E systems  are  disk-bound,  rather  than  CPU-bound.

Improvements to the way in which the disk is used thus have immediate affect on

the overall throughput of a system.

The proper number and type of disks should be included on a system when  it  is

designed.  Some factors to be aware of include:

o  RSTS/E supports overlapped seeks on all disks with the requisite  hardware.

This  means that the move the heads on several drives at the same time.  As

was noted above, this will  yield  a  noticable  performance  gain  over  a

non-overlapped  (or  single-drive) system.  If a second disk drive is added

to a disk-bound single-drive system (and the  files  allocated  to  balance

disk loads), a doubling of system throughput is not unlikely.

o  In addition to adding drives, the system manager  should  carefully  select

the  types of disks being used.  Many systems will benefit from addition of

a fixed-head swapping disk.  On small systems,  this  will  eliminate  many

high-priority  physical disk accesses (for swapping) from the disk used for

file storage.  On large systems -- which do  not  swap  very  much  --  the

fixed-head  disk  has  been  successfully  used to store small heavily used

RSTS/E System Performance Optimization                                 PAGE 1-9

files such as transaction buffers, terminal  operator  forms,  and  program

overlay  files.  On a very large system, moving a few heavily-used files to

a fixed-head disk lead to a 5% improvement in the total system performance.

o  While most RSTS/E systems are disk bound, very large  systems  tend  to  be

CPU-bound  as  there  is  generally  enough memory available that some main

memory resident job is runnable.  On such systems, the only way to  improve

performance   (without   redesigning  applications  or  purchasing  another

machine) is -- paradoxically -- to increase the number of disk accesses.

For  example,  if  a  system  consists  of  a  large  number  of   programs

communicating  with  each  other using the Send-Receive facility of RSTS/E,

the application could be redesigned so that the message is written  into  a

work-buffer  (on  a  fixed-head  disk)  and a small message, containing the

buffer location, transmitted to the other program.  The receiver then reads

the data from the work buffer.  This adds two disk accesses (which can take

place in parallel with  other  processing),  but  it  also  eliminates  the

physical  movement  of  data from one program to XBUF, and from XBUF to the

other program.  Eliminating buffer makes more CPU power available to  other

user processes.

o  When several disk units (and/or types)  are  available  on  a  system,  the

manager  must  correctly allocate Public and Private storage.  It is highly                                                                         ______

recommended that as little public storage be avaliable as possible.  Except

for very small (RK05/RL01-based) systems, only one or -- at the most -- two

private packs are necessary.   All  other  files  should  be  allocated  on

private  volumes.  This not only helps attain better balance between use of

the various disk units, it also eliminates directory overhead.

When a file is created  on  the  public  structure,  all  volumes  of  that                                                         ___

structure  must  be searched to delete any currently-allocated files of the

same name.  This increases the number of physical disk accesses,  increases

the amount of data needed in XBUF, and requires more file-processor time to

create the file.  All of this not only delays the  requesting  program,  it

also delays any other program on the system requiring file service.

1.2.1  Proper Clustersize

Important even if files are contiguous  to  minimize  directory

size.  Note also pack, MFD, and UFD clustersize effects.

The RSTS/E file system allocates blocks to  a  file  (or  directory)  in  units

(called  "clusters") which may be greater than one block.  This improves system

performance in several respects:

o  A larger clustersize means that each directory retrieval entry describes  a

RSTS/E System Performance Optimization                                PAGE 1-10

larger  part of the data file.  This means that less directory I/O overhead

is needed to access the data file.

o  Allocating several disk  blocks  at  at  time  lowers  the  file  processor

overhead  (both in terms of CPU-time and in terms of the number of physical

disk accesses needed) to create or extend a file.

o  Allocating several disk blocks at a time means that the disk blocks will be

located  contiguously on the disk, which in turn should lower the amount of

time to move the disk heads to any part of the cluster.

There are also a few disadvantages to allocating large clusters:

o  If the disk is very full, it may be impossible to find the necessary number

of contiguous disk blocks to satisfy a cluster request.

o  The data file may require fewer blocks than  are  allocated,  thus  wasting

space.   The worst case would be a file which is one block long stored with

a clustersize of 256, which wastes 255 blocks.

Allocating files  with  a  sufficiently  large  clustersize  will  improve  the

throughput  of  any  RSTS/E  system.   It requires some attention by the system

manager  and  application  programs,  but  requires  no  knowledge  of   system

internals, statistics packages, etc.

Note that RSTS/E V6C allows allocating files which occupy contiguous space on a

disk.   This  will eliminate all directory overhead once the file is created or                             ___

opened.  Note, however, that the file cannot be extended after it  is  created.

Such  files  should  still  be  created with a sufficently large clustersize to

minimize the amount of directory information needed to describe the file.

While file clustersizes are most important, the system manager should  also  be

aware of the need to optimize pack and file-directory clustersizes.

o  When a pack is  initialized  for  use  under  RSTS/E,  the  system  manager

specifies  the  pack  clustersize.   This  must  be  at least as great as a

minimum which depends on the actual size of the disk unit.  This  specifies

a  minimum  value  for all file and directory allocation -- including files

explicitly created at particular clustersizes.  Allocating a  pack  with  a

large  clustersize minimizes the size of the storage allocation table (SAT)

which tends to improve system throughput when creating or extending  files.

It   has  the  disadvantage  of  potentially  wasting  space  as  the  pack

clustersize is the minimum by which a file may be extended.

o  File directories are  also  created  at  specific  clustersizes.   A  large

clustersize  increases  the  potential number of files (or accounts for the

MFD) that may be stored in the  directory.   Since  the  maximum  directory

clustersize  is  16,  it  is recommended that all directories on large-disk

systems (rk06/07, rp04/05/06, rm02/03) be created with the maximum value.

RSTS/E System Performance Optimization                                PAGE 1-11

1.2.2  Contiguous Files

Fixed-length   (permanent)   files   should   be    contiguous.

Drawbacks?  Extension impossible before V7A.

The current relase of RSTS/E allows creating contigous files.  As no  directory

overhead is needed to access any part of a contiguous file (as the system knows

the location of the start of the file, and all other  blocks  may  be  computed

from the start), proper use of contiguous file is an essential part of any plan

to improve RSTS/E performance.

ALL PERMANENT FILES WHOSE LENGTH IS KNOWN                   ___ _________ _____ _____ ______ __ _____

SHOULD BE MADE CONTIGUOUS.                          ______ __ ____ ___________

There are no performance disadvantages to contiguous  files.   The  only  known

problems are as follows:

o  Contigous files cannot be extended, they must be recreated  if  their  size

must be increased.  This can be a problem if the initial allocation was not

done correctly.

o  If the disk is full, (or free space is scattered throughout the  disk),  it

may  be impossible to locate sufficient contigous free space.  If this is a

problem, a new disk must be obtained and  the  contiguous  files  allocated

first.

o  Modern high-density disks attain their high density  by  use  of  redundant

recording  techniques  which  automatically correct "soft" errors.  Thus, a

file may be usable even though a particular part of the file has a bad (but

not too bad) block.  However, whenever the particular block is accessed, an

"ECC correctable" error will be logged, which will lower throughput (1)  in

order  to  fix  the error and generate error logging information within the

monitor and (2) by causing  the  error  copy  program  to  run  at  a  high

priority.   An  especially disasterous case in which this problem occurs is

where a soft error is present in a swap file.  Whenever a job swaps out  of

the  bad  area,  an  error will be logged.  (If the error log program swaps

into the bad area, it's all over!) If a heavily used file develops  a  soft

error,  disk files will have to be reallocated.  As disks grow even larger,

the problem becomes more acute.

1.2.3  Account Management

Number of accounts.  Handling 5000 users.  MFD/UFD clustersize.

Pre-extend  UFD  to make it contiguous.  Better have a huge MFD

clustersize if storing files in [1,1].

RSTS/E System Performance Optimization                                PAGE 1-12

RSTS/E  allows  great  flexibility  in  account   allocation,   including   the

establishment  of  account groups, and group libraries.  This flexibility often

tempts the system manager to allocate more accounts than are needed.

On the other hand, it is possible to  improve  system  performance  by  keeping

transitory  data  (files  which  are created and deleted during timesharing) in

accounts which are seperate from permanent data (files which are not deleted or

extended).

Furthermore, system performance  will  be  enhanced  by  keeping  a  reasonable

balance  between  the number of files in an account and the number of accounts.

This is especially true on smaller systems that do not need many  accounts  for

billing  purposes.  For example, instead of having 300 files in one account, it

is better to have 10 files in 30 accounts as fewer physical disk accesses  will

be needed to find any particular file.

The maximum number of user accounts that can be  defined  on  a  RSTS/E  system

depends  on the clustersize of the master file directory (accont [1,1]), and on

the number of data files stored in account [1,1].  If many accounts are  needed

on  a system for security or billing purposes, the manager may have to define a

more complex accounting structure than is standard on RSTS/E.

As an example of how a more complex system might be defined, consider a  school

with a large number of users (perhaps as many as 5000), each of whom has only a

few files.  When defining the system, the system manager allocates one  account

per  terminal  and  a  few accounts for data storage.  Each student would login

with his or her student number or name.  When the name has been  verified  (not

using  the  RSTS/E  LOGIN SYS function call), the LOGIN program would assign an

account determined by the terminal number.  In order  to  access  files  stored

on-line, the student would use a privileged library program that accesses files

stored in the data storage account.  The privileged library program would  also

be  executed  when  the  student logs off to save files in the work area.  Note

that the storage area can easly grow out of porportion:  if  each  student  may

store 100 blocks of data, and there are potentially 5000 students, at least two

RP06 disks will be needed to store all student files.

1.2.4  Reorganization of the Disk

Don't let the disk get full.  Use REORDR (effect  re  "link  to

top"?).   Note  that  REORDR  changes  directory ordering only.

Full backup/restore to clean up entire disk.

RSTS/E does not require files to be pre-defined by the system manager, nor must

files be created at their maximum length.  By allowing full flexibility in file

management, RSTS/E unfortunately allows the lazy manager to  ignore  what  this

flexibility  is  doing to the system's performance.  As disk files are created,

extended, and deleted on line (and the disk slowly fills up), disk clusters are

RSTS/E System Performance Optimization                                PAGE 1-13

allocated  to  new  files from random parts of the disk.  Thus, to read a file,

the head assembly of a moving-head  disk  must  move  over  a  relatively  long

distance.  This takes more time than if all parts of the file were allocated on

contiguous areas of the disk.

The system manager should make regular use of  two  facilities  of  the  RSTS/E

system to reorganize the disk:

o  The system backup utility may be used to copy an entire RSTS/E disk  to  an

archive  medium.   If  the  archive copy is restored (to a clean disk), all

files will be ordered in an optimal  manner.   (This  is  why  the  RESTORE

facility of the BACKUP package is slower than than the BACKUP facility.)

It is recommended that a full backup and restore should be done whenever  a

disk  becomes  full.   The system manager should also perform a full backup

and restore at some interval dependent on the  system  design.   "Classical

timesharing"  systems  generally  requre  more frequent reorganization than

closed-shop production systems.  The need to plan for  disk  reorganization

is another reason why multiple-disk public structures are not recommended.

o  The REORDR.BAS system program may be  used  to  restructure  the  directory

information  in  a particular directory.  It should be used for directories

in which files are continuously being created and  deleted  to  "straighten

out" links between file directory entries.

o  It is possible to configure a disk so that new  files  are  stored  at  the

logical  beginning  of  a directory list.  This improves the performance of

most systems as there is a reasonably high probability that the  last  file

to  be  created  will  be  the next file to be accessed.  If there are many

files in a directory and  the  directory  is  not  reorganized  or  rebuilt

frequently,  the  number  of  physical disk accesses to find any particular

directory entry may be increased.  It is thus recommended  that  REORDR  be

run  more often if the "link to top" option is chosen.  Note that files can

be created at the logical start or  end  of  a  directory,  overriding  the

pack-unique default.

1.2.5  Directory Cacheing

Correct cache size.  What  else?   Advantages/disadvantages  --

CPU overhead.

Most RSTS/E systems -- except those with the bare  minimum  of  available  main

memory  -- will configure the FIP cache facility.  This maintains a main-memory

cache of disk directory information.  When directory information is needed, the

file  processor  checks  whether  the block needed is currently resident in the

cache -- if so, no physical disk access is necessary.  If  the  block  must  be

RSTS/E System Performance Optimization                                PAGE 1-14

read  from  the  disk,  it  may replace a block currently in the cache.  With a

properly sized cache, hit rates of 75% or more  are  easily  obtainable.   This

means  that  3/4  of  all  directory  accesses do not result in a physical disk

access.  As half of all logical disk accesses on a RSTS/E  system  may  be  for

directory information, the reduction of physical disk accesses has an immediate

positive effect on system throughput.

The use of directory cacheing has been discussed  extensively  throughout  this

doucment,  especially  in  terms of XBUF size requirements.  The system manager

should configure enough XBUF so that a reasonable hit-rate is  obtained.   (See

the  discussion  in Chapter 2, Monitor and Disk Statistics for more information

on hit-rates.)

The performance of almost all RSTS/E systems will be improved by proper use  of

directory  cacheing.   The  only pitfalls relate to over-specification of cache

size.  This results in  loss  of  main  memory  for  user  programs  (and  more

swapping)  as well as an increase in CPU time to manage the cache.  In general,

however, the larger the cache, the faster the system.

1.2.6  Swap File Management

How RSTS/E  uses  swap  slots.   Small  SWAP3.SYS  to  soak  up

OPSER/QUEMAN.   Big  SWAP.SYS  just in case.  Placement of swap

files on disk, also  placement  of  SATT,  OVR,  and  ERR,  and

interaction  with  the  location  of directories on the leading

edge of the disk.  Use of fixed-head disk  as  a  file  storage

device.

During timesharing RSTS/E allows specification of  up  to  four  distinct  swap

files.   One of them, SWAP.SYS, must be configured on the system disk, and must

be present during timesharing.  Others may be  dynamically  added  and  removed

without  affecting  timesharing.  This allows the system manager flexibility in

day-to-day operation.  It also permits a certain degree of "fail-soft"  as  the

system can continue to run even if the "swapping disk" develops problems.

The following formula may be used to determine the number of blocks of swapping

space which are needed for a particular combination of JOBMAX and SWAP.MAX:

SWAP.SPACE = JOBMAX * SWAP.MAX * 16

The default swap file need not be as large as the maximum given above, however.

As swapping space is added and removed on-line, the monitor modifies the actual

JOBMAX accordingly.  (SWAPMAX cannot change from  its  start-up  value).   Note

that JOBMAX and SWAPMAX are system-wide values, and are independent of specific

limits which the system manager may  set  using  the  UTILTY  or  PRIOR  system

programs.

RSTS/E System Performance Optimization                                PAGE 1-15

The swap files are numberer SWAP0 to SWAP3  (the  dfault  The  swap  files  are

numbered  SWAP0 to SWAP3 (the default file is equivalent to SWAP2.SYS).  When a

job is to be swapped out, the monitor chooses the lowest numbered swap file for

jobs  which are CPU- or disk-bound, and the highest-numbered swap file for jobs

which are in  sleep-wait  states  (including  message-receive  waits),  or  are

waiting  for terminal input.  Normally, the lowest-numbered swap files would be

allocated to high-speed fixed-head disks, while high-numbered swap slots  would

be allocated to slower moving-head disks.

The system manager may, in fact, want to allocate a relatively small  SWAP3.SYS

on  a  fixed-head  disk  to  speed-up  certain  cyclical  programs, such as the

OPSER/QUEMAN package.  It is also recommended that, if possible, a  large  swap

file  be allocated on a moving-head (even if swapping is normally to fixed-head

disks) so that the system will continue to run even if one  of  the  fixed-head

disks is not working.

Swap files are ususally created using  the  REFRESH  option  to  INIT.   It  is

possible to create swap files "anywhere" on the disk.  Some guidelines for file

creation may, however, be in order:

o  Reallocate the swap file if  it  one  of  the  sectors  develops  a  "soft"

(ECC-correctable)  error.   If  this is not done, many, many errors will be

logged, and the system will run horribly slow.

o  Normally, swap files are allocated in the middle of a moving-head disk,  to

minimize head-movement.  In some cases, however, it may be better to create

a disk so that SATT.SYS, Directories, and Swap files are clustered together

at  the  beginning  of the disk.  This leaves the majority of the disk free

for large, contiguous files.  On a small system, well over 50% of all  disk

processing is for "system" data:  swapping or directory I/O.  By clustering

these files together at one end of the  disk,  head  movment  tends  to  be

minimzed.

o  Systems with a seperate swapping disk which require many  accesses  to  the

overlay  file OVR.SYS, and/or the error text file, ERR.SYS, should strongly

consider allocating these files on fixed-head disks.   As  they  are  quite

small,  they  will  not  seriouslly decrease the number of jobs that may be

swapped on the fixed-head disk, but will cut down  on  contention  for  the

system   disk.    This   is  especially  important  if  the  system  is  so

memory-limited that it cannot afford to include resident disk processing.

o  As has been said several times before, many systems which  do  very  little

swapping  (and much file I/O) may show moderate performance improvements by

using fixed-head disks for heavily-used user files.

RSTS/E System Performance Optimization                                PAGE 1-16

1.3  Character Output Devices

Line printers and terminals impose a heavy load on  the  RSTS/E

monitor because of their character-by-character nature.  Proper

management of these devices should improve system  performance.

Much  of  this  management  must,  however,  be extermal to the

operating system.

1.3.1  Line-printer Optimization

Imposes relatively heavy load on CPU.  Avoid printing too much.

Effect  of  multiple  line  printers  and, especially, multiple

spoolers.  Use of user-taylored print spoolers.  Use  of  LA180

off  of  a  terminal  line.   Note  effect of spooler/QUEMAN on

performance.  Recommend specialized spoolers  "invoice  printer

deamon".

While much effort has gone into the design of the RSTS/E  monitor  to  minimize

the  processor  load  imposed  by high-speed line printers, they still impose a

large load on the monitor.  The  only  solution  is,  of  course,  to  do  less

printing.

Intellegent use of the spooling package can result  in  minimizing  the  system

overhead  associated  with  printing.   Many production systems do not need the

many capabilities of the DEC spooling package and should  consider  writing  an

installation-specific  printing  program.   For  example,  if the system prints

invoices, the "standard" spooling package requires creating a  file  containing

the  invoice  to be printed (file creation takes time, and requires a number of

directory disk accesses), running the QUE program  (or  sending  a  message  to

QUEMAN) which enters the request in the printer queue (much processing overhead

to  verify  request  validity  and  to  manage  the  necessarily-complex  queue

structure,  and  an eventual message send-receive sequence to the LPSPL program

when the file is actually to be printed.  This is an immense overhead for a one

or two block long file.

It would be much better to write an application-specific invoice  printer  that

only  prints  invoices.   The  invoice-creation  program  would  send a message____

containing the invoice number to the printer.  This would be entered in a queue

(which  should  be  on  a  disk file as the printer may go off-line for several

minutes).  The invoice-printer would then format and print the  invoice.   Note

that  there  is  essentially  no  system overhead associated with this processs

other than one message send-receive sequence.  No files are created,  extended,

closed, opened, read, and killed;  nor do programs have to CHAIN to each other.

As the invoice printer does not have the flexibility of the  spooling  package,

it need not be anywhere as large or complex.

RSTS/E System Performance Optimization                                PAGE 1-17

If the standard spooling package is used, the system  manager  should  consider

modifying  it's  idle  cycle  which  executes  every  15  seconds on "standard"

systems.  This cycle checks all spooling jobs (jobs which are online to  OPSER)

to  make  sure  none  of them have gone into hibernation.  On most systems, the

SLEEP can be changed to 60 or 120  seconds  without  affecting  overall  system

reliability.   As  the  OPSER/QUEMAN  programs will run less often (when idle),

they will impose a lighter load on the total system.

(There are a few other utilities, such as the  VT5DPY/VT50PY  display  package,

which will impose a surprisingly heavy load if they are not restrained.)

1.3.2  Terminal Optimization

DZ11 versus DH11.  Baud rate:  Small  output  messages  (<=  30

characters)  ->  9600  Baud;   Long  messages -> 2400 (or less)

Baud.  Message (block) mode input.  Use of  private  delimiter,

echo-control.   Avoidance  of  single-character  input.  Use of

multi-tty service.  Front-end screen formatter:  use of MODE 16

to  get  a  warning  of  modem  hangup.   Don't  let data-entry

operators login.  Use TTY stats.

"9600 Baud theory?"

The major theme of this chapter has been "flexibility costs." This is also true

of terminal usage:

o  RSTS/E supports several types of terminal interfaces, including single-line

interfaces  (DL11),  8-line  multiplexors (DZ11), and 16-line multiplexors.

Each has advantages;  each has disadvantages:

o  Single-line interfaces are least expensive (if only one  or  two  lines

are  needed).   That  is  their  only advantage.  They require the most

amount of CPU power per  character  of  any  terminal-interface.   They

should  only  be  used for extremely small (3-4 terminal) systems which

will not require more terminal connections in the future.

o  DZ11 multiplexors are less expensive per  line  than  the  larger  DH11

interface.   Unfortunately,  they  operate  on a character-by-character

basis, thus requiring more  CPU  power  per  character  than  the  more

expensive  16-line interface.  The larger CPU-load is most noticible if

terminals are connected at high Baud-rates.

o  The DH11 16-line multiplexor can transmit strings of  characters  to  a

terminal  without  interrupting the processor for each character.  (The

design of the RSTS/E monitor limits the maximum length of each block to

30  bytes.)  The  ability to operate without interrupting the CPU makes

more CPU-power available to the user program.  Unfortunately, the price

RSTS/E System Performance Optimization                                PAGE 1-18

per line is higher for the DH11 then for the less efficient DZ11.

If  a  system  is  not  CPU-bound,  the  more  CPU-efficient   terminal

multiplexor will not be cost-effective.

o  RSTS/E allows data  transmission  rates  of  up  to  9600-Baud.   Using

terminals  at  such a high output rate (over 900 characters per second)

will, in many cases, not only lower system throughput,  but  will  make

the system seem unpleasent to terminal operators.

The problem is that  the  operating  system  and  application  programs

require  a certain (small) amount of CPU time to create and output each

character.  When this small amount of time (less than a millisecond) is

multiplied  by  the  large  number  of  characters -- and the number of

terminals -- it is clear that situations can be devised where  the  CPU

just  cannot  create  data  as fast as the terminal(s) can accept data.

Also, the limited amount of permanently-mapped scratch buffering (Small

buffers)  available  serves to further limit terminal I/O bandwidth.  A

very large system with 600 Small Buffers has only enough  buffer  space

for  18000  characters.  While this sounds large, it is only 20 seconds

of data for one terminal.  20 seconds later, another 18000  bytes  must

be  ready  for  output.   If 500 microseconds (one-half millisecond) is

needed to produce and output one character, 45% of the total  CPU-power

available  will  be  used  for terminal I/O.  (And the system will seem

slow.)

In the real world, RSTS/E will in fact stall a user program when  about

200  bytes  of  data  have  been  transmitted  from  the program to the

monitor.  The monitor will then schedule other  programs  to  run.   If

several jobs attempt to output large amounts of data at high Baud rates

at the same time, each will output bursts of about  200  bytes.   Since

200  bytes  is  actually  transmitted  at  9600 Baud  in about 1/5 of a

second, the various displays will exhibit  an  unpleasent  "stuttering"

effect.  Also, if high output rate terminals are present, they may tend

to monopolize the CPU, making less CPU time available for others.  On a

large  11/70  system,  one  or  two  9600-Baud terminals can easily use                               ___  __  ___  _________ _________

50-80% of the available CPU time.

If large amounts of data are to  be  output  to  video  terminals,  the

system manager should strongly consider setting the output Baud rate to

1200 or 2400 to prevent the system loading described above.

On the other hand, if very short messages are being output,  9600  Baud

is  perfectly reasonable, especially with DH11 multiplexors.  If DH11'S

are used, keeping messages shorter  than  30  bytes  will  allow  their

output in one DMA transfer.

o  The RSTS/E monitor may be  configured  to  support  several  additional

software  options,  such  as  multi-terminal  support  and echo contrl.

RSTS/E System Performance Optimization                                PAGE 1-19

These are extremely useful in many  applications.   If  the  system  is

pressed  for memory space, however, they should not be included if they

are not needed.  These options increase the size of the terminal device

control block, which limits the amount of temporary buffers that may be

configured.   In  addition,  the  additional  code  needed  within  the

terminal  service  makes  the monitor (very slightly) larger.  This may

cause problems for very small systems.

Several features of the RSTS/E terminal service aid the system manager  and

applications designer in creating effective systems.  For example:

o  The multi-terminal service option allows one job to control up  to  127

terminals.  (Of course, the number of terminals that can be effectively

controlled by one program or system depends to a great  extent  on  the

application  design  and duty cycle of the terminals.  "Support" is not

identical with "support effectively.") Many applications cannot  afford

the  risk  that  malicious or careless users damage the security of the

system.  RSTS/E  permits  operation  of  the  system  without  allowing

terminal  operators to log into the system -- thus preventing them from

creating and running programs or issuing system commands.

To use this facility, the LOGIN program would be modified to carry  out

necessary   validation   of   prospective   users  (for  example,  only

locally-connected users may log in, others are rejected).   When  LOGIN

decides  that  a terminal should be connected to an application program

(which uses multi-terminal service and runs detached), it performs  the

following steps:

o  ASSIGN the terminal

o  DETACH from the terminal

o  REASSIGN the terminal to the multi-terminal program

o  SEND a message to the multi-terminal program

The multi-terminal program would then be the only program that  "talks"

to  application programs.  This program should open its master keyboard

MODE 16 so as to be fully protected against CTRL/C  and  terminal  line

hangup.    If   a   dial-up   user   hangs   up  the  modem  line,  the

terminal-handler would receive a HANGUP error.  It should then deassign

the  terminal  (so that if somebody dials in again, they do not land in

the middle of another user's process).

RSTS/E System Performance Optimization                                PAGE 1-20

1.4  Summary

The above is a conglomeration of essentially all I know  about  performance

(at  a  detailed  level).  Two articles that have been previously published

are attached as appendecies to this document.  The first  gives  a  general

overview of system performance and system management.  The second discusses

the specific question of RSTS/E system  configuration.   While  they  cover

much the same ground as this chapter, they emphasize user-visible processes

rather than system internals.  If the above was  totally  ununderstandable,

they may offer a useful introduction.