RSTS/E Statistics Collection

RSTS/E Statistics Collection

Author:  Martin Minow

Date:  November 18, 1976

Copyright (C) 1975, 1976

Digital Equipment Corporation

Maynard Mass., 01754, U.S.A.

This software is furnished under a license for use

only on a single computer system and may be copied

only with the inclusion  of  the  above  copyright

notice.    This  software,  or  any  other  copies

thereof, may not be  provided  or  otherwise  made

available  to  any  other person except for use on

such system and to one who agrees to these license

terms.   Title  to  and  ownership of the software

shall at all times remain in DIGITAL.

The information in this  document  is  subject  to

change  without notice and should not be construed

as a commitment by Digital Equipment Corporation.

DIGITAL assumes no responsibility for the  use  or

reliability  of its software on equipment which is

not supplied by DIGITAL.

N O T E

The statistics-collection option is not a supported  feature

of  RSTS/E  systems.   Digital  Equipment Corporation cannot

assume any  responsibility  for  the  correct  operation  of

systems configured with this option.

DIGITAL does not assume any responsibility for the  accuracy

of  data  obtained  by  the  statistics  option, nor for the

interpretation of that data.  It is  strongly  advised  that

users    consult   with   DIGITAL   representatives   before

undertaking any system monitoring operations.

RSTS/E Statistics Collection                                              PAGE 2

The Statistics Collection Option

RSTS/E systems may be configured to  gather  a  variety  of  statistics  on  the

performance of the system.  They fall into three major categories:

Job/CPU operation (collected at each clock interrupt)

Disk operation (recording each disk access)

Directory buffering (recording directory hits/misses)

In order to collect these statistics,  they  must  be  requested  during  system

generation.   This  is  done  by  answering  the "long or short question format"

question by "L/Q" or "S/Q" as described in  Appendix  G  of  the  RSTS/E  System

Generation Manual.

When RSTS/E is configured for statistics, three data buffers  are  allocated  in

the  monitor  area  along  with  code  to collect the data.  The format of these

buffers is described in the listings of RSTS/E and in the appendix.

Collecting the Statistics

The  statistics  package  runs  continuously  during  system   operation.    Job

statistics  are  collected  at  each  clock  interrupt,  while  disk  and  cache

statistics are collected at each disk or directory access.  Console switches  15

and  0 are used to control the collection.  When switch 15 is up, statistics are

collected normally.  When switch 15 is down and switch  0  is  up,  the  current

statistics  are  kept  unchanged  (frozen).   When  both  switches are down, the

statistics buffers are initialized  (cleared).   While  the  Job/CPU  buffer  is

initialized on the next clock interrupt, the disk buffer will not be initialized

until the next disk access occurs.  Thus, if switch 0 is flipped off and on, the

disk  buffer  may  not  have been cleared.  Statistics collection may require as

much as 3-5% of the available CPU time.

Displaying the Statistics

A program, STATS.BAS, has been devised to display the most  important  of  these

statistics  on  a  terminal or in a file.  The program actually collects all the

available Job/CPU statistics, summing disk statistics over all units.   It  can,

of  course,  be  used  as  the  basis  for  other statistics gathering programs.

STATS.BAS must be run by a privileged user if  stored  with  the  default  <124>

protection  code.   It  may  be run by any user if it was stored using the <232>

protection code.

Running STATS.BAS

To run the statistics display program, type  "RUN  STATS".   When  started,  the

program will request a command line containing an output device/file designation

and any optional switches.  To assist in the normal operation of the program, it

is  possible to designate a default command string.  To accept this string, just

type return.

RSTS/E Statistics Collection                                              PAGE 3

The following switches may be given to control the display:

Terminal type switches

/VT05   /VT50   /VT52   /TTY    /SPEC

These switches define the type of terminal.  the  "/SPEC"  switch

simplifies  modification  of  the  software  for  a  non-standard

terminal.

Statistics control switches

/JOB    /DISK   /BOTH   /TOTAL  /INCR

These switches control what statistics are  gathered:   Job/Cache

or  CPU/Disk  only,  or both (the default).  the /TOTAL and /INCR

switches determine whether the statistics are to be presented for

the  entire run (since the buffer was initialized with the switch

register as described above) or for  each  sample  independently.

If  the  switch  register  is  to be used to freeze and clear the

buffer, the /TOTAL switch is preferable.  /INCR is the default.

/SL:nn  /DET

These switches determine  the  sleep  time  (and  thus  the  time

between samples) and whether the program is to run detached.  The

program actually sleeps for a shorter time than is given so as to

refresh  the  display every "nn" seconds.  If /SL:0 is specified,

only one report is printed.  The user must be privileged in order

to detach.

Sample output from STATS.BAS

14-Sep-76 16:02:38              59 Seconds     1 Errors      0 Hung TTY

2:57:38 Uptime 26/63 Jobs,  0.00 Swap in   273 Minimum Small Buffers

System Operation Statistics         Cache/Fip Statistics

Including Idle Time  Excluding Idle Time     17 Monitor Nodes

Charged  Uncharged   Charged Uncharged     920 XBUF Nodes

Idle            55.52%                       24.79% Incremental Hits

Lost             1.19%               2.67%   50.64% Running Sum Hits

User  30.81%               69.26%             0.47% Cache CPU Usage

System   5.56%     6.93%     12.50%   15.58%   15.64% Fip Desired

Total  36.37%    63.63%     81.75%   18.25%   14.46% Fip Waiting

2 KW11P Overruns

CPU Status      Disk Status for all Disks     Xfer Percent

Total    Exec   Process   Block/Sec  Xfer/Sec  Reads   Total

User Job  30.80%          User data     14.71      9.96  98.81%  68.69%

CPU Idle  56.49%          Directory      3.97      3.97  72.65%  27.34%

(Unused)    .00%           Swapping      5.29      0.36  52.38%   2.45%

Monitor    9.05%  71.20%  Other Sys      0.22      0.22  92.31%   1.52%

Slow I/O   0.98%   7.73%    All Sys      9.47      4.54  72.01%  31.31%

Fast I/O   2.68%  21.07%    All I/O     24.18     14.51  90.42% 100.00%

RSTS/E Statistics Collection                                              PAGE 4

The statistics printed are grouped into five sections:

1.   A two line header gives day/date overall status:

The first line records the day and  date,  the  time  over  which  the

statistics  were  collected,  the  number  of  errors  (excluding hung

terminals) that have been recorded, and the number of hung terminals.

The second line records the uptime (hours, minutes, and  seconds)  the

number of active jobs (and the maximum), and a crude estimate of "swap

in time".  This is arrived at by comparing the time the  job  expected

to sleep between collection passes with the actual time.  If large, it

usually means that the job had been swapped out of core memory between

passes.  The minimum number of small buffers is also recorded.

2.   The section headed "System Operation Statistics"  describes  the  work

the  RSTS/E system did during the last sampling interval.  The data is

presented in two sets of four columns each:

Charged time

Charged time is the time actually billed to a specific user while

a  job was running (and doing useful work).  Time is recorded for

the actual User run time,  and  for  monitor  time  that  can  be

charged against a particular user.

Uncharged time

This column records idle time (the system had nothing to do)  and

"lost"  time.   This  latter  is  the  time  that  the system had

selected a job to run, but could not run this job, nor  could  it

run any other in its place.  I.E., no job in memory could be run,

and the job that should run was not  yet  swapped  in.   If  this

value  is reasonably small, it probably means that the system had

a lot of people doing  terminal-bound  processing,  and  the  job

whose  operator  finished  input just happened to be on the disk.

If the value is large, it almost certainly means that the  system

needs more memory and/or a separate swapping disk.

The first two columns record information including totally idle  time.

Idle  time  is  omitted  from the calculations shown in the second two

columns, which may give a truer picture of  actual  system  operation.

In  the  above  example,  the  percentage  of  time that user jobs ran

increased from 31% to 69% when idle time was excluded.

3.   "Cache/Fip Statistics" records the performance of the directory  block

buffering code and the overall usage of the file processor (Fip).

The first two values record the number of monitor  small  buffers

used  for  directory processing and the number of 16-word buffers

being used in the external buffer (XBUF).

The next two values record the directory hit ratio (the number of

times  the  buffer  search code found the particular block in the

buffer divided by the total number of attempts).  The incremental

RSTS/E Statistics Collection                                              PAGE 5

value  records  the performance during the last sample, while the

running sum value records the performance  since  the  statistics

buffer  was  initialized.   The  amount of CPU time needed by the

cache code is also printed.

The two values recorded for Fip processing are the time that some

job  had requested service from Fip (for example, to open a file)

and the time Fip was delayed (for example, while  waiting  for  a

disk  block  or  overlay  segment).   The system records specific

reasons for delays, but STATS.BAS does not print them.

If the hardware has the KW11-P programmable clock, the number  of

times the clock overflowed is recorded if any overflows occurred.

4.   The two columns headed by "CPU Status" record the actual CPU  priority

level  at each sample interval.  In RSTS/E V06B, these levels are used

as follows:

0    User programs.

1    The Null Job (which runs when no other job can run).

2    Not currently used.

3    RSTS/E monitor processing.

4    Terminal and other slow I/O service (paper-tape, etc.).

5    Disk, magtape and other fast I/O service.

The left-hand column records actual priority levels.   The  right-hand

column records only the loading of the three monitor levels.

4.   Disk status is recorded for all disk units on the system.  The monitor

level  code maintains read and write request counts, and the number of

blocks read or written for six request types, or a total of 24 entries

for each unit.  The request types are:

Swaps and run-time system reads

User GET/PUT requests, program loads, or overlay reads

SAT (disk allocation table) requests

Overlay code reads

Directory I/O

DECtape directory processing

This information is compressed  by  grouping  SAT,  Fip  overlay,  and

DECtape  directory  processing into the heading "Other Sys".  The data

are printed in blocks per second and  transfers  per  second.   (Their

ratio  is,  of  course, the average number of blocks per transfer.) In

addition, two columns of percentage information is available:

Reads     The percentage of reads in each type of  transfer.   In

the  sample  printout,  99%  of all user data transfers

were reads.

Total     The contribution of this type of transfer to the  total

amount  of disk processing during this sample interval.

For  example,  in  the  sample  printout,   user   data

accounted for 69% of all disk I/O.

RSTS/E Statistics Collection                                              PAGE 6

Analyzing System Performance

In determining system throughput -- and bottlenecks -- the values  collected  by

RSTS/E  play  a secondary role to knowledge of the system under analysis.  Thus,

it  is  extremely  important  for  application  programs  to   log   performance

information, and for system managers to follow the day by day operation of their

system.  At best, the STATS.BAS printout can isolate a bottleneck;   it  cannot,

however, avoid one.

The following discusssion assumes familiarity with the STATS.BAS printout.   The

order  follows  the  printout,  and  should not be taken as a pattern for system

analysis.

Header Statistics

Number of Errors

If this value starts rising, it  may  indicate  flakey  hardware.

For  example,  the  disk  drive  may  make  several  unsuccessful

attempts before reading data.  While this  may  not  be  directly

noticable,  it,  of  course,  slows the system down.  Each logged

error causes ERRCPY to run at a high priority, thus adding to  to

system load.

Minimum Small Buffers

Small values indicate a serious system bottleneck.  While  a  new

system  generation  may be indicated, the problem may actually be

caused by sending long messages using the Send-Receive  facility.

Increasing  the  size  of the message buffer "XBUF" may alleviate

the problem.  The way in which  messages  are  processed  by  the

application programs should also be analyzed.

System Operation Statisitcs

Idle Time

This value reflects the time the system had nothing  to  do.   If

the  system  was actually clogged, a high value may indicate that

all jobs were stalled for I/O processing.   The  disk  statistics

and  the directory cache statistics should be examined to further

analyze the cause of the unexpectedly high idle time.

Lost time

This value reflects the time spent swapping in a job when no  job

currently  in  memory  was  runnable.   In  general,  high values

indicate a lack of main memory.  If the  system  seems  disk  I/O

bound,  adding  a  separate swapping disk -- or reallocating swap

files on the existing disks --  may  improve  performance.   Lost

time  is  time  that could have been spent profitably.  While low

values are not serious by themselves,  continuously  high  values

should be investigated further.

RSTS/E Statistics Collection                                              PAGE 7

User Time

This is the time spent actually running user programs.  If  high,

and the system still seems slow, the system may be running out of

CPU  power.   Adding  a  hardware  floating-point  processor   or

changing  to  a  faster  processor  may  improve  matters.   More

important, however, is an analysis  of  the  actual  application.

The  system  manager  should determine what the system was doing,

and re-code (or remove) applications  that  are  using  too  much

time.   The  RSTS/E  priority  functions  (or  the DYNPRI dynamic

schedular) may help improve perceived performance.   The  manager

should  also  consider  splitting  the  application  over several

machines if practical.

System Time

This is the time spent in the RSTS/E monitor  or  device  service

routines.   The  System  Operation  Statistics separate this into

Charged and Uncharged time.  High values indicate a system making

extensive  use  of  monitor  facilities.   The  way  in which the

application programs use the monitor should be investigated.

Cache/Fip Statistics

Monitor Nodes

This value gives  the  number  of  small  buffer  nodes  used  by

directory   buffer   cacheing.   These  nodes  are  automatically

returned to the free buffer pool when needed for  normal  monitor

use.   If  the  value  is  high,  a larger XBUF allocation may be

indicated.

XBUF Nodes

This value gives the number of 16 word  buffers  in  use  in  the

Extended  Buffer  pool  "XBUF".   Nodes  are  used  for directory

processing as well as the large message Send-Receive facility.

Incremental Hits

This is the directory cache hit ratio,  indicating  how  often  a

required  directory block was found in main memory.  If small, it

may indicate a small XBUF area, an unorganized directory,  or  an

"unpredictable" system.  Generally, the bigger the better.

Running Sum Hits

This is the directory  cache  hit  ratio  since  the  system  was

started  (or the statistics buffer initialized).  The value shows

the cache performance over a longer period than  the  Incremental

Hit value described above.

Cache CPU Usage

This is the amount of  CPU  time  used  by  the  directory  cache

algorithm.  If large (more than a few per-cent), the cache may be

too large or the system may be unpredictable.

RSTS/E Statistics Collection                                              PAGE 8

Fip Desired

This value indicates how  often  the  file  processing  code  was

needed  by at least one job.  If large, programs may be doing too

much file processing.  Since only one job may be "in" Fip at  any

instant, high values may indicate a serious bottleneck.

Fip Waiting

This value indicates the time Fip was in use, but was waiting for

some  process  to  complete.   The most obvious reason for Fip to

wait is for completion of directory I/O.  Also,  file  processing

might be delayed by the need to fetch overlay modules.  This will

be  indicated  by  the  disk   activity   statistics   as   well.

Regenerating  to include more Fip modules in the resident monitor

may help, as may a larger XBUF area.  STATS.BAS collects specific

reasons for Fip waits (Overlay Code, Disk I/O, Storage Allocation

Table I/O, or "Other") and may easily be  modified  to  log  this

additional information.

KW11P Overruns

Normally, this value will  only  be  seen  on  systems  with  the

programmable  (crystal)  clock  when  it  is run at a rather high

interrupt  rate.   Re-generating  to  decrease  the  rate  should

eliminate  the  problem.  If it persists, there may be a hardware

problem.  Note:  the monitor error logging code runs  with  clock

interrupts  locked  out.   Thus,  numerous errors -- or bursts of

errors -- may lock out the KW11P so often that overruns occur.

CPU Status

As already described, these  values  indicate  the  CPU  priority

levels.  The only interesting information here is the system load

imposed  by  interrupt  service  routines   --   Slow   I/O   for

terminal-type  devices and Fast I/O for disk and magtape devices.

The statistics code cannot record CPU priority  levels  6  (clock

service) and 7 (interrupts locked out).

Disk Status

User data

The values on this line  record  disk  accesses  requested  by  a

terminal  user  or  program.   This  includes  explicit  file I/O

requests (GET and PUT operations) as well  as  implicit  requests

(OLD,  RUN, and CHAIN operations for Basic-Plus and overlay reads

for Fortran, Cobol, Basic-Plus II, etc.).

Large  values  indicate  a  heavily   disk-bound   system.    The

application  should  be  analyzed  to  determine  if  the  I/O is

necessary.  If much chaining is being done, the  application  may

need  to  be  rewritten  to  make  use  of  several  continuously

operating modules that communicate via Send/Receive.

The system manager may also consider upgrading to  faster  disks,

or  spreading  the application over several disks.  If the system

has only one disk (or one disk type) and is doing  much  swapping

in addition to ordinary accesses, additional memory or a separate

RSTS/E Statistics Collection                                              PAGE 9

swapping disk should help.  STATS.BAS  prints  disk  data  summed

over  all  units  and  disk types.  It can be modified to present

data for one particular disk type or specific unit, if necessary.

Directory

These values indicate directory activity that  actually  resulted

in  physical  disk accesses.  If high, the cache areas may be too

small.

Swapping

These values indicate accesses required  for  job  swapping.   If

high,  more main memory may be needed.  Also, a separate swapping

disk may be indicated.  Since RSTS/E uses  multi-block  transfers

to  swap  a  job  in  and  out,  the "Xfer/Sec" value may be more

indicitive of the swapping load than the "Blocks/Sec" value.

Other Sys

These values indicate accesses required to fetch non-resident Fip

modules  as well as error message reads.  If high, the system may

be regenerated to include more Fip modules resident.

All Sys

These values are  the  sum  of  Directory,  Swapping,  and  Other

described above.

All I/O

These values are the sums of  all  I/O  values.   If  this  value

approaches the access rates for the disk hardware, it indicates a

severe throughput  bottleneck.   While  additional  hardware  may

solve the problem, a reanalysis of the application may prove more

successful in the long run.

RSTS/E Statistics Collection                                             PAGE 10

Appendix

Format of Statistics Tables

The following tables describe the layout of the  Statistics  buffers  in  RSTS/E

V06B.  Note the following definitions:

WORD    a 16-bit integer

DOUBLE  a 32-bit integer stored in two 16-bit words

The following PDP-11 code is used to increment DOUBLE entries:

ADD     #1,LOW-WORD

ADC     LOW-WORD+2

The following BASIC-PLUS function returns a DOUBLE  entry  as  a  floating-point

number.  The parameter is the address of the first word of the table entry.

21000 DEF FNP(P%)

! RETURN DOUBLE ENTRY AT P%

21010 Q = PEEK(P%)

\ Q = Q + 65536.0 IF Q < 0.0

\ FNP = Q + (65536.0 * PEEK(P% + 2%))

! GET FIRST WORD AS AN UNSIGNED NUMBER

!THEN ADD TO HIGH-ORDER WORD TO GET RESULT

21020 FNEND

The statistics tables are addressed from a fixed location in the low-core area

of the RSTS/E monitor as follows:

STSTBL := PEEK(156)             Base of statistics pointers

Disk   := PEEK(STSTBL)          Disk  data buffer if non-zero

Job    := PEEK(STSTBL+2)        Job   data buffer if non-zero

Cache  := PEEK(STSTBL+6)        Cache data buffer if non-zero

RSTS/E Statistics Collection                                             PAGE 11

The Job Statistics Table has the following format:

Decimal Offset        Used for

0 WORD    Clock interrupts per second

2 DOUBLE  Uptime

6 WORD    Smallest free buffer count

8 DOUBLE  Level 3 (monitor) accounted

12 DOUBLE  Lost time

16 DOUBLE  Level 3 (monitor) unaccounted

20 DOUBLE  Null time

24 DOUBLE  Fip needed

28 DOUBLE  System mode

32 DOUBLE  Fip waiting

36 DOUBLE  Fip waiting for code

40 DOUBLE  Fip waiting for disk information

44 DOUBLE  Fip waiting for storage-allocation table I/O

48 DOUBLE  Fip waiting for other reasons

52 DOUBLE  CPU priority 0, cache not running

56 DOUBLE  CPU priority 1, cache not running

60 DOUBLE  CPU priority 2, cache not running (not used)

64 DOUBLE  CPU priority 3, cache not running

68 DOUBLE  CPU priority 4, cache not running

72 DOUBLE  CPU priority 5, cache not running

76 WORD    KW11P overruns

78 DOUBLE  CPU priority 0, cache running

82 DOUBLE  CPU priority 1, cache running

86 DOUBLE  CPU priority 2, cache running (not used)

90 DOUBLE  CPU priority 3, cache running

94 DOUBLE  CPU priority 4, cache running

98 DOUBLE  CPU priority 5, cache running

RSTS/E Statistics Collection                                             PAGE 12

The Disk Statistics buffer consists of 2-word entries for each process  logged

(Swaps,  File  I/O,  SAT,  Overlay code, Directory, DECtape buffer).  For each

process, the number of transfers and the number of  blocks  per  transfer  are

logged  for Reads and Writes.  Information is logged for each disk unit on the

system.  This buffer is proceeded by a word entry which has the  total  number

of  disk  units configured on the system.  For example, for a system with only

one disk, the following information is recorded:

Decimal Offset        Used for

0 WORD    Number of units (low byte only)

2 DOUBLE  Swaps   Reading number of transfers

6 DOUBLE  Swaps   Reading number of blocks

10 DOUBLE  Swaps   Writing number of transfers

14 DOUBLE  Swaps   Writing number of blocks

18 DOUBLE  File    Reading number of transfers

22 DOUBLE  File    Reading number of blocks

26 DOUBLE  File    Writing number of transfers

30 DOUBLE  File    Writing number of blocks

34 DOUBLE  SAT     Reading number of transfers

38 DOUBLE  SAT     Reading number of blocks

42 DOUBLE  SAT     Writing number of transfers

46 DOUBLE  SAT     Writing number of blocks

50 DOUBLE  Overlay Reading number of transfers

54 DOUBLE  Overlay Reading number of blocks

58 DOUBLE  Overlay Writing number of transfers

62 DOUBLE  Overlay Writing number of blocks

66 DOUBLE  Direct  Reading number of transfers

70 DOUBLE  Direct  Reading number of blocks

74 DOUBLE  Direct  Writing number of transfers

78 DOUBLE  Direct  Writing number of blocks

82 DOUBLE  Buffer  Reading number of transfers

86 DOUBLE  Buffer  Reading number of blocks

90 DOUBLE  Buffer  Writing number of transfers

94 DOUBLE  Buffer  Writing number of blocks

If there is more than one disk on the system, the data is ordered so that Swap

information  for  all  disks  comes  first,  followed  by  File, SAT, Overlay,

Directory, and  DECtape  buffer  data.   The  DEVTBL  and  DEVCNT  entries  --

available by using the Get Monitor Tables SYS call -- may be used to associate

Statistics table entries with their respective disks.

RSTS/E Statistics Collection                                             PAGE 13

The following function may be used to  access  disk  data.   It  performs  the

addressing  needed and executes FNP (described above) to read the table entry.

The following global data are needed:

B%      Disk buffer base + 2

N%      The total number of disk units:

PEEK(B%) AND 255%

22000   DEF FND(D%,P%,A%)

! RETURN DISK TABLE ENTRY FOR:

! D%    DISK UNIT NUMBER

! P%    PROCESS (SWAP, ETC.)

! A%    ACCESS VALUE (READ/WRITE)

22010   FND=FNP(B% + (P%*N% + D%)*16% + A%*4%)

22020   FNEND

Directory Cache processing statistics are stored as follows:

Decimal Offset        Used for

0 DOUBLE  Total possible number of hits

4 DOUBLE  Total possible number of Fip hits

8 DOUBLE  Total actual number of hits

12 DOUBLE  Total actual number of Fip hits

16 WORD    Number of cache restarts

18 WORD    Cache elements in use in monitor area

20 WORD    Cache elements in use in XBUF area