LSFORM.OBJ	DRIVER FOR CENTRONIX LINE PRINTER

FOR RSTS V4A

THIS PROGRAM, WHEN INCORPORATED AS PART OF THE RESOURCE SHARING

TIME-SHARING SYSTEM (RSTS) MONITOR, MAKES AVAILABLE TO THE USER THE

SPECIAL FEATURES OF THE CENTRONIX LINE PRINTER (DEC DESIGNATION: LS11),

NOTABLY:

A) EXPANDED (DOUBLE WIDTH) PRINTING

B) VERTICAL TABBING OF FORMS

THE PROGRAM IS SUPPLIED ON DECTAPE AS AN OBJECT MODULE (SOURCE LISTING

IS ALSO INCLUDED).  IT MUST BE LINKED INTO THE RSTS MONITOR IN PLACE

OF THE DEC SUPPLIED MODULE LPFORM.OBJ AS FOLLOWS:

$RUN LINK

.

.

.

#DTN:LSFORM		(WHERE N IS THE DECTAPE UNIT CONTAINING

THE MODULE)

TO ENABLE THE LS11'S SPECIAL FEATURES, IT IS NECESSARY TO MAKE A

PATCH TO THE RSTS MONITOR AS FOLLOWS:

HELLO 1,2

RSTS V4A...

PASSWORD:		(TYPE THE ASSIGNED PASSWORD HERE)

NEW OR OLD--RUN ODT

FILE? [0,1]CORE.SYS<LF>(END WITH LINE-FEED, NOT CARRIAGE-RETURN)

* 1001;1R

* 1,NNNNNN\ 052 072<CR>

*^Z			(CTRL/Z COMBINATION)

READY

WHERE NNNNNN IS THE OCTAL NUMBER FROM THE SYSTEM LOAD MAP CORRESPON-

DING TO THE LABEL "LPTDDB".

IT IS THEN NECESSARY TO RELOAD THE RSTS MONITOR FROM DISK (SEE RSTS

SYSTEM MANAGER'S GUIDE, SEC. 3.2.1).