***********************************************************************

*                                                                     *

*                         JOBSDUMP V4.2/4.0                           *

*                                                                     *

*                   A GRAPHICS SCREEN-DUMP PROGRAM                    *

*                  FOR THE DEC RAINBOW 100 PC FAMILY                  *

*                                                                     *

*             Copyright (C) 1985,6,7,8,9 James A. O'Brien             *

*                                                                     *

*                         1166 Hope St., #6                           *

*                         Stamford, CT 06907                          *

*                               U.S.A.                                *

*                                                                     *

*                    Tel: +1 203 322 7222 (Evenings)                  *

*                                                                     *

***********************************************************************

This software may be copied and the copies distributed without

restriction for PRIVATE INDIVIDUAL use.  Copyright is retained by the

author, and the source code will not be distributed.  If you find the

software useful (i.e. you use it!), a $25 donation to the author at the

above address is requested for those users who are private individuals.

In other words, this software is not public domain - it is SHAREWARE.

Commercial site licenses may be obtained by contacting the author

at the above address.  NOTE that this software is NOT FREE FOR USE IN A

COMMERCIAL ESTABLISHMENT, no matter how humble.

Hardware Requirements:  (a) DEC Rainbow with Rainbow Graphics Option (RGO)

and 192k of random access memory (at least 256k

in order to run from within GW-BASIC).  CP/M

version only requires 128k.  [Has been tested

with Suitable Solutions Turbow-286 board]

(b) DEC LA50, LA75, LA100 or LA210

or Epson-compatible dot matrix printer.

Software Requirements:  (a) MS-DOS V2.05 (or later) for MS-DOS version.

(b) CP/M 86/80 V2.0 (or later) for CP/M version.

-------------------------------------------------------------------------------

The complete distribution package includes the files:

JOBSDUMP.EXE          MS-DOS version of JOBSDUMP V4.2

JOBSDUMP.CMD          CP/M 86/80 version of JOBSDUMP V4.0

JOBSDUMP.DOC          This documentation file.

-------------------------------------------------------------------------------

WHAT'S NEW IN VERSION 4.0,4.1 ?

*******************************

a)  It runs FASTER than 3.x and previous versions.

b)  The DEC sixel files are about A FACTOR OF 10 SMALLER than 3.x

and previous versions.

c)  The -e option has been added to the command-line option set.  It

lets you output the graphics in EPSON-compatible (IBM) format.

d)  The -dnn option has been added to the command-line option set.  It

lets you DISPLAY on the screen for nn seconds the graphics that

will go to the printer.

e)  The -n option has been added to the command-line option set.  It

lets you print the image in NEGATIVE mode to save printer

ribbons when a picture has very little white space.

f)  The user interface is a more friendly cursor-driven environment,

reminiscent of DEC-written utilities.  The -g (GO) option has been

added to the command-line option set, to allow command-line user

to avoid the cursor-driven user interface.

g)  The CP/M version is functionally IDENTICAL to the DOS version as of

version 4.0.

h)  The ERROR HANDLING for disk file output has been improved.  The output

now continues until the disk is full, at which time a graceful

exit (accompanied by a message) occurs.

i)  The CP/M version is frozen at V4.0.  Version 4.2 for DOS includes

an option to select the colours which will print as black.  This

is accomplished using a DOS environment variable called JOBSCOLS.

WHAT'S NEW IN VERSION 4.2 ?

***************************

a)  The -l option has been added to the command-line option set.  It

compensates for the aspect ratio of the LA100 printer (different

from that of the LA50 and the Rainbow screen).

b)  The operation of the -e (Epson mode) option has been enhanced, to

give improved pictures with correct aspect ratio.

c)  The speed of the -e (Epson mode) option has been increased by means

of a sixel lookup table.

d)  The menu display now shows more clearly the options in effect at any

given time.

(1) OVERVIEW

************

JOBSDUMP is a utility program designed to give the Rainbow computer

an important ability not included with the system - the ability to print out

graphics screens directly on DEC or Epson-compatible printers.  It prints out

both medium and high resolution graphics screens in either of two

formats :

(a) Small, which is a dot-for-dot representation of the graphics

screen and is smaller than the screen since printer dots are smaller than

screen pixels for the Rainbow computer.

(b) Large, which is a double-size printout rotated 90 degrees

counter-clockwise.  Thus, the right-hand-side of the screen picture is the

first part to arrive at the printer.  Large printouts just about fit on a page.

The program is interactive, prompting for the resolution, the

printout size, the output file, the graphics protocol format (DEC or Epson),

the printing color and the preview option.  At any time, you have the option of

aborting the program by pressing the <EXIT> or the <CANCEL> key.  This is useful

when you are not interested in printing the remainder of the graphics screen

(for example, when it is blank).

Since most printers are monochrome devices, there is no simple way to

distinguish among the various colours or shades of grey visible on the

screen.  There are methods such as the error diffusion technique (see, e.g.

"Digital Halftones by Dot Diffusion", by Donald E. Knuth in ACM Transactions

on graphics 6, 245-273, 1987, and references therein).  They operate by

simulating, in a local average way, grey tones using randomly chosen black dots.

However, these tend to be very demanding in terms of storage space and

floating-point computation. Such techniques are better suited to larger

machines.  The latest MS-DOS and CP/M versions of JOBSDUMP handle this in two

different ways.

**V4.0:

Any colour which differs from black on the screen is

printed out in black on the printer.  When the -n option is in force, everything

except the background is white.

**V4.2:

An environment variable called JOBSCOLS may be used to specify the

colours to print.  e.g.

SET JOBSCOLS=1234ABC

will cause colours 1, 2, 3, 4, 10, 11, 12 (hex notation) to print as black,

while all others will be white.  If JOBSCOLS is not defined, then the behaviour

will revert to that of V4.0 above.

If you're not sure about what to do with JOBSCOLS, ignore it!  It's

not at all necessary in most situations.

In order to give a correctly-scaled representation of the screen

graphics, the printer has to have a 2:1 pixel aspect ratio.  This is no

problem for the LA50, LA75 and (I think) LA210 printers.  However, the

LA100 printer does not support an aspect ratio of 2:1 exactly.  This

caused LA100 printouts not to be proportional to the screen image in previous

(4.1 and earlier) versions of JOBSDUMP.  JOBSDUMP 4.2 takes care of this by

repeating every fourth sixel.  While this approach reproduces the correct aspect

ratio, the output may not always be pleasing;  you should experiment and make

your own judgement.

A similar problem exists for the Epson-compatible

mode of output due to the difference in dot density between Epson printers

and DEC printers.  JOBSDUMP 4.1 ignored every sixth column of dots in

Epson mode, in order to achieve the correct aspect ratio.  This occasionally led

to a loss in picture quality, especially in pictures with lots of vertical

lines.  JOBSDUMP 4.2 instead bitwise OR's together every

fifth and sixth sixel, giving much better results.  In any case, this

feature can be disabled by invoking the command-line option -ei (the Epson

option, (I)gnoring the aspect ratio correction), if necessary.

(2) PRINTING OPERATIONS

***********************

JOBSDUMP allows you to print the graphics either a) directly to the

printer or b) to a file, for later printing using the DOS PRINT command.  This

is useful if you want to print several pictures in background while you do

something else on the computer.  The files created by pre-4.xx versions of

JOBSDUMP were large - about 40k for a small printout and 110k for a large

printout.  This has been drastically reduced in the present version to typical

values of about 4k for a small printout and 8k for a large printout.  It IS

possible to get up to about 35k (small) or 90k (large) if the picture is

sufficiently complex.  However, even with digitized photographs, I have not

yet encountered a large printout larger than 30k or a small printout larger

than 15k.  (For the DEC format, that is - the Epson-compatible output is just

about as big as the old DEC sixel files.)

In any case, to get started, all you have to do is type

JOBSDUMP

and follow the prompts.  This simple invocation will be all that most people

will want.  All of the options are set in roughly the same way.  To select an

option to change, use the up/down cursor keys.  The selected option has an arrow

to its left and its current value is displayed in reverse video.  To change the

value of the current option, use the left/right cursor keys.  The screen

immediately reflects the changed value.  In the print destination option, the

left/right keys toggle between the values "Printer" and "File  OUT.SIX".  If you

have selected "File OUT.SIX" and you would like a file other than one called

"OUT.SIX", pressing the <SELECT> key will prompt you for the new filename.  If

you prefer not to use the arrow keypad, <RETURN> has the same effect as the

down arrow, while the spacebar has the same effect as the right arrow.

(3) COMMAND-LINE OPTIONS

************************

Although the program has a cursor-driven interface, the options can

instead be supplied as command-line parameters using the same type of syntax as

UNIX commands.   For example (spaces other than the first one

after "JOBSDUMP" are optional),

JOBSDUMP -R 2 -S L -F E:\GRAPHICS\SIXLFILE.PIC -D 7 -N -E I

will prepare to dump the graphics memory, assuming high (-R)esolution (2), as a

large (L) (-S)ize picture, into the (-F)ile E:\GRAPHICS\SIXLFILE.PIC, pausing

first to (-D)isplay the graphics on the screen for 7 seconds.  Furthermore, the

picture will be in (-N)egative format (just like a photographic negative), if

you have chosen the default colours to print - all except the background.  If,

on the other hand, you have specified a group of colours to print, the -n

option will choose these as the colours NOT to print i.e. again, a negative.

Also, the output will be in (-E)pson-compatible format, (I)gnoring correction

of the scaling.  The options set in the command-line will be displayed on the

screen along with the default values for all options not specified.  At this

point, other changes may be made interactively if desired.  When you have set

all of the options, start JOBSDUMP by pressing the <DO> key.  If you want to run

JOBSDUMP unattended or in batch mode, set all of the required options on the

command-line as well as the -g option.  The latter option instructs JOBSDUMP to

-(G)o ahead without waiting for the <DO> key to be pressed.  Full instructions

on the use of the command-line parameters may be found by typing (the H stands

for help!)

JOBSDUMP -H

at the MS-DOS prompt.  The latter instructions may be redirected into the

file "filename" by typing

JOBSDUMP -H >FILENAME

in order to get a permanent record of the instructions.  (A copy of the

latter file is appended to this documentation file.)  As an example of

employing defaults, the command line

JOBSDUMP -g

will cause JOBSDUMP to proceed directly to an active state without any

prompting of the user, using the default values for resolution (2), size (S),

file (LST:, the printer), graphics (DEC), color (colours specified or all

except background) and preview (off).

NOTE that there are three items which may only be specified in the commane-line

option set.  The first of these is the -ei option, described above.  In

addition, the -d (display) option takes a second argument which is a number

between 1 and 30 - the number of seconds to display the graphics preview.  If it

is not set on the command line (e.g. -d10) the default preview time is 5

seconds.  Finally, the aspect ratio for the DEC LA100 printer may only

be set from the command line using the -l option described above.  If not

specified, the default is to use the LA50 aspect ratio (2/1).

The parameters may be entered in ANY ORDER on the command line.  In

addition, there is extensive error checking included in the program.  If any

command line parameter does not make sense to the parser, it is disregarded.  In

addition, if the same parameter is given more than once, the value farthest to

the right in the command line will be the one used.  As the printing proceeds,

JOBSDUMP periodically reports the percentage of the output which has been

completed.

------------------------------------------------------------------------------

**WARNING**

If you plan to use the graphics preview facility, be sure that you GET THE

RESOLUTION RIGHT!  If you give the incorrect resolution and try to use the

preview option, you will DESTROY THE PICTURE in graphics memory!  (You'll also

get garbage printout, but without preview you won't destroy the picture by

specifying the wrong resolution).  Thus, although the preview option can be

useful, use it carefully.  If you are not sure what the correct value of the

resolution is for a certain type of picture, try each possibility (1,2) WITHOUT

preview first.  You can deduce the correct resolution by whether you get a

printed picture or garbage.

e.g. Lotus 123      uses      MEDIUM                        resolution

GW-BASIC                 MEDIUM or HIGH

DRAW14                   MEDIUM or HIGH

GSX-86                   MEDIUM or HIGH

RBDVI                    HIGH

etc., etc.

_________________________________________________________________________

(4) ERROR CHECKING (File output only)

*************************************

Each (size, resolution) combination generates a distinctive

size of sixel-file in the worst case.  If there is less space than this on the

default disk (or the disk specified in the sixel-file <filespec>), the program

flashes a warning on the screen that there MAY be insufficient space.  (The

warning is not given in the CP/M 86/80 version of the program).  Usually,

however, this can be safely ignored except for very complex pictures or very

full disks!  If, during the course of the program, the disk fills up, JOBSDUMP

will halt with a message to that effect.  The portion of the file written up

to that point will be saved.

If the pathname in <filespec> is not valid, or a nonexistent

disk drive is selected, the program reports this condition and halts.

(If a nonexistent disk drive is specified in CP/M, the program halts with

a BDOS error.  However, any other type of file error in CP/M will be

handled by JOBSDUMP.)

(5) USE WITH GW-BASIC

*********************

JOBSDUMP may be used as follows inside GW-BASIC:

XX SCREEN 2                 'or "SCREEN 1"

.

.

XX REM User's graphics program, etc.

.

.

XX REM The next line is all you need

XX SCREEN 0 : SHELL"JOBSDUMP <parameters>"

As shown in the example above, the program is invoked as a child process

running under GW-BASIC in MS-DOS.  It is very important to include the statement

"SCREEN 0", since otherwise JOBSDUMP will interfere with the graphics screen.

This is since the "SHELL" command goes to MS-DOS in whatever graphics mode the

computer happens to be using at the time the command is encountered.  The "XX"

in the example above just refers to an arbitrary line number in GW-BASIC. The

field "<parameters>" refers to the command-line parameters described above.

Both COMMAND.COM and JOBSDUMP.COM must be accessible from within

your GW-BASIC program, so you should already have issued the PATH command

(possibly in AUTOEXEC.BAT) to enable MS-DOS to find JOBSDUMP.COM.  In

addition, make sure that the environment variable COMSPEC is pointing

at a valid copy of COMMAND.COM.  This is automatically taken care of as long as

COMMAND.COM is on the disk from which MS-DOS was started.

(6) USE DIRECTLY FROM THE MS-DOS or CP/M 86/80 PROMPT

*****************************************************

JOBSDUMP may instead be run directly from the MS-DOS prompt, and it

will print out whatever data remains in the RGO memory, even if it has not

been generated by a GW-BASIC program.  Once an application (such as Lotus

123, GW-BASIC etc.) has written data into graphics memory, it remains there

until the computer has been turned off unless it is overwritten or erased.

The graphics memory will even survive a <SETUP>/<CTRL><SETUP> reset

operation, as long as the power is not interrupted.  You must know what the

resolution of the graphics data is, in order to get the correct results the

first time.  However, if you make the wrong choice and garbage is printed

out, it is possible to abort and try again with the correct resolution.  This

is possible since JOBSDUMP was written to be unable to erase or otherwise harm

graphics memory.

For example, instead of the last line in the GW-BASIC example above, you

could use the statement:

XX SCREEN 0 : SYSTEM

which would take you back to MS-DOS.  Now, typing "JOBSDUMP <parameters>" would

have exactly the same results as you would get using the GW-BASIC example above.

(7) DIFFERENCES IN CP/M VERSION OF JOBSDUMP

*******************************************

The CP/M version is identical to release 4.0 of MS-DOS JOBSDUMP.  It

has all the same features as V4.2 except:  (i) the enhanced Epson correction;

(ii) the LA100 handling capability;  (iii) some minor cosmetic bug fixes.

Other minor differences occur in the error handling described in

section (4) above: (i) the program does not like to get an incorrect disk

drive in CP/M  and (ii) disk space warnings will not be given in CP/M.

-------End of JOBSDUMP documentation

-------Output of JOBSDUMP -h follows

JOBSDUMP [-r [{1|2}]] [-s [{s|l}]] [-f [<filespec>]]

[-e [i]] [-n] [-l] [-d [nn]] [-h]

JOBSDUMP V4.2 prints out DEC Rainbow graphics memory contents

to a DEC- or Epson-compatible printer, or to a disk file.

Any subset of colours may be specified for printing as black.

-r sets the resolution   :         1 = medium

2 = high (default)

-s sets the size         :         s = small (default)

l = large

-f sets the output file  :         PRN = printer (default)

<filespec> = named file (w/pathname)

-e outputs Epson format  :         i = ignore correction

-n gives a "negative" printout

-l corrects the aspect ratio for LA100 output

-dnn shows the picture   :         nn = display time (secs)

-h gives this help

-g starts the program immediately without pressing <DO>

The default is to print out all colours except colour number 0,

the usual background colour.  If the environment variable

JOBSCOLS exists, it is used to set the colours to be printed

as black.   For example, entering

SET JOBSCOLS=12ACD

prints colours 1, 2, 10, 12, 13 in black.  Colours are in hex

notation and are 0-3 for high resolution and 0-F for medium

resolution.  Note that the default is equivalent to

SET JOBSCOLS=123456789ABCDEF

Note also that, if -n were invoked on the command line, the default

colour settings could be obtained using

SET JOBSCOLS=0

____________________________________________________________

If you appreciate the benifits of this software, and would

like to show your appreciation, please send a $25 donation

to the author:

James A. O'Brien

1166 Hope St. #6

Stamford, CT 06907

USA

-------End of JOBSDUMP -h output